3GPP TSG RAN WG4 Meeting #114bis		R4-2503480

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	TP to TR 38.922 on power dynamic range

Agenda Item:	5.2.7

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#107, TR 38.922 was approved. In this contribution, we provide some clarification on power dynamic range definition for each frequency range, since there is no such definition in UE specs, which implies in the subsequent calculations.

# 2 Text Proposal

< Start of Change>

#### 4.3.1.1	Power dynamic range

Power dynamic range is defined as the range between maximum output power and minimum output power levels. The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value. For Rel-18 FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. Hence, the power dynamic range is 56 dB for 100 MHz channel bandwidth with power class 3 (i.e. 23 dBm maximum output power) UE.

<Next change>

#### 5.3.1.1	Power dynamic range

Power dynamic range is defined as the range between maximum output power and minimum output power levels. The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For existing FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. The minimum output power can be reused for 7.125 – 8.4 GHz, i.e. power dynamic range is 56 dB for 100 MHz channel bandwidth with power class 3 (i.e. 23 dBm maximum output power) UE.

<Next change>

#### 6.4.1.1	Power dynamic range

Power dynamic range is defined as the range between maximum output power and minimum output power levels. The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. Based on the study in sub-clause 6.5.2, the UE antenna type for 14800 – 15350 MHz can reuse existing FR1 antenna characteristics so that both nominal maximum and minimum output power refer to existing FR1 bands, i.e., power dynamic range is 56 dB with power class 3 (i.e. 23 dBm maximum output power) UE.

<End of change>

# Reference

[1] RP-250539, “TR 38.922 v2.0.0: Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”, Ericsson