3GPP TSG RAN WG4 Meeting #114bis		R4-2503486

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on Testability for UE RF requirements for LP-WUS

Agenda Item:	7.26.2.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114, there seems no progress regarding testability for UE RF requirements for LP-WUS, although a WF was approved [1]:

### Sub-topic 4-1 Testability for UE RF requirements Issue 4-1-1: General framework for LP-WUR testability Way Forward: - FFS Issue 4-1-2: Separate RF test case for idle and connection mode Way forward: - RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?

In this contribution, we continue to present our views on testability.

# 2 Discussion

As agreed in the previous meeting, only one RRC state needs to be tested. Of all three RRC states, there are several benefits if the UE is tested in the RRC_CONNECTED state:

- A dedicated test mode for the UE is unnecessary: test parameters and configurations to UE under test could utilize ordinary message delivering procedure.

- Avoid waking up MR frequently, thus reducing the required testing time: Statistics such as MDR and FAR can be stored during the test and sent via MR to TE by waking up MR at the end of test.

- No need for auxiliary tools

Therefore, we propose to test LP-WUS performance solely in the RRC_CONNECTED state.

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Actually, after the test configuration is completed, and LR is ready for receiving LP-WUS, NR MR can be switched to sleep mode during the test while statistics such as MDR and FAR etc could be countered and buffered. At the end of the test, NR MR can be wakened up and utilized to send packed test statistics to TE. That is to say, the detection rate for LP-WUS is counted without waking up MR during the test.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

Proposal 3: Detection rate for LP-WUS is counted without waking up MR during the test.

# 3 Conclusion

In this contribution, we have the following proposals for testability for UE RF requirements for LP-WUS:

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

Proposal 3: Detection rate for LP-WUS is counted without waking up MR during the test.

# Reference

[1] R4-2503028, “WF on UE RF requirements for LP-WUS”, vivo