3GPP TSG-RAN4 Meeting #114Bis	R4-2503560

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Relevance of phase noise and LLS for SNR for REFSENS

Source:	Qualcomm Incorporated

Agenda item:	7.24.2.2

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#114 approved a WF [8] for BS and CW requirements. WF contains simulation assumptions for LLS for reader SNR and also FFS for phase noise for CW. In this paper we discuss relevance of phase noise specification and provide simulation results for the reader SNR for D2R signal.

# 2. 	Discussion on CW phase noise

In TR 38.769 the CW phase noise is analysed and concluded that in the topology, the CW level seen at the reader is constant -39 dBm. This means that the CW and its noise is present in the reader input with CW power level of -39 dBm. This topology is presented in Figure 1.

Figure 1. Illustration of phase noise overlaying the backscattered D2R signal in CW coming outside from the topology

In TR, in Figure 6.7.5.2.2.4-1 (reproduced below) the consolidated impact to the SINR from different CW phase noise is presented. As it can be read, if the CW phase noise at the 15 kHz offset is -112 dBc/Hz, that corresponds to integrated phasenoise of -70 dBc/15kHz, and with the D2R bandwidth of 15 kHz, the SINR degradation due to CW phase noise is 4 dB.

Observation 1: With realistic phase noise performance, the impact of CW phase noise to SINR of the D2R is 4 dB for 50 % of the device connections.

This provides enough concer to specify the CW phase noise performance and in order to reduce the impact, we propose to specify the phase noise integrated over 15 kHz to be below -90 dBc/15 kHz for the CW node.

Proposal 1: Specify CW node phase to be better than -90 dBc/15 kHz.

# 3 Discussion on reader SNR

In WF [8] assumptions for Link Level Simulations were approved. In the assumptions the BW was agreed to be 15 and 180 kHz but the BW also depends on the bit rate which has not been agreed in ran1.

Below we present our simulations results.

F

Figure 2. 15 kHz BW results

Figure 3. 2 kHz BW results

These results indicate that there is a dependency on the BW, as expected. But since the BW is undecide in the RAN1, RAN4 can not conclude needed SNR. Results can be used for calibrating result between companies.

Observation 2: Achievable SNR is dependent on the BW

Proposal 2: RAN4 to wait for confirmation of the D2R bandwidth before agreeing final SNR values

# Conclusion

We discussed, observed and proposed:

Observation 1: With realistic phase noise performance, the impact of CW phase noise to SINR of the D2R is 4 dB for 50 % of the device connections.

Observation 2: Achievable SNR is dependent on the BW

Proposal 1: Specify CW node phase to be better than -90 dBc/15 kHz.

Proposal 2: RAN4 to wait for confirmation of the D2R bandwidth before agreeing final SNR values

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[8]	R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025