3GPP TSG-RAN4 Meeting #114Bis	R4-2503564

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Frequency error and UE regulatory requirements

Source:	Qualcomm Incorporated

Agenda item:	7.31.4

Release:	Rel-19

SI: 	IoT_NTN_TDD-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

WF [5] was approved for TDD NTN in RAN4#114. In the WF there was an agreement: “Further discuss the potential update of frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot/segment.”

WF also indicated about AMPR that: “Further discuss AMPR for 249. Proponent to clarify at next meeting which band has to be protected through this AMPR mechanism.”

In this paper we discuss both matters.

# 2. 	Discussion

## 2.1 	Freq error

RAN1 is discussing how to handle segmentations, the RAN1 agreement is in Annex of this paper. Segmentation was originally created to allow gaps for UE to update timing and frequency. Since this is TDD system, this may not be needed at all. The option 2 in seems most straight forward solution so that UE can pre-compensate every start of the UL burst, meaning the consecutive 8 UL slots, as described in [6].

Clause 6.4B.1of TS36.102:

…

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

…

Depending on which option RAN1 will choose, the Ran4 requirement may or may not be needing changes, but it seems that at least UE should be allowed to updated the pre-compensation in between UL bursts.

Proposal 1: UE is allowed to perform pre-compensation between two UL bursts.

Corresponding specification change could be to add sentence: “For NTN TDD system, UE shall pre-compensate in the beginning of the UL burst.”

UE behaviour during the burst is better left to RAN1 discussions.

## 2.2	Regulatory requirements

As discussed in [6], some AMPR may need to be re-used from b254 work, where there is discussion ongoing regarding the TBD AMPR for the in-band emissions mask. It also should be noted that the Table 6.2B.3-1 is missing reference to the requirements in clause 6.5A.3.3.2-1.

Is some discussions applicable emission limits are said to be derived from ITU Rec. ITU-R M.1480 but that document only defines limits for within the 1 626.5-1 662.5 MHz band which is not b249.

The current requirements in 36.102 do not contain the ITU 1480 inband emissions. These seem to be different values and offsets.

In some discussions, emission limits from ETSI EN 301 441 are referred and there applicable band 1 610,0 MHz to 1 626,5 MHz fits the b249 spectrum but it is not clear if this ETSI harmonized standard is applicable for 3GPP NTN system.

Maybe better source is ITU-R M.1343-1 where the scope is aligned with the b249. In there, there are e.g. inband emissions mask:

Which alignes with the NS_04N ant the table 8 of ITU-R M.1343-1 aligns with the NS_05N requirement in clause 6.5A.3.3.3. However, this is a recommendation so what 3GPP should adopt, needs to be clarified.

Observation: Clarity on which regulative document applies for this frequency band is needed from the spectrum owner

In the absence of any guidance from the license holder or other sources, 3GPP should not define any additional requirements for the b249.

Proposal 2: In the absence of clear regulatory rules that may not be covered under general emission requirements, 3GPP should not define any additional requirements.

## 2.3	Power class

The WID or any document anywhere does not mention the power class for this band. Since the UL duty cycle is much lower than FDD system, about 1/8th , we could say it will need 8 time more power to reach the same UL throughput. 8 x 23 dBm is 32 dBm.

Proposal 3: Power class for the NTN IoT TDD system is defined as 32 dBm.

# Conclusion

We observed and proposed.

Proposal 1: UE is allowed to perform pre-compensation between two UL bursts

Observation: Clarity on which regulative document applies for this frequency band is needed from the spectrum owner

Proposal 2: In the absence of clear regulatory rules that may not be covered under general emission requirements, 3GPP should not define any additional requirements.

Proposal 3: Power class for the NTN IoT TDD system is defined as 32 dBm.

# Reference

[1]	RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2419795, “Way Forward for [113][317] IoT_NTN_TDD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-1711536, “Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs”, RAN4 (Ericsson), 3GPP TSG RAN WG2 Meeting #84bis, Dubrovnik, Croatia, 9th – 13th October 2017

[4] R1-2500003, Report of RAN1#119 meeting

[5]	R4-2502292, “Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500491, “Initial views on RF impact for NTN IoT TDD”, Qualcomm, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[7]	R4-2501153, “AMPR value for NB-IoT in IoT NTN band b254”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, GR, 17th – 21st February, 2025

# Annex: Agreement in RAN1