3GPP TSG-RAN WG4 Meeting # 114bis	R4- 2503580

# Wuhan, Hubei, China, 07th – 11st April, 2025

Source:	LG Electronics

Title:	Discussion on simulation assumptions and results for LP-WUS/WUR

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

In this contribution, we provide our simulation results for NR LP-WUS/WUR based on listed simulation assumptions in approved WF [1].

Discussion

During last RAN4#114 meeting, RAN4 had a lot of consensus regarding simulation assumptions. So, according to the agreement, we provide our simulation results for NR LP-WUS/WUR RRM.

- Topic #2: Simulation assumptions and results

Sub-topic 2-1 Simulation assumptions and parameters

Simulation model

Basic elements of the system model

- Waveform generation (OOK-1, OOK-2, OOK-4)

- Digital-to-analog (D/A) converter

- TX/RX LPF: Butterworth filter

- RF modulation/demodulation signal

- RSRP measurement: RSRPEstimated estimated based on the signal energy received during the ON time corrected for the energy (noise and interference) received during the LP-SS OFF time, i.e. ∆E=EON-EOFF

  - is the mean value of the received power in the linear scale under the assumption of no noise and perfect channel knowledge

  - measured mean value of the received signal power in linear scale

Simulation assumption

Baseline assumption

| Parameters | Assumption |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| LP-SS BW | 5 MHz (132 subcarriers) |
| Waveforms | OOK-1, OOK-2, OOK-4 |
| SCS | 30 kHz |
| Number of measurement samples Code sequence length L | 2, 4, 8 4-32 bits |
| LP-SS measurement interval | 320 ms |
| Filter | Butterworth (5 Order) |
| ADC | 4 bits |
| Channel propagation | AWGN, TDLC300 |
| CFO | 5, 10, 20 ppm |
| Time offset | 1, 2, 5 us |
| Mobility | 3 km/h |
| Antenna configuration | 1x1 |

Cell parameters

SINR and SNR configurations

Code sequences

Sub-topic 2-2 Simulation results

Simulation results

- SINR -6dB, AWGN

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | 0 | 1.08 | 0 | 0.76 | 0 | 0.54 |
| {1, 6} | -0.01 | 0.87 | -0.01 | 0.6 | 0 | 0.42 |
| {1, 8} | 0 | 0.74 | -0.01 | 0.52 | 0 | 0.37 |
| {2, 8} | 0.03 | 0.61 | 0.03 | 0.43 | 0.02 | 0.31 |
| {2, 12} | 0.01 | 0.5 | 0 | 0.36 | 0 | 0.25 |
| {2, 16} | 0.02 | 0.43 | 0.02 | 0.31 | 0.01 | 0.21 |
| {4, 16} | -0.02 | 0.65 | -0.01 | 0.44 | -0.01 | 0.31 |
| {4, 32} | -0.01 | 0.44 | 0 | 0.31 | -0.01 | 0.22 |

- SINR -6dB, TDLC 300ns

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | -0.83 | 4.09 | -0.57 | 2.79 | -0.44 | 1.97 |
| {1, 6} | -0.78 | 3.97 | -0.54 | 2.8 | -0.46 | 1.92 |
| {1, 8} | -0.84 | 3.8 | -0.58 | 2.78 | -0.45 | 1.94 |
| {2, 8} | -0.8 | 3.92 | -0.52 | 2.84 | -0.44 | 2.03 |
| {2, 12} | -0.82 | 3.74 | -0.59 | 2.69 | -0.47 | 1.95 |
| {2, 16} | -0.79 | 3.64 | -0.53 | 2.68 | -0.45 | 1.92 |
| {4, 16} | -1 | 3.91 | -0.79 | 2.75 | -0.64 | 1.94 |
| {4, 32} | -1.04 | 3.88 | -0.73 | 2.71 | -0.63 | 1.97 |

- SINR -3dB, AWGN

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | -0.01 | 0.57 | 0 | 0.41 | 0 | 0.3 |
| {1, 6} | 0 | 0.47 | 0 | 0.34 | 0 | 0.24 |
| {1, 8} | 0.01 | 0.42 | -0.01 | 0.29 | 0 | 0.21 |
| {2, 8} | 0.01 | 0.36 | 0.01 | 0.25 | 0.02 | 0.18 |
| {2, 12} | 0 | 0.3 | -0.01 | 0.21 | 0 | 0.15 |
| {2, 16} | 0 | 0.26 | 0.01 | 0.18 | 0.01 | 0.13 |
| {4, 16} | 0 | 0.38 | -0.01 | 0.27 | 0 | 0.2 |
| {4, 32} | 0.01 | 0.27 | 0 | 0.19 | 0 | 0.14 |

- SINR -3dB, TDLC 300ns

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | -0.82 | 3.77 | -0.63 | 2.64 | -0.43 | 1.91 |
| {1, 6} | -0.81 | 3.67 | -0.59 | 2.68 | -0.44 | 1.91 |
| {1, 8} | -0.78 | 3.78 | -0.61 | 2.69 | -0.45 | 1.91 |
| {2, 8} | -0.73 | 3.96 | -0.52 | 2.8 | -0.46 | 2.03 |
| {2, 12} | -0.8 | 3.59 | -0.52 | 2.65 | -0.45 | 1.9 |
| {2, 16} | -0.86 | 3.73 | -0.57 | 2.75 | -0.43 | 1.97 |
| {4, 16} | -1.01 | 3.84 | -0.75 | 2.71 | -0.62 | 1.95 |
| {4, 32} | -0.96 | 3.82 | -0.81 | 2.74 | -0.66 | 1.92 |

- SINR -0.5dB, AWGN

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | 0 | 0.37 | 0 | 0.26 | 0 | 0.18 |
| {1, 6} | 0 | 0.3 | 0 | 0.21 | 0 | 0.15 |
| {1, 8} | 0 | 0.26 | 0 | 0.18 | 0 | 0.13 |
| {2, 8} | 0 | 0.24 | 0.01 | 0.17 | 0.01 | 0.11 |
| {2, 12} | 0 | 0.2 | 0 | 0.14 | 0 | 0.1 |
| {2, 16} | 0 | 0.17 | 0 | 0.12 | 0 | 0.08 |
| {4, 16} | 0 | 0.27 | 0 | 0.19 | 0 | 0.13 |
| {4, 32} | 0 | 0.19 | 0 | 0.13 | 0 | 0.1 |

- SINR -0.5dB, TDLC 300ns

| {M, L} | RSRP measurement accuracy [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # of sample : 2 |  | # of sample : 4 |  | # of sample : 8 |  |
|  | mean | delta | mean | delta | mean | delta |
| {1, 4} | -0.82 | 3.78 | -0.6 | 2.71 | -0.46 | 1.95 |
| {1, 6} | -0.78 | 3.65 | -0.57 | 2.61 | -0.45 | 1.94 |
| {1, 8} | -0.81 | 3.72 | -0.63 | 2.68 | -0.46 | 1.88 |
| {2, 8} | -0.82 | 3.79 | -0.58 | 2.75 | -0.43 | 2.01 |
| {2, 12} | -0.83 | 3.76 | -0.6 | 2.71 | -0.45 | 1.9 |
| {2, 16} | -0.75 | 3.72 | -0.6 | 2.65 | -0.44 | 1.9 |
| {4, 16} | -0.99 | 3.86 | -0.74 | 2.72 | -0.62 | 1.92 |
| {4, 32} | -1.02 | 3.87 | -0.79 | 2.72 | -0.63 | 1.94 |

Observation 1: In AWGN channel propagation, one sample is enough to meet the accuracy requirements at every scenario.

Observation 2: In TDLC300 channel propagation, 5 samples are necessary to meet the 2.5dB accuracy requirements and 3 samples are necessary to meet the 3.5dB accuracy requirements.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Observation 1: In AWGN channel propagation, one sample is enough to meet the accuracy requirements at every scenario.

Observation 2: In TDLC300 channel propagation, 5 samples are necessary to meet the 2.5dB accuracy requirements and 3 samples are necessary to meet the 3.5dB accuracy requirements.

Reference

[1] R4-2502600, “WF on RRM requirements for NR_LPWUS,” Moderator (vivo)

[2] R4-2500539, “Topic summary for [114][225] NR_LPWUS,” Moderator (vivo)