3GPP TSG-RAN4 #114-bis	R4-2503711

Wuhan, China, 7th Apr. – 11th Apr. 2025

Title:	FR2 test setup for AI/ML beam management evaluation

Source:			Anritsu Corporation

Agenda Item:			7.19.3

Document for:			Approval

# 1.	Introduction

At the RAN4 #114 meeting in Athens, we continued a discussion on the AI/ML beam management test setup and the associated way forward [1] was created, extracted below are a few issues for reference.

- 2.4 Testing for one sided models - 2.4.1 Agreements in main session Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

In this contribution, we share our views on test setup for AI/ML beam management evaluation mainly from a viewpoint of 1AoA test setup.

# 2.	Discussion

# 2.1. General view on AI/ML beam management

RAN4 continued a discussion on AI/ML beam management for several meetings. However, when we consider test cases for the beam management evaluation, we need to note that TR 38.843 [2] is assuming two kinds of sub-usecases, i.e. BM-Case 1 and BM-Case 2. And it appears that RAN4 was discussing the associated test setup mainly based on BM-Case 1 as follows.

BM-Case 1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams

BM-Case 2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams

To fulfill the test coverage of beam management, the associated test setup needs to take into account both BM-Case 1 and Case 2. Thus, we need to clarify all the necessary conditions for both BM-Case 1 and Case 2 before down-selecting the test setup into 1AoA or multiple AoA setup.

Observation 1: RAN4 needs to clarify all the necessary conditions of associated test setup for both BM-Case 1 and Case 2 before down-selecting the test setup from 1AoA and multiple AoA.

# 2.2 Views on Beam management case 1

BM-Case 1 (Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams)

We would first like to discuss presumable UE patterns for BM-Case 1 based on the previous discussions in RAN1 and RAN4. Based on the previous discussions in the group and the report in TR 38.843 [2], when we conduct the evaluation of BM-Case 1, we think there are currently three possible UE patterns as follows.

Observation 2: There are currently three kinds of presumable UE patterns from the AI model input PoV.

UE pattern 1: No need to input Rx Beam information in the AI model.

In this case, it is expected that the UE Rx beam is pointing towards the direction of the measurement probe in an OTA chamber where the DL signals are transmitted.

UE pattern 2: One Rx Beam information is input to the AI model.

In this case, direction of the UE Rx beam is same as UE pattern 1 and that Rx Beam information is input to the AI model. TBD if the actual Rx beam pattern is necessary or it can be standardized.

UE pattern 3: Multiple Rx Beam information is input to the AI model.

In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID.

Next, we’d like to consider the feasibility of BM-Case 1 tests by the 1AoA test setup for each UE pattern above.

Feasibility of BM-Case 1 tests with UE pattern 1 and 2

As explained in the previous contribution [3], BM-Case 1 tests for UE pattern 1 and pattern 2 can be conducted by 1AoA test setup with the following conditions.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the test.

Details of the actual test procedure were described first in the contribution [3]. This time we slightly changed our assumption from before and described them in Appendix A at the end of this contribution.

Observation 3: BM-Case 1 tests with UE pattern 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) can be conducted with the following conditions.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the evaluation.

Details of the actual test procedure were described first in the contribution [3]. This time we slightly changed our assumption from before and described them in Appendix A at the end of this contribution.

Feasibility of BM-Case 1 tests with UE pattern 3

As for the UE pattern 3, we would like to show our expectation of behavior of the UE Rx beam during the test.

Figure 2.2-1 depicts an example of the Rx beam behavior during the test of BM-Case 1 in MPAC. During the test in the MPAC, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to the UE implementation. In this situation the UE can obtain multiple measurement results of L1-RSRP even for one Tx Beam ID setting. Note the Rx beam direction is not always matching the direction of each measurement probe.

Observation 4: During the test with UE pattern 3 in the MPAC, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to the UE implementation.

Figure 2.2-1: Example of the Rx beam behavior during the test of BM-Case 1 in MPAC

Next, we would like to consider a corresponding test procedure with the 1AoA test setup. Since there is only one measurement probe in the chamber, measurement procedures are slightly different from the case of MPAC and there are some conditions which need to be agreed.

Condition 1: Allow some time to move UE positions by a positioner. For example by introducing a test mode to set a gap to run measurements of L1-RSRP, or signaling is introduced to pause and resume L1-RSRP measurements while the positioner is rotating.

Condition 2: Rx beam directions for the measurement are declared by UE vendors.

Condition 3: Like the UE pattern 1 and 2, Rx beam profile is standardized.

Proposal 1: Agree following conditions to conduct the BM-Case 1 test with UE pattern 3 by 1AoA test setup

Condition 1: Allow some time to move UE positions by a positioner. For example, by introducing a test mode to set a gap to run measurements of L1-RSRP, or signaling is introduced to pause and resume L1-RSRP measurements while the positioner is rotating.

Condition 2: Rx beam directions for the measurement are declared by UE vendors.

Condition 3: Like the UE pattern 1 and 2, Rx beam profile is standardized.

Figure 2.2-2 and Figure 2.2-3 demonstrate the measurement procedure of the BM-Case 1 with UE pattern 3 by 1AoA test setup. The main difference between MPAC is that Tx Beam IDs are switched per one measurement direction till they reach the number of Tx Beam ID in set B while we expect that Rx beam ID is switched per Tx Beam ID in MPAC.

Figure 2.2-2: Measurement by 1AoA test setup #1

Figure 2.2-3: Measurement by 1AoA test setup #2

The associated measurement procedure would be as follows.

Observation 5: Measurement procedures with UE pattern 3 by 1AoA test setup would be as follows.

Step 1: Load the Tx Beam IDs from Set B in TE.

Step 2: Rotate the positioner to one of the declared measurement directions.

Step 3: Set one of the loaded Tx Beam ID and its associated CDL channel model in TE.

Step 4: Establish the wireless (virtual) cable connection between the UE and TE.

Step 5: Create a composite DL signal from each cluster under the conditions of Step 3 on the fading simulator and transmit it from the measurement probe.

Step 6: Measure the L1-RSRP in UE.

Step 7: Repeat Step 3 to Step 6 based on the next Tx Beam ID as soon as the L1-RSRP measurement for the first Tx Beam ID is completed.

Step 8: When the measurement reaches the maximum Tx Beam ID x of Set B, rotate the positioner to the next declared measurement direction, and repeat Steps 3 to 7.

Step 9: Repeat from Step 3 to 8 until it reaches the total number of declared measurement directions.

Step 10: UE inputs the obtained Tx Beam ID, Rx Beam ID and L1-RSRP to the AI model.

Step 11: The AI model answers the TE with the best Tx Beam ID (Top-k or Top-1, etc.) as predicted by the AI model.

TBD how the UE triggers the predictions by AI or how the predicted best Tx Beam IDs are signaled.

Step 12: While switching all Tx Beam IDs as Set A, measure L1-RSRP in the same way as for Set B. However, the combination that has already been measured as Set B is skipped.

Step 13: From all the measurement results of Set A, the Tx Beam ID with the largest L1-RSRP is determined and compared with the estimation results sent in Step 11) to make a pass/fail decision.

Though our suggestion is still to allow the conditions listed in Proposal 1 above, in case they are not acceptable to the UE vendors and chipset vendors, we would suggest agreeing alternative solutions as follows. As mentioned above in Observation 2, there seems to be currently 3 kinds of UE patterns depending on the AI model inputs. And even when the UE Rx Beam information is input to the AI model, there is a case that the prediction by the AI model can be made by measurement results conducted only from one UE Rx beam direction. Thus, we would propose allowing different types of test systems depending on the previously mentioned UE patterns, with one condition if the group could confirm that the tests with the multiple AoA test setup with simplified CDL condition can ensure the AI performance in the field.

Proposal 2: Allow different types of test systems depending on the UE patterns on condition if the group could confirm that the tests with the multiple AoA test setup with simplified CDL condition can ensure the AI performance in the field. i.e. 1AoA test system can be applied to the UE pattern 1 and 2 in Observation 2 and the multiple AoA test system can be applied to all the UE patterns.

# 2.3 Views on Beam management case 2

BM-Case 2 (Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams)

As mentioned in sub-clause 2.1, there are two sub-usecases for beam management. And we think the group also needs to discuss associated channel models to be used for the evaluation of this BM-Case 2. According to the report attached in TR 38.843 [2], the link level simulation in RAN1 appears to apply an assumption of the channel model described in TR 38.901 clause 7.6.3 [4], i.e. Spatial consistency.

Observation 6: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3 [4], i.e. Spatial consistency.

When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

Observation 7: When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE is moving, though there is a limitation that it cannot change the Rx Beam ID in a short time.

Observation 8: There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE moves, though there is a limitation that it cannot change the Rx Beam ID in a short period.

Companies are encouraged to provide a methodology on how MPAC can reproduce the corresponding environment for BM-Case 2 under the situation where probes are all fixed.

Proposal 3: Companies are encouraged to provide a methodology on how MPAC can reproduce the channel environment of the moving UE condition for BM-Case 2 under the situation where probes are all fixed.

- Conclusion

In this contribution, we shared our views on test setup for AI/ML beam management evaluation mainly from a viewpoint of 1AoA test setup.

Observation 1: RAN4 needs to clarify all the necessary conditions of associated test setup for both BM-Case 1 and Case 2 before down-selecting the test setup from 1AoA and multiple AoA.

Observation 2: There are currently three kinds of presumable UE patterns from the AI model input PoV.

UE pattern 1: No need to input Rx Beam information in the AI model.

In this case, it is expected that the UE Rx beam is pointing towards the direction of the measurement probe in an OTA chamber where the DL signals are transmitted.

UE pattern 2: One Rx Beam information is input to the AI model.

In this case, direction of the UE Rx beam is same as UE pattern 1 and that Rx Beam information is input to the AI model. TBD if the actual Rx beam pattern is necessary or it can be standardized.

UE pattern 3: Multiple Rx Beam information is input to the AI model.

In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID.

Observation 3: BM-Case 1 tests with UE pattern 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) can be conducted with the following conditions.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the evaluation.

Observation 4: During the test with UE pattern 3 in the MPAC, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to the UE implementation.

Proposal 1: Agree following conditions to conduct the BM-Case 1 test with UE pattern 3 by 1AoA test setup

Condition 1: Allow some time to move UE positions by a positioner. For example, by introducing a test mode to set a gap to run measurements of L1-RSRP, or signaling is introduced to pause and resume L1-RSRP measurements while the positioner is rotating.

Condition 2: Rx beam directions for the measurement are declared by UE vendors.

Condition 3: Like the UE pattern 1 and 2, Rx beam profile is standardized.

Observation 5: Measurement procedures with UE pattern 3 by 1AoA test setup would be as follows.

Step 1: Load the Tx Beam IDs from Set B in TE.

Step 2: Rotate the positioner to one of the declared measurement directions.

Step 3: Set one of the loaded Tx Beam ID and its associated CDL channel model in TE.

Step 4: Establish the wireless (virtual) cable connection between the UE and TE.

Step 5: Create a composite DL signal from each cluster under the conditions of Step 3 on the fading simulator and transmit it from the measurement probe.

Step 6: Measure the L1-RSRP in UE.

Step 7: Repeat Step 3 to Step 6 based on the next Tx Beam ID as soon as the L1-RSRP measurement for the first Tx Beam ID is completed.

Step 8: When the measurement reaches the maximum Tx Beam ID x of Set B, rotate the positioner to the next declared measurement direction, and repeat Steps 3 to 7.

Step 9: Repeat from Step 3 to 8 until it reaches the total number of declared measurement directions.

Step 10: UE inputs the obtained Tx Beam ID, Rx Beam ID and L1-RSRP to the AI model.

Step 11: The AI model answers the TE with the best Tx Beam ID (Top-k or Top-1, etc.) as predicted by the AI model.

TBD how the UE triggers the predictions by AI or how the predicted best Tx Beam IDs are signaled.

Step 12: While switching all Tx Beam IDs as Set A, measure L1-RSRP in the same way as for Set B. However, the combination that has already been measured as Set B is skipped.

Step 13: From all the measurement results of Set A, the Tx Beam ID with the largest L1-RSRP is determined and compared with the estimation results sent in Step 11) to make a pass/fail decision.

Proposal 2: Allow different types of test systems depending on the UE patterns on condition if the group could confirm that the tests with the multiple AoA test setup with simplified CDL condition can ensure the AI performance in the field. i.e. 1AoA test system can be applied to the UE pattern 1 and 2 in Observation 2 and the multiple AoA test system can be applied to all the UE patterns.

Observation 6: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3 [4], i.e. Spatial consistency.

Observation 7: When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

Observation 8: There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE moves, though there is a limitation that it cannot change the Rx Beam ID in a short period.

Proposal 3: Companies are encouraged to provide a methodology on how MPAC can reproduce the channel environment of the moving UE condition for BM-Case 2 under the situation where probes are all fixed.

# 4.	References

[1] R4-2502856, “WF on the requirements for AI/ML air interfance”, Moderator (Qualcomm), RAN4 #114, Athens, Greece

[2] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V18.0.0

[3] R4-2500167, “FR2 test setup for AI/ML beam management evaluation”, Anritsu, RAN4 #114, Athens, Greece

[4] TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz", v18.0.0, 2024-03

# Appendix A: Measurement procedures of 1AoA + Fading simulator with wireless (virtual) cable connection

In this appendix, we provide supplemental explanations on the measurement procedures of 1AoA + Fading simulator with wireless cable connection in sub-clause 2.2. Measurement procedures may vary depending on an assumption whether the UE Rx beam pattern is the AI model input or not, and whether the actual UE Rx beam pattern is necessary or not.

AI model input case 1: UE Rx beam information is AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator.

AI model input case 2: UE Rx beam information is not AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator, though it is not mandatory.

We will explain the associated measurement procedure for each case as follows.

AI model input case 1 a): UE Rx beam information is AI model input and use a standardized UE Rx beam pattern in the fading simulator

Refer to the following AI model input case 2 a) since the procedures are same.

AI model input case 1 b): UE Rx beam information is AI model input and use an actually measured UE Rx beam pattern in the fading simulator

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at a certain position and set the UE in the Rx-beam-lock mode by UBF test command. The Rx-beam-lock mode needs to be kept in step 5, 6 and 7.

- Rotate the UE orientation by the positioner to measure a 3-dimensional Tx-antenna pattern of the UE, while assuming reciprocity between Tx and Rx antenna pattern. (Without ATF: Antenna Test Function being equipped in the UE, it wouldn’t be possible to directly measure Rx-antenna pattern.)

- Set the UE orientation to the same position as in step 4 and establish a wireless (virtual) cable connection. By this connection, DL signals based on the simulated channel model will be distributed virtually directly to an antenna connector of the UE.

- Set the measured Tx/Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management, where the Rx-beam ID in UE recognition should be aligned with the Rx-antenna pattern in the fading simulator.  (If necessary, we can perform testing several times with various UE orientations in the simulation while fixing the Rx-beam ID.)

Cross-polarized antenna

UE

Positioner

Step 4. : Stop changing UE orientation and set Rx-beam-lock mode

Cross-polarized antenna

UE

Positioner

Step 5. : Rotate UE while measuring Tx-antenna pattern

Cross-polarized antenna

UE

Positioner

Step 6. : Set UE orientation to the one in step4 to establish wireless-cable connection

Wireless cable connection

Rotate

If it is required to perform testing with multiple Tx/Rx-antenna patterns or Rx-beam IDs, repeat from step 3 to step 7.

AI model input case 2 a): UE Rx beam information is not AI model input and use a standardized UE Rx beam pattern in the fading simulator

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at the position where the UE Rx beam directs towards the bore sight direction and set the UE in the Rx-beam-lock mode. The Rx-beam-lock mode needs to be kept in step 5 and step 6.

- Establish a wireless (virtual) cable connection.

- Set a standardized Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management.  (If it is required for the AI model to correctly output inferring results with any Rx-antenna steering patterns, it would be necessary to perform testing multiple times with various combinations of a UE orientation and a standardized Rx-antenna pattern being loaded into the fading simulator.)

AI model input case 2 b): UE Rx beam information is not AI model input and use an actually measured UE Rx beam pattern in the fading simulator

The measurement procedure for “AI model input case 2 b)” would be similar to the one in “AI model input case 1”.  The difference would be as the following.

- The same measurement procedure for “AI model input case 1” can be used for “AI model input case 2 b)”, but it is not required that the Rx-beam recognition in the UE and the Rx-beam pattern used in the fading simulator are aligned.  So, in the case of “AI model input case 2 b)”, it should be allowed to change the Rx-beam direction between the time when measuring the Tx/Rx-beam pattern and the time when establishing the wireless connection.  If necessary, the wireless connection can be established with UE Rx-beam directed towards the bore sight direction.

Comparing cases between the AI model input case 2 a) and 2 b), AI model input case 2 b) has a benefit that the fading simulator can use the actual UE Rx beam pattern in its channel simulation with a cost of an additional measurement time.