# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503772

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.20.1

Source: 	CMCC

Title: 			 Discussion on general aspects for study on AI/ML

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [2]. This contribution provides discussion on general aspects.

- Discussion

Issue 1-1-1: How many requirement sets to define - Proposals - Option 1: Define requirement sets for at least every feature and applicable subset (Apple proposal 1/12, Nokia proposal 1) - Option 1a: FFS how to relate subsets and applicable conditions depending on definition of applicable conditions (Apple proposal 2, 13) - Option 1b: A minimum set of configurations/IDs shall be mandatory (Nokia proposal 2) - Option 1c: Do not define requirements with a change of associated ID during testing (Nokia proposal 3) - Option 1d: Ensure consistency between applicable conditions signalling during training and inference (Apple proposal 10) - Option 2: At least a requirement set per functionality, TBC for subsets depending on conclusions on granularity (CMCC proposal 3, CATT proposal 4)

According to RAN1 discussion, for AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. It can be seen that UE will report the supported configurations for a certain model/functionality. The scenarios and/or configurations for requirements/tests can be decided based on the supported configuration reported by UE.

A LS on applicable functionality reporting for beam management UE-sided model is sent from RAN2 to RAN1. One of the questions is what is the granularity of functionality. For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others. According to RAN1 agreements, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. The concept/terminology “functionality”of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs. The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters. The Activated functionalities may be enabled based on CSI framework. The definition is also alligned with RAN2 understanding. Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling). Applicable functionalities refers to functionalities that the UE is ready to apply for inference. Activated functionalities refers to functionalities already enabled for performing inference.

RAN1 agreements in RAN1 #119 - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. RAN2 agreements in RAN2 #127 Agreements on definitions - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refers to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference

Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities in step 1. UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. The applicable functionalities can be sent to network in step 3 or updated in UAI. The funtionalily can be activated after step 3 or step 5 pending on whether full inference configuration is signalled in step 3 or step 5. The UE applicable functionalities can be activated autonomously by UE or activated by network indication.

Based on RAN1 and RAN2 agreements, it can be seen that the interaction between network and UE is in the granularity of funtionality. From RAN4 point of view, the requirements/tests are defined at least based on funtionality (funtionality refer to AI/ML-enabled Feature/FG) .

Proposal 1: RAN4 requirements are specified at least based on functionality (funtionality refer to AI/ML-enabled Feature/FG)

Issue 1-1-2: How many requirements to define per requirement set - Proposals - Option 1: To determine how many requirements to define, a study is needed of how AI/ML behaviour changes with different conditions (Apple proposal 3, Nokia proposal 4) - Option 2: Different test configurations/parameters are used for tests, at least for generalization (CMCC proposal 2). - Option 3: It is necessary to ensure that the testing avoids the overfitting problem; if not other solutions may be needed (Korea test laboratory proposals 4 and 5) - Option 4: Create a mixed test-set for generalization testing, based on UE capability signaling configuration report (Apple proposals 7, 8) - Option 5: Scenarios not identified in the capability report are other scenarios for test (ZTE)

In previous meeting, the testing goals are updated as to verify whether the minimum performance of AI/ML functionality/feature can be achieved, but FFS on whether and how many different test configurations/parameters are used for tests. Our view is that it is necessary to consider different test configurations/parameters at least for generalization. According to Rel-18 discussion, the goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations.

Based on RAN1 evaluation, generalization is impacted by many factors. In high level, according to TR 38.843, deployment scenarios, UE distributions, carrier frequency, TxRU mappings are considered for generalization of CSI compression. And CSI payload sizes, Tx port numbers, are considered for scalability evaluations for CSI compression. For beam management, the performance is impacted by deployment scenarios, UE mobility, etc. Similar situation for positioning. These factors which have impact on performance need to be considered in define requirements

Proposal 2: it is proposed that different configurations/parameters are considered for requirements/tests definition.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: RAN4 requirements are specified at least based on functionality (funtionality refer to AI/ML-enabled Feature/FG)

Proposal 2: it is proposed that different configurations/parameters are considered for requirements/tests definition.

- Reference

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R4-2503953, WF on AI/ML air interface study, Ericsson

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R5-247647, Discussion on handling of AI/ML in RAN5, China Mobile, AT&T, CAICT, Ericsson, Huawei, MediaTek, Nokia, ZTE

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface