3GPP TSG-RAN WG4 Meeting #114bis	R4-2503873

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 A-IoT device

Document for:	Discussion

# Introduction

The study item of Ambient IoT [1] was completed with WI approve in [2] and further updated in [3]. The RRM discussion is scheduled to start from RAN4#114bis meeting. In this contribution, we provide our views on potential RRM impacts for Ambient-IoT.

# Discussion

Per WID description, RRM scope is to specify core requirements for device 1 if necessary as shown as follows:

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary

To have better understanding of this completely new type of UE, we brief introduce the background per outcome of study phase and the progress of WI so far.

As captured in TR 38.769, following types of device and scenario as considered during the study.

Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device's D2R transmission is backscattered on a carrier wave provided externally. Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. D1T1: Deployment scenario 1 with connectivity topology 1, according to TR 38.848. D2T2: Deployment scenario 2 with connectivity topology 2, according to TR 38.848.

Based on the WID description, the device type and scenarios are further down selected in Rel-19 as follows. D1T1-B refers to Deployment scenario 1 (indoor-to-indoor) with Topology 1, and indoor microcell basestation with CW outside the topology.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

Observation 1: Only Device 1 and D1T1-B are considered for Rel-19 WI.

For the supported services, inventory and command are studied and the definitions are as follows:

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s). Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

Per latest progress in RAN2, only “inventory only” and “inventory and command” are considered.

Agreements From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Observation 2: Only “Inventory only” and “Inventory and command” are considered.

Considering the characteristics of use case, and the extremely low complexity, the PHY and higher design is greatly simplified. Per WID description about the scope of RAN1/2, it could be observed that

For PHY layer:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R

For Higher layer:

- Only Paging, random access and data transmission is supported.

- Only MAC layer is included.

By looking at the outcome of study phase as captured in the TR 38.769, we can find more information on simplification in terms of RS/physical channel/functionalities.

PHY A dedicated physical broadcast channel, e.g. PBCH-like, and reference signals including at least DMRS, PTRS, CSI-RS/TRS, are not considered for R2D Reference signals including DMRS, PTRS, SRS, are not considered for D2R. CSI feedback and autonomous SR are not considered for L1 D2R control information. Protocol stack and signalling procedures For A-IoT, it is assumed the commands (e.g., read/write/disable) and/or inventory information are carried over A-IoT radio interface as the upper layer data. As to the protocol stack for A-IoT radio interface between A-IoT device and reader, it is assumed: - RRC layer is not supported - SDAP layer is not supported - PDCP layer is not supported - RLC layer is not supported - A-IoT MAC layer is supported - A-IoT physical layer is supported - There is no control plane and user plane differentiation As to the A-IoT required functionalities, the following functionalities are supported: - A-IoT paging (see clause 6.3.3) - A-IoT random access procedure (see clause 6.3.4) - A-IoT data transmission (see clause 6.3.5) As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): - AS security (The AS layer design assumes no support of AS security, unless the study in [152] further concludes differently.) - RRC states - RRC connection management - RRM L3 measurement reporting - Mobility - ASN.1 encoding/decoding - Periodical system information and MIB - Tracking/RAN area update procedure - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition

Based on analysis above, it could be observed that all most all typical RRM activities considered for NR UEs are not supported for Rel-19 A-IoT device, e.g. measurement/reporting, Cell reselection, Handover, beam management.

Observation 3: Most RRM requirements defined for normal NR UE are not supported for A-IoT device.

Thus, based on above analysis, the RRM impacts for A-IoT device are quite limited. Taking legacy RRM spec as reference, the only impact could be the random-access requirements, which defined correct UE behaviour during random access. The behaviours are defined in RAN2 MAC spec and quoted in RAN4 RA requirements.

Based on the agreements in last RAN2 meeting, there will be both CBRA and CFRA, and only 3-step CBRA is up ported in Rel-19.

Agreements - For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA.

Different from legacy random-access procedure, random access of A-IoT is always triggered by R2D message (i.e. paging). To be specific, for CBRA, device will transmit a 16bits random ID, and reader will respond with the successfully received random ID, and if device receives the random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. Then device will perform data transmission in Msg3 (e.g. device ID), and other data transmission (e.g command).  For CFRA, UE will transmit the 16 bits random ID and device ID together as Msg1.

Apart from above, RAN2 is also discuss access failure handling, with following progress:

Agreements - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback.

Based on the analysis above, the potential RRM impacts is to define device random access procedural requirements. However, it should be noted that only limited/essential behaviour to be considered. Subsequent transmission/reception (e.g. data transmission) is not part of RRM requirements/behaviour. Besides, the RAN4 random access procedure requirements are directly reference to RAN2 MAC spec. The same principle shall apply to avoid causing inconsistence between RAN2 and RAN4 spec.

Proposal 1: RAN4 to include procedural requirements for random access. Only limited/essential behaviour during Random access to be included, which should be selected from and consistent with RAN2 A-IoT MAC spec.

Considering that the discussion in RAN2 is still ongoing without stable running CR, the details could be further discussed with more concrete RAN2 progress.

Apart from the random-access procedural requirements, another potential RRM impact is about timing requirements for D2R transmission. In legacy RRM spec, RAN4 define timing requirements for transmit timing error, TA adjustment accuracy, MRTD/MTTD, etc. For A-IoT device, there no maintained DL timing, nor close-loop TA. The D2R transmission is always triggered by a R2D message, and the A-IoT device backscatter the CW accordingly. It could be observed that the timing relationship is under discussion in RAN1. Whether to define D2R transmission timing requirement shall wait for RAN1 conclusion on timing relationship design. And we should avoid parallel discussion in different WG.

Proposal 2: RAN4 to discuss whether there is RRM impact on D2R transmit timing requirements based on RAN1 final conclusion on timing relationship design.

Considering the extremely low complexity and low cost of A-IoT device, except for RA and timing aspect as mentioned in Proposal 1 and 2, we see no other RRM impacts for A-IoT device.

Proposal 3: Except for procedural requirements for random access and D2R transmit timing requirements if needed, no other RRM impacts for A-IoT device.

# Conclusions

Observation 1: Only Device 1 and D1T1-B are considered for Rel-19 WI.

Observation 2: Only “Inventory only” and “Inventory and command” are considered.

Observation 3: Most RRM requirements defined for normal NR UE are not supported for A-IoT device.

Proposal 1: RAN4 to include procedural requirements for random access. Only limited/essential behaviour during Random access to be included, which should be selected from and consistent with RAN2 A-IoT MAC spec.

Proposal 2: RAN4 to discuss whether there is RRM impact on D2R transmit timing requirements based on RAN1 final conclusion on timing relationship design.

Proposal 3: Except for procedural requirements for random access and D2R transmit timing requirements if needed, no other RRM impacts for A-IoT device.

# References

[1] RP‑234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR