# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503911

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.2.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #114 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1] and summary [2].

- Discussion

Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1]:

### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue.

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”, when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction, UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE, and it’s preferred to introduce UE indication to the NW to assist to NW’s better decision and scheduling.

Observation 1: UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE.

Proposal 1: UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

Requirement of L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1] and summary [2]:

### Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. - Proposals - Option 1a: (ZTE, CATT, Ericsson, CT, CMCC) - Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6. - Option 1b: (QC) - At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS. - Option 1c: (MTK) - On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either: - Single value for reduced BSF (e.g., 6 or 4) - Multiple candidate values for BSF (e.g., 6 and 4) - Option 2: (vivo) - RAN4 to consider the following approaches as solutions: - Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc. - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 3: (HW) - The UE capability design shall allow enough flexibility on supporting this feature considering: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency - Option 4: (LGE) - Introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay - Recommended WF - Introducing the UE capability of reduced N to indicate the BSF value(s) or fraction value(s). FFS the value(s). - Discuss option 3 in regard to the UE capability design: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency

For UE capability, we prefer to introduce the UE capability of reduced N, the candidates could be 2,4 or 6.

Proposal 2: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Status in the WF [1] and summary [2]:

### Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. - Proposals - Option 1a: (NTT DCM, Ericsson) - For each scenario, the highlighted factors should be replaced with capability value. Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 2; Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 3; Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3. 4; Same as scenario 3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1.. 7; RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell... - Option 1b: (ZTE, OPPO) - For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability. - Option 1c: (CATT) - The reduced Rx BSF can be applied to the following formula according to the trigger conditions to be specified: - TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap; - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap; - Tsearch for FR1-FR2 HO and FR2-FR2 HO; - Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay; - Tidentify-NR for RRC connection release with redirection to NR. - The reduced Rx BSF can apply to the associated SSB in CSI-RS based measurement, CHO and DAPS HO without new specification impact. - Option 2: (LGE) - For L3 measurement, introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay - Table. Example of Proposed measurement period for intra-frequency measurements without gaps (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(400ms, ceil(1.5x Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Pmulti-Rx x Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. - Option 3: (Intel) - Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements. - When the criterion is met, the Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms, and Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms; where TSSB_processing = 2 ms, where TPSS/SSS_sync + TSSB_measurement_period is cut in half. - Option 4: (Nokia) - For L3 measurement delay requirements, cover the impact of Rel-19 BSF reduction through an additional scaling factor NFBS = [reduced BSF]/8, where the reduced BSF depends on UE capability. - For HO delay requirements, define the value for N in Tsearch depending on UE capability for BSF reduction. - For RRC re-establishment delay, o apply the scaling factor NFBS to the fixed value of Tidentify_intra/inter_NR e.g., NFBS x MAX (1000 ms, 104 x TSMTC, i), where NFBS = [reduced BSF]/8. - Capture the applicability of L3 FBS requirements in section 3.6. - Option 5: (Samsung) - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline. - Option 6: (MTK) - The following delay requirements/parameters of L3 measurement can be reduced by optimizing beam sweeping factor: - For scenarios (1/2/3/4): Mpss/sss_sync and Mmeas_period used in intra/inter-frequency (w/o gap) in FR2 - For scenario (5): N in Tsearch used in handover delay when target cell is unknown in FR2 - For scenario (7): Tidentify_intra and Tidentify_inter used to scale RRC Re-establishment/RRC Connection Release - Recommended WF Discuss this issue using option 1 as starting point.

For delay requirements of L3 measurement delay reduction by optimizing Rx BSF, for scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

Proposal 3: For delay requirements of L3 measurement delay reduction by optimizing Rx BSF, for scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Observation 1: UE autonomous quitting should be avoided since it may cause inconsistency between NW and UE.

Proposal 1: UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

Proposal 2: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Proposal 3: For delay requirements of L3 measurement delay reduction by optimizing Rx BSF, for scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

- References

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114, 17 – 21 February, 2025.

- R4-2500532, Topic summary for [114][218] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114, 17 – 21 February, 2025.