# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503950

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – system parameters and UE RF

Agenda item:		7.8.2

Document for:		Approval

# Introduction

In RAN#106, a new WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

Based on the agreed WF ([2]) from last RAN4#114 meeting, this contribution further discusses the remaining open issues related to system parameters and UE RF.

# Discussion

## UE specific issues

### Relevant NSs for n26

In last RAN4#114, it was agreed that the introduction of 7 MHz channel BW in band n26 will not impact A-MPR for NS_100. Also, it was agreed that NS_12 and NS_15 A-MPRs should be evaluated for 7 MHz channel BW but it was left for further study if NS_13 and NS_14 A-MPRs should also be considered.

When E-UTRA band 26 was specified, RAN4 studied the needed spurious emission limits to ensure protection of the adjacent PS service. It was finally agreed to specify a limit of -42dBm/6.25kHz with different offset values and various channel bandwidth ([3]). Based on this agreement, RAN4 introduced NS_12, NS_13, NS_14 and NS_15, evaluating the corresponding A-MPR values:

- NS_13 was specified with a requirement of -42dBm/6.25 kHz to protect the 806-816 MHz frequency range with a 3 MHz offset for 1.4, 3 and 5 MHz channel BW.

- NS_14 was specified with a requirement of -42dBm/6.25 kHz to protect the 806-816 MHz frequency range with an 8 MHz offset for 5, 10 and 15 MHz channel BW. 5 MHz was finally removed from the approved CR as no A-MPR was needed for this channel bandwidth

Based on those observations, we would make the following proposal:

Proposal1: Consider NS_13 and NS_14 with 7 MHz channel bandwidth and evaluate the corresponding A-MPR needed for the 2 offsets (3 and 8 MHz).

### REFSENS

For band n5, the difference between the REFSENS values for 5 and 10MHz is 3.2 dB which corresponds to a RBs bandwidth scaling. A similar approach should then be possible to evaluate the REFSENS value for 7 MHz channel bandwidth.

Similarly, for band n26, the difference between the REFSENS values for 5 and 10 MHz is 3 dB. This should correspond to a RBs bandwidth scaling for E-UTRA (for NR, it should be 3.2dB difference). But when comparing the REFSENS values for 10 and 15 MHz, the difference corresponds then to a RBs bandwidth scaling (NR). Again, a similar approach should then be possible to evaluate the REFSENS value for 7 MHz channel bandwidth.

Proposal2: Adopt the following REFSENS values (red) for 7 MHz channel bandwidth:

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 7 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | -98.0 | -96.5 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n26 | 15 | -99.7 | -97.56 | -96.0 | -94.56 | -92.76 | -87.6 | -84.5 | -81.7 |  |  |  |  |
|  | 30 |  |  |  | -94.86 | -92.76 | -87.7 | -84.6 | -81.8 |  |  |  |  |

And the corresponding RB allocation:

| Operating Band | SCS | 3 | 5 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | 25 | 251 | 251 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  | 121 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n26 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  | 121 | 121 | 121 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we analyzed the system parameters impacts and gave an overview of the UE specifications impacts when introducing 7 MHz channel bandwidth. We made the following observations and proposals:

Proposal1: Consider NS_13 and NS_14 with 7 MHz channel bandwidth and evaluate the corresponding A-MPR needed for the 2 offsets (3 and 8 MHz).

Proposal2: Adopt the following REFSENS values (red) for 7 MHz channel bandwidth:

| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 7 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | -98.0 | -96.5 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n26 | 15 | -99.7 | -97.56 | -96.0 | -94.56 | -92.76 | -87.6 | -84.5 | -81.7 |  |  |  |  |
|  | 30 |  |  |  | -94.86 | -92.76 | -87.7 | -84.6 | -81.8 |  |  |  |  |

And the corresponding RB allocation:

| Operating Band | SCS | 3 | 5 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 |  | 25 | 251 | 251 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  | 121 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n26 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  | 121 | 121 | 121 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |  |

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2502870, WF on requirements for 7 MHz channel bandwidth, Ericsson

- R4-B26ah-0045, Way forward on co-existence with Narrow Band Systems in the 806-824 MHz range, Ericsson, ST-Ericsson, Alcatel-Lucent, Intel, KDDI, KT, Motorola-Mobility, Nokia, NSN, Qualcomm, Renesas, SouthernLINC, Sprint, Verizon