3GPP TSG-RAN WG4 Meeting#114bis		R4-2503954

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.24.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RRM requirements for ambient IoT device 1

Document for:	Discussion

Introduction

In RAN4#113 meeting, in SI stage, conclusion in TR 38.769 about RF requirements is agreed. There is no RRM requirements. The WID [1] was approved. This contribution discussed RRM requirements for device 1 in the WI stage as follows.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

Discussion

Discussion on RRM working scope

According to the SI stage and WI stage, device 1 is a simple and low-cost device, which does not have mobility and measurement ability. There is no need to consider RRM measurement, cell-reselection, and handover and so on. Based on RAN2 scope, A-IoT random access, contention-based and contention-free cases are supported. For the contention-based random access, only solution 1(3-step only) is included.  In addition, in SI stage, timing relationships have been studied.

### 6.1.3 Timing relationships A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

RAN1 has still discussed timing relationships in WI stage, including TR2D_min , TR2D_max, TD2R_min , TD2R_max and the starting/ending position of Msg2/Msg3 monitoring. At the same time, there are specific values about timing requirements. In our understanding, whether RAN4 to define these timing requirements needs to discuss. We can wait for RAN1’s conclusion.

From our point of view, RRM working scope should include the requirements for 3-step CBRA and 2-step CFRA to judge and verify the correct behaviour for RA procedures.

Proposal 1: RRM requirements for R19 A-IoT can be included as aspects.

- Correct behaviour for CBRA (3-step) and CFRA (2-step).

Proposal 2: Whether RAN4 to define timing requirements needs to discuss. We can wait for RAN1’s conclusion.

Discussion on detailed RRM requirements

- Contention-based random access

Based on the requirements for 4-step RA type in TS 38.133 and the TR in SI stage about A-IoT random access procedure, contention-based random access including 3-step, the procedures could be as follows. For msg1, when the device identifies the start of its own access occasion, the device has the ability to send one 16-bit random ID by the A-IoT device to the Reader. For msg2, the device may stop monitoring for random access response and shall transmit msg3 if the Random Access Response contains a Random ID corresponding to the transmitted Random ID. If the device does not receive the Random Access Response or the received Random ID is not corresponding to the transmitted Random ID, the device should wait for the next A-IoT paging or subsequent paging.

Proposal 3: Correct behaviour for 3-step CBRA could be as follows.

Correct behaviour when transmitting msg1

-  When the device identifies the start of its own access occasion, the device has the ability to send one 16-bit random ID by the A-IoT device to the Reader.

Correct behaviour when receiving msg2

-  The device may stop monitoring for random access response and shall transmit msg3 if the Random Access Response contains a Random ID corresponding to the transmitted Random ID.

Correct behaviour when not receiving msg2

- If the device does not receive the Random Access Response or the received Random ID is not corresponding to the transmitted Random ID, the device should wait for the next A-IoT paging or subsequent paging.

- Contention-free random access

Contention-free random access includes 2-step, the procedures as follow in figure2. Similar to 3-step CBRA, for msg1, if the received device ID from Reade is corresponding with itself Device ID, the device shall transmit msg1, otherwise, the device should wait for A-IoT paging or subsequent paging. In addition, there is no conclusion about msg2 by RAN2, we also need to wait for RAN2’s conclusion.

Proposal 4: Correct behaviour for 2-step CFRA can be as follow.

Correct behaviour when transmitting msg1

- When the received device ID from Reader is corresponding with itself Device ID, the device shall transmit msg1, otherwise, the device should wait for A-IoT paging or subsequent paging.

Conclusions

In this paper, we provide our views on RRM requirements for ambient IoT device, we have made the following proposals.

Proposal 1: RRM requirements for R19 A-IoT can be included as aspects.

- Correct behaviour for CBRA (3-step) and CFRA (2-step).

Proposal 2: Whether RAN4 to define timing requirements needs to discuss. We can wait for RAN1’s conclusion.

Proposal 3: Correct behaviour for 3-step CBRA could be as follows.

Correct behaviour when transmitting msg1

-  When the device identifies the start of its own access occasion, the device has the ability to send one 16-bit random ID by the A-IoT device to the Reader.

Correct behaviour when receiving msg2

-  The device may stop monitoring for random access response and shall transmit msg3 if the Random Access Response contains a Random ID corresponding to the transmitted Random ID.

Correct behaviour when not receiving msg2

- If the device does not receive the Random Access Response or the received Random ID is not corresponding to the transmitted Random ID, the device should wait for the next A-IoT paging or subsequent paging.

Proposal 4: Correct behaviour for 2-step CFRA can be as follow.

Correct behaviour when transmitting msg1

- When the received device ID from Reader is corresponding with itself Device ID, the device shall transmit msg1, otherwise, the device should wait for A-IoT paging or subsequent paging.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT(Internet of Things) in NR, Huawei, TSG RAN-P meeting #106