3GPP TSG-RAN4 Meeting # 114bis	R4-2503961

Wuhan, China, Apr.7th – 11th, 2025

# Source: 	ZTE Corporation

# Title: 	On PC2 RedCap in FR1

# Agenda item:	7.6.1

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID[1] of PC2 (e)RedCap FR1 to include PC2 FDD (e)RedCap was approved, in which n1 and n3 are selected as example bands. In the previous meeting, there are several agreements on FR1 PC2 TDD (e)RedCap, in which the PC2 TDD (e)RedCap UE was introduced as band agnostic way and PC2 RF requirements for non-RedCap UE are re-used. Consequently, there are less impacts on the specification. Moreover, the draft CR to introduce FR1 PC2 TDD (e)RedCap was endorsed.

For FR1 PC2 FDD (e)RedCap, the agreements are captured in the WF [2], which are:

Issue 1-1-1: MPR/A-MPR requirements Online agreement: - The 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused for the FDD PC2 RedCap UE. No need to define special MPR/A-MPR for FDD PC2 RedCap UE - Note: This agreement also applies to HD-FDD PC2 RedCap UE under the condition that, for NS cases where a filter is assumed, the assumptions of the PC3 HD-FDD filter are also valid for PC2 HD-FDD operation in the completed PC2 FDD bands. Issue 1-2-1: REFSENS for HD-FDD Online agreement: - The existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD. Issue 1-2-2: REFSENS for 2Rx in FD-FDD Online agreement: - 2Rx RSD for 1Tx PC2 is provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1, and this requirement coud apply to RedCap UE for channel BWs up to 20MHz. Issue 1-2-3: REFSENS for 1Rx in FD-FDD Agreement: - Further discuss the following formats for further evaluations - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR - Evaluate the values of ΔR band by band - Note: Other options are not precluded Way forward: It is encouraged for companies to provide analysis for the above options and give the preference in next meeting.

In this contribution, we give some further discussion on PC2 FDD (e)RedCap.

# 2	Discussion

The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

In terms of the previous discussion, the RF requirements agreements for PC2 1Tx FDD (e)RedCap can be summarized in the Table 1.

Table 1. RF requirements agreements for PC2 1Tx FDD (e)RedCap

|  | 1Tx HD-FDD | 1Tx FD-FDD |
| --- | --- | --- |
| Maximum output power Tx power tolerance | Reuse requirements of non-RedCap PC2 FDD band in clause 6.2.1 |  |
| MPR/A-MPR | 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused |  |
| REFSEN/RSD | For both 1Rx and 2Rx: Reuse REFSEN requirements for RedCap PC3 for HD-FDD in sub-clause 7.3I Note: No RSD for HD-FDD | 1Rx: FFS 2Rx: RSD is reused from 1Tx/2Rx non-Redcap UE PC2 FDD in Table 7.3.2-1c |
| Other requirements | No impact |  |

For PC2 HD-FDD (e)RedCap UE, there are no new RF requirements defined, and the existing requirements are reused. In other words, PC2 (e)RedCap requirements can be specified in the band agnostic for HD-FDD.

For PC2 FD-FDD (e)RedCap UE, except how to define the REFSENS for 1Rx in FD-FDD, all of the other requirements are reused from the existing PC2 non-RedCap requirements. It is aligned with our initial understanding in [3] that the only additional work for 1Tx FDD PC2 (e)RedCap UEs is to develop RSD for 1Rx/1Tx FD-FDD case.

In our understanding, the relationships between the REFSENs for PC3/PC2 non-RedCap/(e)RedCap can be shown below:  (PC3 non-RedCap 2Rx REFSEN here means up to 20MHz REFSEN.)

Equation 1). PC2 non-RedCap 2Rx REFSEN = PC3 non-RedCap 2Rx REFSEN + RSD

Equation 2). PC3 (e)RedCap 2Rx REFSEN = PC3 non-RedCap 2Rx REFSEN

Equation 3). PC2 (e)RedCap 2Rx REFSEN = PC3 (e)RedCap 2Rx REFSEN +RSD

Equation 4). PC3 (e)RedCap 1Rx REFSEN = PC3 non-RedCap 2Rx REFSEN + ΔR1R

Then we can get:

For 2Rx:

Equation 5). PC2 (e)RedCap 2Rx REFSEN = PC2 non-RedCap 2Rx REFSEN

For 1Rx:

Equation 6) . PC3 (e)RedCap 1Rx REFSEN = PC2 (e)RedCap 2Rx REFSEN - RSD + ΔR1R

Therefore, for PC2 (e)RedCap 1Rx REFSEN, we think:

Equation 7). PC2 (e)RedCap 1Rx REFSEN = PC3 (e)RedCap 1Rx REFSEN + RSD’

= PC2 (e)RedCap 2Rx REFSEN - RSD + ΔR1R + RSD’

Comparing with PC3(e)RedCap REFSEN, if the RSD requirements for 1Rx and 2Rx for PC2 (e)RedCap are the same, i.e. RSD’ = RSD, then the Equation 7) can be transformed as:

Equation 8). PC2 (e)RedCap 1Rx REFSEN = PC3 (e)RedCap 1Rx REFSEN + RSD

= PC2 (e)RedCap 2Rx REFSEN + ΔR1R

From the Equation 8), we found the two options in the WF are equivalent.

Observation: Comparing with PC3(e)RedCap REFSEN, when the RSD requirements for 1Rx and 2Rx are the same for PC2 (e)RedCap, option 1 and option 2 in the WF are equivalent.

However, there are no discussion for the RSD requirements for 1Rx PC2 (e)RedCap so far, so it might premature to say the RSD requirements for 1Rx and 2Rx are the same although we expect it would be the case. Nevertheless, we think option 1 is a straightforward way.

Proposal: To use Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD for further evaluations.

# 3 Conclusion

In this contribution, we give some further discussion on PC2 FDD (e)RedCap, the conclusions are summarized below:

Observation: Comparing with PC3(e)RedCap REFSEN, when the RSD requirements for 1Rx and 2Rx are the same for PC2 (e)RedCap, option 1 and option 2 in the WF are equivalent.

Proposal: To use Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD for further evaluations.

# 4	Reference

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom

[3] R4-2500939, Discussion on HPUE RedCap in FR1, ZTE Corporation