3GPP TSG-RAN4 Meeting # 114bis	R4-2503965

Wuhan, China, Apr.7th – 11th, 2025

# Title: 	On release independent for 6Rx RF requirement

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.1.3.1

# Document for:	Approval

# 1 Introduction

The issue related to release independent for 6Rx was raised in the RAN4#110bis and RAN4#111 meeting. In WF [1], the agreement is:

Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable.

- From then on, there were no discussion on the 6Rx RF requirements.

- Regarding the 6Rx RF requirements, there are only two RF requirements involved in this area, which are ΔRIB,6R value and ΔTRxSRS requirements. Up to RAN4 #114meeting, the requirements have already stable, which are:

- - For ΔRIB,6R values: ΔRIB,6R values for handheld UE and FWA were agreed in WF [1], and the draft CR for Rx part was endorsed in [2].

- - For ΔTRxSRS requirements: ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 were agreed in WF [3].

It is observed that all of the RF requirements for 6Rx discussion have been completed.

Therefore, we think it is time to discuss the release independent for 6Rx RF requirements, and in this contribution, we give some discussion on it.

# 2	Discussion

When RAN4 discussed the 4Rx and 8Rx feature in the previous releases, it was agreed that 4Rx RF requirements are release independent from R15 while 8Rx RF requirements are release independent from R17. The release independent information are included in the TS38.307. It should be noted that both FWA and handheld form factor UE are applicable for 4Rx, and only FWA/CPE/Vehicle/Industrial devices form factor are applicable for 8Rx.

For 6Rx, it includes handheld UE and FWA UE, and the ΔRIB,6R values for handheld UE and FWA are not the same. In addition to that, the supportive of 6 layer for handheld UE and FWA undergo a very long controversial discussion in demodulation. Consequently, different performance requirements are defined for 6Rx handheld UE and 6Rx FWA, which are:

-	6-layer performance requirements will be only defined for FWA.

-	4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

It should noted that whether 4-layer or 6-layer is not considered when RAN4 discuss the RF requirements.

Similar as 8Rx, where the UE capability for SRS antenna switching beyond 4Rx (like t1r6, t2r6 for 6Rx, t1r8, t2r8 for 8Rx) was introduced in Rel-17, we think it will be no harmful that this 6Rx feature for both handheld UE and FWA can be release independent from Rel-17, and only 4-layer is applicable for handheld UE.

Proposal: Release independent for 6Rx RF requirements for both handheld UE and FWA can be from Rel-17.

# 3 Conclusion

In this contribution, we give some discussions on the release independent for 6Rx RF requirements.

Proposal: Release independent for 6Rx RF requirements for both handheld UE and FWA can be from Rel-17.

# 4	Reference

[1] R4-2414285, WF on 6Rx UE requirements, AT&T

[2] R4-2417094, Introduction of 6Rx (Rx part), ZTE Corporation, Sanechips, Ericsson

[3] R4-2503014, WF on 6Rx requirements, AT&T