3GPP TSG-RAN WG4 Meeting #114bis	R4-2504010

Wuhan, China, April 7th – 11th 2025

Agenda Item:	7.23.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF RX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF TX requirements for SBFD slots, new requirements for SBFD slots, and SBFD general aspects are presented in separate submissions [2-4].

# 2	Co-existence simulation for in-band blocking

2.1	Simulation assumption

# Issue 9: In-band blocking Agreement: - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements. - For FR1 WA BS, FFS following options. - Option 1: a unified blocking requirement to consider both scenarios - Option 2: separate requirements are defined - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). - For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - Option 5: 5% - Option 6: 20% ~ 50% - For FR2-1 WA SBFD BS, FFS following two options. - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used - Option 2: Define in-band blocking interferer signal power to -23 dBm

In RAN4#112bis and RAN4#113 meetings, it was agreed that for FR1 and FR2 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. However, there are still some open issues that require further study.

About simulation assumptions for further study:

- About different grid-shift values, we believe the low grid-shift is a reality and thus the lowest grid-shift scenario should be considered.

The worst-case scenario is a grid-shift as small as possible, to account for uncoordinated deployments. We propose to consider 5%. As comparison, the results for 10% and 100% are also provided.

Here we would like to recall why 100% grid-shift was defined. The 100% grid-shift was originally considered during TDD-TDD co-existence simulations for UE, not because it was a feasible deployment, but because it was a worst-case scenario, where a UE might be close to another operator’s BS but far from its own, making it an extreme case, and it sets the most stringent requirements for TDD-TDD UE coexistence, ensuring robustness. However, in the current scenario for BS coexistence, 100% grid-shift is the best case and is unrealistic and impractical to deploy. Therefore, we propose to consider the worst case 5% grid-shift for in-band blocking simulation, to maintain a functional ecosystem​.

The worst-case scenario should be considered when defining in-band blocking requirements, i.e., grid-shift should be as small as possible, and 5% grid-shift is used.

- About how to use the power level probability distributions to derive the in-band blocking levels:

Find the X% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios. We propose X to be 99.

Find the 99% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios.

In the following sections, our simulation results for FR1 and FR2, as well as the discussion and proposals based on the results, are provided.

2.2	Discussion of simulation results for FR1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR1 scenarios: scenarios 1 (urban macro) and 5 (micro). The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858 [1], and are also summarized in the TR Annex.

2.2.1 FR1 Urban Macro

We consider a coexistence scenario where FR1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%. The grid-shift examples are depicted in Figure 2.2.1-1.

Figure 2.2.1-1: Grid-shift 10% and 100% between two UMa network deployments

Two different values of BS TRP are considered 1) 49 dBm, which is the default value in the assumptions of the SI; 2) 53 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.1-2.

Figure 2.2.1-2: FR1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.1-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases. In addition, it can be observed that when considering TRP 49 dBm and grid-shift 100% (best case assumptions), we are already around 20 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS.

- Considering TRP 53 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

- Considering TRP 49 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.2.2 FR1 Urban Micro

We consider a coexistence scenario where FR1 urban micro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 5% and 10%. The grid-shift examples are depicted in Figure 2.2.2-1.

Figure 2.2.2-1: Grid-shift 10% and 100% between two UMi network deployments

Two different values of BS TRP are considered 1) 46 dBm, which is the default value in the assumptions of the SI; 2) 38 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.2-2.

Figure 2.2.2-2: FR1 UMi: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.2-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases.

Current in-band blocking level for Medium Range BS is -38 dBm, so we can observe that when we consider lower power levels and coordinated deployments (100% grid-shift), there would not be need for higher in-band blocking requirements. However, when the transmission power is higher or the grid-shift is reduced, more stringent requirements would be needed.

- Considering TRP 46 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

- Considering TRP 38 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

- In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.3	Discussion of simulation results for FR2-1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR2-1 urban macro and indoor office scenarios. The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858, and are also summarized in the TR Annex.

2.3.1 FR2-1 Urban Macro

We consider a coexistence scenario where FR2-1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. Similarly to the other discussed scenarios, we are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%.

Two different values of BS TRP are considered 1) 30 dBm, which is the default value in the assumptions of the SI; 2) 40 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.3.1-1.

Figure 2.3.1-1: FR2-1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.3.1-1 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases. It can be observed that when considering TRP 40 dBm and grid-shift 5%, we are around 35 dB above the currently specified in-band blocking requirement (-58 dBm) for FR2-1 Wide Area BS.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

It is worth mentioning that depending on the hypothesis on the beamforming architecture, whether analog or digital/hybrid (no subarray is considered in simulation assumptions from SI Rel.18), the assumption of whether the Rx beamforming gain should be included or not in the in-band blocking interference received at the BS may change. The results in Figure 2.3.1-1 are obtained without including the Rx beamforming gain in the interference received from the adjacent channel.

The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

The above methodology considers the absolute value of the total wideband power received in the UL of the SBFD BS, from the adjacent channel, to derive the in-band blocking power requirement. In TS 38.104 [5], the OTA blocking requirement for FR2 is derived based on a relative value of the in-band blocking power, compared to the declared EISrefsens and considering 6 dB relaxation. The procedure to follow this methodology should be better clarified, by identifying the declared EISrefsens value to be considered in the evaluation. An approach based on simulations to derive the in-band blocking power / wanted signal, where the wanted signal is derived based on simulations, would lack of a fixed declared reference value, which is instead required by the specification.

To further discuss on the methodology to derive the in-band blocking requirement and decide on if it should be based on an absolute value of the in-band blocking power, or on a relative value of the in-band blocking power compared to a declared EISrefsense + 6 dB, which should be further discussed and agreed.

2.4	Compromised option

# Issue 2: In-band blocking and additional emission WF: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

In Section 2.1 to 2.3, we presented our view on setting in-band blocking requirements for SBFD-capable BS. Given that discussions regarding the in-band blocking requirements and the new additional emission requirement have been ongoing for numerous meetings without any progress, we are prepared to compromise to Option 1A, which is captured in the WF [7] approved during RAN4#114.

Regarding in-band blocking requirement and new requirement on additional emission, another option is to define the in-band blocking requirement with grid-shift 20%, while additional emission requirement is not considered in Rel-19. If regulators decide to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

It is important to note that no deployment restrictions are required when the in-band blocking requirement is defined based on simulation results with a 5% or 10% grid-shift. However, when the requirement is defined using simulation results with a 20% or 50% grid-shift, deployment restrictions must be implemented. Specifically, a 20% grid-shift corresponds to 58 meters, while a 50% grid-shift corresponds to 144 meters.

# 3	Impact on BS RX requirements

Dynamic range

In normal uplink, the IoT depends on the expected interference from other UEs. While for SBFD, it actually depends on the interference from other base stations. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

- RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

In-band selectivity and blocking

Other operators BSs may be co-located with the SBFD BS or may be at some distance. There is currently no requirement for supporting co-location of non-synchronized operators in the same band. The nearest requirement is the FDD protection of own receiver requirement. A co-location blocking requirement could be created for SBFD and declaration based; declaring compliance to the requirement would demonstrate sufficient RX performance to enable co-location. However, the requirement would not necessarily ensure that emissions from the co-located BS would not desensitize the SBFD receiver.

- The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

For non-co-located blocking, dedicated co-existence simulations were conducted to determine the expected blocker levels due to other operator site BS during SBFD slots. Results and discussion are presented in Section 2.

Moreover, it worth noting that, the definition of in-band blocking contains an offset, while the simulation results do not take the offset into account. However, the offset cannot be applied outside of both DUD and DU/UD configurations, which means that the proposed interferer level will be the ACS interferer level and needs to be applied directly outside the carrier supporting SBFD, as illustrated in Figure 3-1 and Figure 3-2.

Figure 3-1 Interferer applied outside of SBFD DU configuration

Figure 3-2 Interferer applied outside of SBFD DUD configuration

ACS

Agreement from RAN4#112bis: - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. Content from TR 38.858: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. Agreement from RAN4#113: # Issue 8: ACS requirement Agreement: - FFS on the following: - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Regarding ACS, there is one agreement reached in RAN4#112bis, as shown above. By looking at the content from TR 38.858 [1] regarding conducted ACS, we would like to further refine the agreement to the following wording, which is more precise:

The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

In the WF [6] agreed in RAN4#113, we need to further study if the interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

In our understanding, ACS and in-band blocking are derived based on different methods, and 9 dB offset may not be valid for SBFD.

- ACS is derived from co-existence simulations that evaluate ACIR versus the 5th percentile and mean throughput degradation, identify an ACIR and then decide a split of ACLR and ACS. The ACS is applied in the adjacent channel.

- The in-band blocking is derived from the 99.9th percentile RX power and is applied in the 2nd adjacent channel.

For SBFD, the ACS should be decided based on BS-BS co-existence simulations of mean throughput degradation and ACIR.

The requirement level of ACS should be derived from the simulation results obtained in SI, and RAN4 need to decide which scenario to use to extract the results.

The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

Derive ACS requirement level based on the simulation results using 5% GS in SI.

ICS

Similar to ACS, the requirement level of ICS should be derived from the simulation results obtained in SI, and RAN4 need to decide which scenario to use to extract the results.

The requirement level of ICS should be derived from the BS-BS co-existence simulation results obtained in SI.

Derive ICS requirement level based on the simulation results using 5% GS in SI.

Receiver intermodulation

# Issue 10: Necessity of New RX intermodulation requirement with 1 interfering signal Agreement: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal. # Issue 11: Impact on existing RX intermodulation requirement with 2 interfering signals Agreement: - FFS on the following options: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

The RX intermodulation requirement for conventional TDD consists of a CW signal and a modulated signal that are located in frequency such that their intermodulation produce falls within the receiver frequency range. The RX intermodulation sets a minimum requirement on receiver linearity.

For SBFD, it is possible that a single signal can intermodulate in the receiver with the TX sub-band TX signal. It is necessary to supress such intermodulation sufficiently that the receiver is not desensitized. In this case, transmission in DL sub-band needs to be activated when testing RX IM, and the offsets are chosen in a way that intermodulation product between DL sub-band and interferer falls in the UL sub-band.

The scenarios for 2 input signals and 1 input signal + TX sub-band may need different levels of receiver linearity to be met, depending on the level of the TX sub-band signal in the receiver.

Since receiver linearity should meet both scenarios, in our view, the requirement should be set based on the tougher case.

As defined in Table 7.7.2-1 in TS 38.104 [5], the general intermodulation requirement for WA BS is -52 dBm, which is derived by the corresponding in-band blocking level (-43 dBm) – 9 dB. Hence, for scenario with two input signals, the interfering level for both signals should be equivalent to [the final agreed in-band blocking level] – 9 dB.

Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS +6 dB | -52 |  |
| Medium Range BS | PREFSENS +6 dB | -47 | See Table 7.7.2-2 |
| Local Area BS | PREFSENS +6 dB | -44 |  |
| NOTE 1: PREFSENS depends on the RAT and the BS class. For NR, PREFSENS depends also on the BS channel bandwidth, see clause 7.2. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |  |

Use FR1 WA as example for comparison of the two considered scenarios,

- considering the scenario of 2 inputs signals, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

- considering the scenario of 1 input signal + TX sub-band, the CW interfering level is [the final agreed in-band blocking level] – 9 dB, and the signal level of TX sub-band is -27 dBm, which equals to 53 dBm (TRP) – 80 dB (isolation).

As mentioned above, the in-band blocking level is proposed to be 4.6 dBm, which results that the power of the two input signals will be higher when comparing the two scenarios.

In addition, the signal level of TX sub-band is based on declaration, whereas RAN4 need to define a RX IMD requirement with fixed interfering signals. And for SBFD DU pattern, whether the intermodulated product will fall into the UL subband depends on the placement of the CW interfering signal. Therefore, the scenario for 2 input signals should be used when testing receiver intermodulation.

The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity.

For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	Considering TRP 53 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

Observation 2	Considering TRP 49 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

Observation 3	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

Observation 4	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 5	For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 6	Considering TRP 46 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

Observation 7	Considering TRP 38 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

Observation 8	In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Observation 9	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

Observation 10	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 11	For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 12	RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

Observation 13	The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The worst-case scenario should be considered when defining in-band blocking requirements, i.e., grid-shift should be as small as possible, and 5% grid-shift is used.

Proposal 2	Find the 99% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios.

Proposal 3	Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 4	Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 5	Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

Proposal 6	The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

Proposal 7	To further discuss on the methodology to derive the in-band blocking requirement and decide on if it should be based on an absolute value of the in-band blocking power, or on a relative value of the in-band blocking power compared to a declared EISrefsense + 6 dB, which should be further discussed and agreed.

Proposal 8	Regarding in-band blocking requirement and new requirement on additional emission, another option is to define the in-band blocking requirement with grid-shift 20%, while additional emission requirement is not considered in Rel-19. If regulators decide to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

Proposal 9	The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Proposal 10	The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

Proposal 11	Derive ACS requirement level based on the simulation results using 5% GS in SI.

Proposal 12	The requirement level of ICS should be derived from the BS-BS co-existence simulation results obtained in SI.

Proposal 13	Derive ICS requirement level based on the simulation results using 5% GS in SI.

Proposal 14	The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity.

Proposal 15	For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2504009, Impact on SBFD BS RF TX requirements, Ericsson

R4-2504008, New SBFD BS RF requirements, Ericsson

R4-2504007, SBFD general aspects, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF

R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF