3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504036

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.5.2

Source:	Ericsson

Title:	Discussion on UE capability indication for single Rx chain

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss some signaling-related open issues of fragmented carriers.

# Discussions

## MIMO layers supported in 4Rx mandatory band

In the previous meetings, some discussions were held on the support of 2Rx in the 4Rx mandatory bands, which provided another possibility to free up the Rx path to allow more CCs. The discussion of MIMO layers support is relevant as the observation below is related to the proposals in the next section.

It is noted that the number of layers supported per CC is a capability, the UE needs to only support 4 layers for non-CA transmission (w.r.t single CC), as shown in TS 38.306 and excerpted below. Currently, there is no requirement to support 4 layers per CC for CA for any type of band combination.

Table 1 Excerpt from TS 38.306

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

This is also aligned with the REFSENS requirement for single carrier NR as specified in TS 38.101-1 Clause 7.3.1:

Excerpt from TS.38.101-1 Clause 7.3.2 for single carrier

“For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.”

the REFSENS requirement for 4Rx shall be met, while for CA clause 7.3A.1, shown below

“For operations with 4 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2-2 when MSD > 0.”

the requirements are only modified with the ΔRIB,4R for operations with 4Rx.

- UE can reduce the number of supported layers for NCCA, as there are only requirements of 4 layers for bands with 4Rx in a single configured CC.

Support of 4Rx is of course beneficial for the PCell when configured in a band requiring 4Rx support for a single CC, but this is not a requirement. However, a reduced number of layers for SCells may be acceptable if this means that an additional band could be added to a band combination.

- A reduced number of layers for SCells may be acceptable if this means that an additional band could be added to a band combination.

## Indication of UE capability for supporting “one Rx RF chain”

### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers [2]

### Discussion on semi-statically switch hardware resources procedure and fallback considerations

During the last meeting, various signaling-related proposals were presented by companies. It is mainly focused on how the UE should indicate the fragmented carrier operation. In our opinion, the necessity of whether relaxation is needed, or to what extent the relaxation would be, needs to be decided, taking into account the risk of proliferation of combinations with these relaxed requirements using a single Rx chain, and afterward, decide how to indicate these particular band combinations. The reason is that for instance, a slight relaxation of an RF requirement by a few dB (if necessary) on SCell may be acceptable if this means that a UE would be able to indicate support of a more complex band combination. The NW only configured supported band combinations based on the UE capability and the gNB band support, not RF requirements in 38.101-1.

The next section presents three example configurations, and one practical example use case in Canada to elaborate our observations and proposals.

### Configuration: nXA + nXA with separate RX chains

It is assumed that bands nX and nY share the antenna and have 8 Rx chains available in the frequency range. As shown in Figure 1(a), two carriers in band nX are aggregated, each carrier supports 4 layers (4L). The PCell and SCell are supported by separate Rx chains and no relaxation of PCell REFSENS as proposed in [3].

Figure 1. Example configuration with PCell (dark grey) and SCell (light grey): (a) nXA + nXA.

### Configuration: nXA” + nXA” + nYA with single Rx chains

In case the fragmented carriers in nX are supported by a single RX chain and the number of supported layers is reduced to 2, an additional band nY can be supported with 4 layers without being in a more complex band combination without increasing the total number of Rx chains. The PCell is configured in band nY as shown in Figure 2 assuming that the UE cannot support a PCell in band nX without a REFSENS degradation.

This band combination is indicated in the UE capability, but for the network to correctly parse the capability in nX (the number of layers supported) a condition on the maximum frequency separation must be added. This could be done by specifying a new BW class A’’ only applicable to intra-band NCCA and conditioned on a maximum frequency separation between the lowest and highest carrier edges by 100 MHz (possibly 50 MHz for FDD). A NW not comprehending the new BW class would not configure a band combination containing the new BW class, thus ensuring that this feature does not interfere with the operation of legacy networks. Adding a new frequency separation capability for a BC alone would be non-backward compatible.

Figure 2. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA (assuming that the UE cannot support a PCell in band nX without a REFSENS degradation).

A slight relaxation of REFSENS may be allowed in SCells in band nX. However, REFSENS degradation in PCell is not acceptable, as the PCell coverage may be lost and lead to radio link failure. The detailed rationale is explicitly explained in our RF impact contribution [3].

A limited frequency spacing in band nX will thus enable a single Rx implementation. In this case, a larger number of carriers can be supported with the same 8 Rx chains if the frequency separation of nX is ≤ 50 MHz (based on a single FFT in the FDD case) while maintaining the number of MIMO layers possibly with a slight REFSENS relaxation in SCells. i.e. the spared Rx chains would be used in band nY.

### Configuration: nXA” + nXA” + nYA + nZA with single Rx chains and with modified MIMO capability

Furthermore, an additional configuration in Figure 3 is given below to show that reducing the number of supported layers in one of the bands allows for reducing the number of Rx chains, thus the free-up Rx chains can support more CCs. Note that the fragmented carriers implemented with a single Rx chain only support SCells.

Figure 3. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA + nZA (fragmented carriers implemented with a single Rx chain only support SCells).

Based on the above examples, we observe and propose:

- All band combinations (BC) containing fragmented carriers supported by single Rx chain(s) must be indicated by existing BC capabilities, possibly including a new BW class, including UL and DL association.

All supported band combinations, regardless of how it is implemented, must be included in the UE capability. The UE cannot inform the NW that it has a spare Rx chain and expects the NW’s action based on that information. Use of RF requirements for configuring/releasing/activating/deactivating carriers is not feasible.

However, new RF requirements could enable implementations with fully shared Rx chains and thus more complex band combinations. Therefore, we propose as below:

- Instead of indicating ‘‘a single Rx chain’’, introduce a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios). The capabilities of the corresponding band entry (feature set) would thus be conditioned on a maximum frequency separation of the carriers.

### Practical example use case for Canada: n2 + n66 + n78

We could consider a practical example use case for Canada with X = 2, Y = 66, and Z = 78 as depicted in the figure below. Note that although the n2 and n66 bands are ordered as n66 UL, n2 UL, n2 DL, and n66 DL, this order does not affect the overall argument. The layers supported in each band are provided as examples only.

Figure 4. Example use case in Canada.

## Enabling a single Rx chain

FFS [2]:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

As we elaborated in [3] a band combination received by a single Rx chain with significant REFSENS degradation in PCell DL is not considered feasible. In the case of a fragmented carrier in TDD or FDD with PCell operating in another band (as example configurations presented in the previous section), a slight SCell degradation may be acceptable (e.g. band nX in the previous example) if this means that a more complex band combination could be supported.

Having said that, a few dB of slight degradation in SCell cannot be used as a criterion for the NW to configure or release a cell for a supported band combination. The NW would use existing mechanisms for releasing, configuring, and de/activating carriers. The number of Rx chains is always up to UE implementation, but slightly modified RF requirements could facilitate the use of shared Rx chains for fragmented carriers. As suggested in the previous section, a new BW class capability for the fragmented carriers with restricted frequency separations and MRTD (ensure collocated scenarios) would work.

- Given that a gNB could use the existing mechanisms for releasing, configuring, and de/activating carriers, neither the UE indication of “a single Rx chain” nor the in-gap interference measurement is needed.

# Proposal

In the previous sections, we made the following observations:

Observation 1	UE can reduce the number of supported layers for NCCA, as there are only requirements of 4 layers for bands with 4Rx in a single configured CC.

Observation 2	A reduced number of layers for SCells may be acceptable if this means that an additional band could be added to a band combination.

Observation 3	All band combinations (BC) containing fragmented carriers supported by single Rx chain(s) must be indicated by existing BC capabilities, possibly including a new BW class, including UL and DL association.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Instead of indicating ‘‘a single Rx chain’’, introduce a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios). The capabilities of the corresponding band entry (feature set) would thus be conditioned on a maximum frequency separation of the carriers.

Proposal 2	Given that a gNB could use the existing mechanisms for releasing, configuring, and de/activating carriers, , neither the UE indication of “a single Rx chain” nor the in-gap interference measurement is needed.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2502873, WF on NR FR1 DL fragmented carriers study, MediaTek, OPPO, vivo, ZTE, Nokia, Ericsson. Xiaomi, Apple, Samsung, Huawei, Murata, CHTTL.

- R4-2504037, Discussion on the DL PCell REFSENS degradation of fragmented carriers, Ericsson.