# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504058

# Athens, GR, 17th – 21st February 2025

Agenda Item:	7.14.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN4#114, WF in [1] capturing agreements and open issues for FR1 NTN OTA testing. In this paper, we provide our further views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

The usage scenario and OTA performance metric for NTN handheld UE has been discussed in several previous RAN4 meetings and the following agreements were reached in RAN4#114 [1].

Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE Agreements: - OEM declaration of scenario applicability can be used to declare the supported usage scenario. - Details on declaration and spec language can be further discussed. Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: - RAN4 further discuss the performance metric for UE declared only support scenario 2. - Details on performance metric data processing should also be discussed. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements - Same performance metric for GSO and NGSO handheld UEs.

Satellite operators’ preference on the usage of NR-NTN/IoT NTN handheld UE is to make sure NTN UE to be close to normal NR UE. Therefore, for UE supporting Scenario 1, it was agreed to use the legacy full sphere TRP and TRS as the performance metric for NR-NTN and IoT NTN handheld UE.

For UE only supporting Scenario 2, UE needs to use software to track the satellite, so this UE should be able to align its maximum direction as closely as possible to the satellite. Therefore, the performance metric for such UE could use a conical range centered on the maximum EIRP/EIS direction as the metric. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

Proposal 1: RAN4 should consider defining the partial TRP/TRS with a conical range centred on the maximum EIRP/EIS direction for NTN UE only supporting Scenario 2. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposal was made:

Proposal 1: RAN4 should consider defining the partial TRP/TRS with a conical range centred on the maximum EIRP/EIS direction for NTN UE only supporting Scenario 2. The maximum direction of EIRP/EIS can be based on the measurement or UE declaration.

# 4.	References

- R4-2502370, Way Forward for [114][327] TRP_TRS_Ph3, vivo