3GPP TSG-RAN WG4 Meeting #114bis				                 R4-2504062

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	7.1.1.2.1

Document for:	Approval

# Background

In RAN4#114, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed, where it is agreed that the boundary and integral region of OOBE will be based on original UE CBW rather than the extended UE CBW to meet the ITU requirement.

The major remaining issues are how to determine the extended CBW and how the IBE can be applied, where our views on how to resolve the remaining issues are discussed in this contribution.

# Extended RB numbers

The number of extended RB on each side of the original UE CBW is the key issue to be resolved. By looking at the MPR currently specified for PC3 and PC2, it can be seen that about 1 to 1.5 dB gain can be obtained by transfer to the edge/outer RB to inner RB while there is no gain or very limited gain (~ 0.5 dB) by transfer the edge RB to outer RB allocations. Therefore, it is reasonable to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated. In other words, the “new emission BW” shall be at least 1.5* (legacy UE CBW).

Proposal 1: To obtain sufficient gain in the field, it is proposed to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

From the implementation aspect, it is also critical to simplify the possible configurations on MPR reduction. Suppose 3GPP specification does not limit the possible configuration to a fixed extended RB number set. In that case, the device must comply with all the possible configurations, which will overload the verification and testing effort to support this feature, regardless of how the test setup is defined in 3GPP. It is clear this will negatively impact the adoption of this feature in the field, and thus, it is strongly proposed that the extended RB number be fixed on each side of UE.

Observation 1: Without fixing the possible extended RB configuration, device support for this feature needs to guarantee the MPR performance for all the possible configurations regardless of how the 3GPP conformance is defined, which will negatively impact the adaptation of this feature in the real world.

Proposal 2: RAN4 shall take a fixed 0.5 UE CBW on each side of the original UE CBW as default configuration for Rel-19 MPR reduction feature

Meanwhile, we acknowledge that a more flexible configuration of the extended RB number might be beneficial for certain deployment scenarios. Therefore, to balance between the device complexity while enable the feature for more scenarios, it is proposed that the default extended RB number is 0.5 UE CBW on each side of the original UE CBW for Rel-19, while flexible configuration of RB number can be supported with additional UE capabilities.

Proposal 3: As a compromised proposal, RAN4 can consider adopt a fixed 0.5 UE CBW on each side of the original UE CBW as default value for Rel-19 MPR reduction feature while more flexible configuration of RB number can be supported with additional UE capabilities.

# IBE

As previously agreed, IBE will be applied to the edge between the original UE CBW and the extended UE CBW. However, the exact IBE limitation is still to be discussed within this region. We believe the existing IBE requirement can be applied directly by applying the NRB in IBE equations according to the extended UE CBW, without any special treatment of the original guard band beside the original UE CBW.

Proposal 4: the IBE requirements shall be applied within the extended UE CBW, with existing IBE requirements, but use the NRB with extended UE CBW, the original guard band beside the original UE CBW.

# Limitations on devices types and signaling aspects

In addition to the methodology study, the actual performance gain of supporting this reduced MPR also needs to be evaluated. Focus on the scenario that a narrower UE channel BW within wider BS bandwidth, allocating a device with narrow bandwidth, e.g., Redcap and eRedcap, towards the inner location within an operator’s spectrum block, may not be a common scenario in real life to avoid spectrum fragmentation from the network aspect. In addition, since such an operation scenario requires the operator spectrum block to be wide enough, it may not be feasible for most FDD bands, especially those at sub-1GHz.

Observation 2: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 3: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Observation 4: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 5: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Observation 1: Without fixing the possible extended RB configuration, device support for this feature needs to guarantee the MPR performance for all the possible configurations regardless of how the 3GPP conformance is defined, which will negatively impact the adaptation of this feature in the real world.

Observation 2: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 3: The frequency bands are usually small in FDD bands in sub 1 GHz, making it not feasible to move the UE away from the edge of the band.

Observation 4: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 1: To obtain sufficient gain in the field, it is proposed to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated.

Proposal 2: RAN4 shall take a fixed 0.5 UE CBW on each side of the original UE CBW as default configuration for Rel-19 MPR reduction feature

Proposal 3: As a compromised proposal, RAN4 can consider adopt a fixed 0.5 UE CBW on each side of the original UE CBW as default value for Rel-19 MPR reduction feature while flexible configuration of RB number can be supported with additional UE capabilities.

Proposal 4: the IBE requirements shall be applied within the extended UE CBW, with existing IBE requirements, but use the NRB with extended UE CBW, the original guard band beside the original UE CBW.

Proposal 5: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability.

# References

- R4-2500668	Topic summary for [114][116] NR_ENDC_RF_Ph4_part1 	Moderator (Huawei)

- R4-2503013	WF on power domain enhancement 	Huawei	approved