3GPP TSG-RAN WG4 Meeting # 114bis						         R4-2504209

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.4.1.2

Source: 	OPPO

Title: 	On UE capability and behavior for type 4a and 4b

Document for:	Approval

- Introduction

In RAN4#114 meeting, the UE capability, behaviour and the BS signalling for type 4a and 4b has been fully discussed and a WF[1] has been agreed. In this paper we give further discussion on these topics.

Discussion

The WF agreement have been captured as below.

Issue 2-2-3: UE behavior during initial connection for maxMIMO-layers＞4

Agreement:

- When maxMIMO-layers ＞4, Type 1(collocated). No new BS signaling is needed.

Issue 2-2-4: UE behavior during initial connection for maxMIMO-layers = 4

Agreement:

- RAN4 reaches the consensus on the observations, and send out LS to RAN2 with understanding that RAN2 will design the signalling.

Firstly, it has been agreed that for maxMIMO-layers>4, then type 1 collocated is assumed. However, it has also been agreed that larger then 4 MIMO layers, i.e. 6 or 8 MIMO layers is an optional capability, which means although the UE support intra-band non-collocated EN-DC or NR-CA with 4+2 layers and 4+4 layers, it does not ask UE to be mandatory support 6 or 8 MIMO layers for single cc collocated scenario.

Observation 1: For UE support 6 or 8 MIMO layers for single CC, it should be an optional capability.

Proposal 1: With LS inform RAN2, to include that it is an optional capability that a UE support 6 or 8 MIMO layers for single CC even with the capability of intra-band non-collocated EN-DC or NR-CA with 4+2 and 4+4 layers.

Furthermore, for the UE default behaviour, it has been discussed during the last RAN4 meeting and no consensus has been reached. Although it has been agreed that RAN2 to define such signalling, we still think it better to clarify the behaviour within RAN4 and give RAN2 some preference. For the type 2 capability, the default behaviour is non-collocated, so without the BS signalling, the UE will deem to work under non-collocated mode. From initial connection perspective, we believe this is still an intra-band non-contiguous CA and hence the whole CA procedure as Scell activation should proceed. In such case, the capability to handle larger power imbalance and larger MRTD/MTTD should be a better choice because the UE will not know the exact deployment scenario unless it is connected to the network. Hence in such case, the default behaviour is non-collocated is preferred. Also, this follows the type 2 definition in RAN2 so it will keep the consistency of the signalling design.

Observation 2: Default is non-collocated mode can help to deal with larger power imbalance and MRTD/MTTD.

Observation 3: Default is non-collocated mode keep the signalling design consistent with type 2.

Hence we propose to still use the non-collocated as default UE behaviour and tell RAN2 about the preference.

Proposal 2: Still use the non-collocated as default UE behaviour and tell RAN2 about the preference.

3	Conclusions

In this paper, we give further discussion on the UE behaviour and network signalling and the observations and proposals are as below:

Observation 1: For UE support 6 or 8 MIMO layers for single CC, it should be an optional capability.

Observation 2: Default is non-collocated mode can help to deal with larger power imbalance and MRTD/MTTD.

Observation 3: Default is non-collocated mode keep the signalling design consistent with type 2.

Proposal 1: With LS inform RAN2, to include that it is an optional capability that a UE support 6 or 8 MIMO layers for single CC even with the capability of intra-band non-collocated EN-DC or NR-CA with 4+2 and 4+4 layers.

Proposal 2: Still use the non-collocated as default UE behaviour and tell RAN2 about the preference.

4 References

[1] R4-2502924 	WF on UE requirements for non-collocated scenario, KDDI