3GPP TSG RAN WG4 Meeting #114bis		 R4-2504213

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.5.2

Source: 	MediaTek Inc.

Title: 	Discussion on remaining signalling aspects for DL fragmented carriers study

Document for:	Approval

1. Introduction

UE architecture assumption and initial evaluation based on existing test configurations were agreed and captured in the WF [12]. Signalling design aspects were still open for further discussion. In this contribution, we continue discussion and provide our proposals on the open issues.

2. Discussion

2.1 Continue discussion on signalling aspects

The concept of signalling design can be illustrated as below Figure 1.

Figure 1. Conceptual signalling flow of DL fragmented carriers

The signalling aspects can be discussed into two parts, static part and measure/config part. The contribution provide views for the two topics in the following discussion.

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

In last RAN4 meeting, the issue is left open for further discussion:

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

When UE is attached to a serving cell, UE needs to report its category and supported band combos per network enquiry. UE may further report its capability including RF parameters (include band combos), feature group indicators and UE-NR-Capabilities. For the UE supporting DL NCCA with one Rx RF chain capability, there may be corresponding UE capability to be introduced and reported in the above procedure.

If UE supports one Rx RF chain, the UE would be able to simultaneously support higher order CA combos. Considering UE may also be capable to apply the new capability on more than one band, it needs to be 	considered how the new capability is properly reported. There are some alternatives can also be considered for UE report the new capability as discussed in [17] and copied below:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

Alt. 2: Band combination capability update with the supported number of CCs

Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

To minimize RAN2 signalling design effort, it would be nature to consider Alt 1. Furthermore, it also need to consider bellow different signalling approaches:

Alt 1-1: The capability is reported per CC per band per BC. This is to consider for future extension when the capability applies for the intra-band with three or more CCs to indicate which two adjacent non-contiguous carriers are applied.

Alt 1-2: The capability is reported per band per BC. If the capability is indicated in this approach, then all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode.

With Alt 1-1 or 1-2 above, the signalling design can handle the case that number of intra-band non-contiguous carriers is more than two. Other approaches can be further discussed in RAN4.

Regarding Alt 2, the band combination capability update with the supported number of CCs. For the intra-band NCCA with only two carriers, the approach would be OK. But if there are three or more non-contiguous carriers, this approach does not indicate which two-carriers out of all carriers are implemented with one Rx RF chain. It may be possible to follow similar thinking as alt 1-2 above, all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode. Any two of the carriers are received with sharing Rx path is left to UE implementation.

As for Alt 3, it seems like RAN4 may specify a separate band combo table for the new band combination definition. This may bring more complexity on the spec structure since the current band combo table already include tens of thousands of combinations.

Proposal 1:

We propose legacy band combination reporting scheme + Rx sharing capability indication with further considerations below:

- Alt 1-1: The capability is reported per CC per band per BC. This is to consider for future extension when the capability applies for the intra-band with three or more CCs to indicate which two adjacent non-contiguous carriers are applied.

- Alt 1-2: The capability is reported per band per BC. If the capability is indicated in this approach, then all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode.

If RAN4 does not reach consensus on down selection of the alternatives, it is proposed to capture discussions above in the technical report

Triggering condition to enable ”One Rx RF chain”

When discussing the signalling consideration of the new capability, it would also need to think together with how to determine the mechanism to enable or disable one Rx RF chain configuration as the agreed WF[18] stated:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

  - Triggering condition for UE working in one Rx RF chain is needed

  - Triggering condition for UE working in legacy Rx RF chains is needed

  - The switching should be not be dynamic and be under network control with UE indication of condition.

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers in some cell environments that may seriously degrade the receiving signal quality of wanted carriers.

A new measurement mechanism can be needed to justify the in-gap blocker power level and compare with the wanted signal power levels.

In one Rx RF chain mode, gNB may trigger and configure UE to measure the power levels of total received signals, including out-of-channel, PCC, SCC and in-gap blocker/interference and noise with measurement procedure using existing RSSI measurement method. The network can configure carriers to the UE then UE would understand the in-gap range of configured CCs. Or network can configure the interference measurement bandwidth and the virtual RB location to the UE, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. It is also possible to apply the measurement together with wanted CC so that measurement gap may not be needed.

In the separated Rx RF chain mode, measurement gap based measurement may be required, while in the one Rx RF chain mode, the UE may be able to perform gap-less measurement on both wanted CCs and in-gap range since the supported bandwidth of one UE is at least up to 100MHz to cover both wanted CCs and in-gap range.

Figure 2. Spectrum plot of DL fragmented carriers and in-gap interference

Observation 1: The nominal NR UE can support Channel BW up to 100MHz so it would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 2: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can compare the ratio of in-gap interference level and wanted CCs level to judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be saved with this approach

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Observation 3: There are different signalling methods can be considered. One aspect is UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) to inform the network that network can schedule CA configurations to UE accordingly. Another aspect is UE report all measured information including in-gap interference level to let the network judge whether one Rx RF chain can be enabled and corresponding CA configurations.

In-gap inter-operator interference may not be the only factor that has impact on DL receiving performance when one Rx RF chain mode is configured. During Rx performance evaluation and discussion between companies, it was found self-band Tx leakage also plays an important role. It would be more clear to understand these impacts via evaluating the two factors with the help from system level simulation. The simulation assumptions are listed in the Table 1 below.

Table 1: Assumptions for system level simulation

| Parameter | NR |
| --- | --- |
| Carrier | 2G Hz |
| DL BW | 20MHz |
| UL BW | 20MHz |
| Antenna pattern | TR 38.901 Table 7.3-1 |
| Network layout | 19-sites (57 sectors) with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% |
| Network layout | 19-sites [57 sectors] with wrap-around |
| DL power control | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| NR UE dropping | NR UE: 100% outdoor UE number: DL active UE: 10 UE per cell |
| Pathloss model | TR 38.901 UMA |
| O2I penetration loss | High penetration loss as in TR 38.901 |

It is assumed co-located inter-operators, taking 19-sites with 3 sectors in each site. 10 UEs in every sector per operator. These are illustrated below:

Figure 1. Network layout model for system level simulation

For the UE parameter assumptions, they are listed in the Table 2. Per agreed WF, one Rx RF chain ACS is assumed around 15 to 26 dB so the two values are simulated. 33dB is assumed as performance reference of partial separated Rx RF chain configuration.

Table 2: UE assumptions for system level simulation

|  |  | NR |
| --- | --- | --- |
| UE | Tx ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | Duplexer isolation between T/R | 50dB |
|  | Rx baseband analog filter Tx leakage rejection | 5dB (one Rx RF chain) 15dB (partial separated Rx RF chain) |
|  | Rx ACS | 15, 26dB (one Rx RF chain) 33dB (partial separated Rx RF chain) |

Simulation result is illustrated in below figures.

Figure 2. CDF of DL signal and interference distribution w w/o ACI of Tx leakage

From the simulation results, some important aspects can be observed.

- In-gap interference and Tx leakage both have impact on DL receiving SINR performance. Which one occupies larger portion may depend on UE implementation.

- UE self-judgement mechanism would be needed with considering both in-gap interference and Tx leakage. As long as UE can guarantee pass the adjusted requirements, if specified. How to judge whether UE is capable for one Rx RF chain mode can be left to UE implementation.

- Another sign can be observed, even though there are some degradations on the DL receiving SINR, there are certain percentage of UEs can still benefit from one Rx RF chain, when higher order CA combo is configured, with acceptable degradation.

Observation 4: In-gap interference and Tx leakage both have impact on DL receiving SINR performance when one Rx RF chain is configured.

Proposal 2: The existing RSSI measurement method can be used for the in-gap interference and Tx leakage measurement

Proposal 3: UE self-judgement mechanism considering at least both in-gap interference and Tx leakage is proposed when one Rx RF chain mode is enabled.

Discussion on semi-statically switch hardware resources procedure and fallback considerations

The discussion on fallback configuration was also not yet clarified. The views on the fallback operation from single band point of view and view of higher order configuration are provided though the higher order discussion may beyond the SI scope.

For single band operation, where there are only two CCs which is baseline of the study, when network or UE judge the total interference is too large that one Rx RF chain is not applicable, fallback configuration needs to be applied. The fallback of the DL NCCA configuration shall be a single CC case as illustrated in the Figure 3 below. During the discussion in previous meeting one company was worried about what would the UE behaviour be when the CC1 is not PCC. Regarding the role change, it is already supported by current RRC specification which is not a part of fallback operation. This is also same for a single band with more than two CCs that may be considered as extension of the feature.

Figure 3. Illustration of single band fallback for one Rx RF chain

For two band (band A and B) operation when there are two DL non-contiguous CCs in band A and single DL CC in band B as illustrated in Figure 3 bellow. The assumption here is fallback options are up to network scheduling. When network or UE judge the total interference is too large that one Rx RF chain is not applicable, there may be several possible cases:

Case 1. Network may ask UE to release CC3. For this case, current RRC signalling is already capable so no new signalling is needed for the fallback case. The band A will continue suffer performance degradation from large total interference if the network does not schedule any change on band A.

Case 2. Network may ask UE to release CC2 which is same with that of single band fallback operation. No new signalling is need for this case either.

Case 3. This could be the case if network does not response to UE’s measurement report. UE may apply less Rx path for the band A. e.g., UE only apply 2Rx in band A if band A requires mandatory 4Rx. This is not expected by UE but this could happen since it is up to network how to treat the UE measurement report. It may need to address somewhere in the spec that no UE requirement is expected for the case.

Figure 3. Illustration of two-band fallback for one Rx RF chain per band

Proposal 4: It is up to network how to apply the fallback. However there shall be no UE requirement for the case 1 and 3 (no configuration change on the band configured one Rx RF chain mode) if UE reports unable to continue one Rx RF chain mode but network does not response to this.

Discussion on applicability for PCell and SCell

The fundamental CA configuration is single band with two downlink non-contiguous carriers in the study item. So there must be either one of the two-fragmented carriers to be a PCELL. For the extension application of this capability, it may be possible to be applied for an intra-band three non-contiguous carrier aggregation with two adjacent carriers of them are possible to enable one Rx RF chain capability. So the signalling may also need to consider how to indicate the capability for three or more carriers scenarios.

Proposal 5: For single band operation, there must be either one of the two-fragmented carriers to be a PCELL. For higher order configuration, the DL fragmented carriers with one Rx RF chain can be including one PCell or pure SCells. The signalling design for one Rx RF chain capability shall also consider the higher order configuration usage as well.

Proposal 6: We propose to capture above signalling approach in the last part of the contribution into TR38.755

3. Conclusion

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Proposal 1:

We propose legacy band combination reporting scheme + Rx sharing capability indication with further considerations below:

- Alt 1-1: The capability is reported per CC per band per BC. This is to consider for future extension when the capability applies for the intra-band with three or more CCs to indicate which two adjacent non-contiguous carriers are applied.

- Alt 1-2: The capability is reported per band per BC. If the capability is indicated in this approach, then all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode.

If RAN4 does not reach consensus on down selection of the alternatives, it is proposed to capture discussions above in the technical report

Triggering condition to enable ”One Rx RF chain”

Observation 1: The nominal NR UE can support Channel BW up to 100MHz so it would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 2: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

Observation 3: There are different signalling methods can be considered. One aspect is UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) to inform the network that network can schedule CA configurations to UE accordingly. Another aspect is UE report all measured information including in-gap interference level to let the network judge whether one Rx RF chain can be enabled and corresponding CA configurations.

Observation 4: In-gap interference and Tx leakage both have impact on DL receiving SINR performance when one Rx RF chain is configured.

Proposal 2: The existing RSSI measurement method can be used for the in-gap interference and Tx leakage measurement

Proposal 3: UE self-judgement mechanism considering at least both in-gap interference and Tx leakage is proposed when one Rx RF chain mode is enabled.

Discussion on semi-statically switch hardware resources procedure and fallback considerations

Proposal 4: It is up to network how to apply the fallback. However there shall be no UE requirement for the case 1 and 3 (no configuration change on the band configured one Rx RF chain mode) if UE reports unable to continue one Rx RF chain mode but network does not response to this.

Discussion on applicability for PCell and SCell

Proposal 5: For single band operation, there must be either one of the two-fragmented carriers to be a PCELL. For higher order configuration, the DL fragmented carriers with one Rx RF chain can be including one PCell or pure SCells. The signalling design for one Rx RF chain capability shall also consider the higher order configuration usage as well.

Proposal 6: We propose to capture above signalling approach in the last part of the contribution into TR38.755

TP for TR38.755

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

<Start of TP>

8.1 Signalling consideration from company A

On signalling aspects

The concept of signalling design can be illustrated as below Figure 1.

Figure 5-14. Conceptual signalling flow of DL fragmented carriers

The signalling aspects can be discussed into two parts, static part and measure/config part. The contribution provide views for the two topics in the following discussion.

How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

When UE is attached to a serving cell, UE needs to report its category and supported band combos per network enquiry. UE may further report its capability including RF parameters (include band combos), feature group indicators and UE-NR-Capabilities. For the UE supporting DL NCCA with one Rx RF chain capability, there may be corresponding UE capability to be introduced and reported in the above procedure.

If UE supports one Rx RF chain, the UE would be able to simultaneously support higher order CA combos. Considering UE may also be capable to apply the new capability on more than one band, it needs to be 	considered how the new capability is properly reported. There are some alternatives can also be considered for UE report the new capability as discussed in [17] and copied below:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

Alt. 2: Band combination capability update with the supported number of CCs

Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

To minimize RAN2 signalling design effort, it would be nature to consider Alt 1. Furthermore, it also need to consider bellow different signalling approaches:

Alt 1-1: The capability is reported per CC per band per BC. This is to consider for future extension when the capability applies for the intra-band with three or more CCs to indicate which two adjacent non-contiguous carriers are applied.

Alt 1-2: The capability is reported per band per BC. If the capability is indicated in this approach, then all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode.

With Alt 1-1 or 1-2 above, the signalling design can handle the case that number of intra-band non-contiguous carriers is more than two. Other approaches can be further discussed in RAN4.

Regarding Alt 2, the band combination capability update with the supported number of CCs. For the intra-band NCCA with only two carriers, the approach would be OK. But if there are three or more non-contiguous carriers, this approach does not indicate which two-carriers out of all carriers are implemented with one Rx RF chain. It may be possible to follow similar thinking as alt 1-2 above, all carriers are allowed to apply the requirements that are specified when enabling one Rx RF chain mode. Any two of the carriers are received with sharing Rx path is left to UE implementation.

As for Alt 3, it seems like RAN4 may specify a separate band combo table for the new band combination definition. This may bring more complexity on the spec structure since the current band combo table already include tens of thousands of combinations.

Triggering condition to enable ”One Rx RF chain”

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers in some cell environments that may seriously degrade the receiving signal quality of wanted carriers.

A new measurement mechanism can be needed to justify the in-gap blocker power level and compare with the wanted signal power levels.

In one Rx RF chain mode, gNB may trigger and configure UE to measure the power levels of total received signals, including out-of-channel, PCC, SCC and in-gap blocker/interference and noise with measurement procedure using existing RSSI measurement method. The network can configure carriers to the UE then UE would understand the in-gap range of configured CCs. Or network can configure the interference measurement bandwidth and the virtual RB location to the UE, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. It is also possible to apply the measurement together with wanted CC so that measurement gap may not be needed.

In the separated Rx RF chain mode, measurement gap based measurement may be required, while in the one Rx RF chain mode, the UE may be able to perform gap-less measurement on both wanted CCs and in-gap range since the supported bandwidth of one UE is at least up to 100MHz to cover both wanted CCs and in-gap range.

Figure 5-15. Spectrum plot of DL fragmented carriers and in-gap interference

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can compare the ratio of in-gap interference level and wanted CCs level to judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be saved with this approach

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Observation: The existing RSSI measurement method can be used for the in-gap interference and Tx leakage measurement

In-gap inter-operator interference may not be the only factor that has impact on DL receiving performance when one Rx RF chain mode is configured. During Rx performance evaluation and discussion between companies, it was found self-band Tx leakage also plays an important role. It would be more clear to understand these impacts via evaluating the two factors with the help from system level simulation. The simulation assumptions are listed in the Table 1 below.

Table 5-1: Assumptions for system level simulation

| Parameter | NR |
| --- | --- |
| Carrier | 2G Hz |
| DL BW | 20MHz |
| UL BW | 20MHz |
| Antenna pattern | TR 38.901 Table 7.3-1 |
| Network layout | 19-sites (57 sectors) with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% |
| Network layout | 19-sites [57 sectors] with wrap-around |
| DL power control | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| NR UE dropping | NR UE: 100% outdoor UE number: DL active UE: 10 UE per cell |
| Pathloss model | TR 38.901 UMA |
| O2I penetration loss | High penetration loss as in TR 38.901 |

It is assumed co-located inter-operators, taking 19-sites with 3 sectors in each site. 10 UEs in every sector per operator. These are illustrated below:

Figure 5-16. Network layout model for system level simulation

For the UE parameter assumptions, they are listed in the Table 2. Per agreed WF, one Rx RF chain ACS is assumed around 15 to 26 dB so the two values are simulated. 33dB is assumed as performance reference of partial separated Rx RF chain configuration.

Table 5-2: UE assumptions for system level simulation

|  |  | NR |
| --- | --- | --- |
| UE | Tx ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | Duplexer isolation between T/R | 50dB |
|  | Rx baseband analog filter Tx leakage rejection | 5dB (one Rx RF chain) 15dB (partial separated Rx RF chain) |
|  | Rx ACS | 15, 26dB (one Rx RF chain) 33dB (partial separated Rx RF chain) |

Simulation result is illustrated in below figures.

Figure 5-17. CDF of DL signal and interference distribution w w/o ACI of Tx leakage

From the simulation results, some important aspects can be observed.

- In-gap interference and Tx leakage both have impact on DL receiving SINR performance. Which one occupies larger portion may depend on UE implementation.

- UE self-judgement mechanism would be needed with considering both in-gap interference and Tx leakage. As long as UE can guarantee pass the adjusted requirements, if specified. How to judge whether UE is capable for one Rx RF chain mode can be left to UE implementation.

- Another sign can be observed, even though there are some degradations on the DL receiving SINR, there are certain percentage of UEs can still benefit from one Rx RF chain, when higher order CA combo is configured, with acceptable degradation.

Discussion on semi-statically switch hardware resources procedure and fallback considerations

For single band operation, where there are only two CCs which is baseline of the study, when network or UE judge the total interference is too large that one Rx RF chain is not applicable, fallback configuration needs to be applied. The fallback of the DL NCCA configuration shall be a single CC case as illustrated in the Figure 3 below. During the discussion in previous meeting one company was worried about what would the UE behaviour be when the CC1 is not PCC. Regarding the role change, it is already supported by current RRC specification which is not a part of fallback operation. This is also same for a single band with more than two CCs that may be considered as extension of the feature.

Figure 5-18. Illustration of single band fallback for one Rx RF chain

For two band (band A and B) operation when there are two DL non-contiguous CCs in band A and single DL CC in band B as illustrated in Figure 3 bellow. The assumption here is fallback options are up to network scheduling. When network or UE judge the total interference is too large that one Rx RF chain is not applicable, there may be several possible cases:

Case 1. Network may ask UE to release CC3. For this case, current RRC signalling is already capable so no new signalling is needed for the fallback case. The band A will continue suffer performance degradation from large total interference if the network does not schedule any change on band A.

Case 2. Network may ask UE to release CC2 which is same with that of single band fallback operation. No new signalling is need for this case either.

Case 3. This could be the case if network does not response to UE’s measurement report. UE may apply less Rx path for the band A. e.g., UE only apply 2Rx in band A if band A requires mandatory 4Rx. This is not expected by UE but this could happen since it is up to network how to treat the UE measurement report. It may need to address somewhere in the spec that no UE requirement is expected for the case.

Figure 5-19. Illustration of two-band fallback for one Rx RF chain per band

<End of TP>

4. Reference for the contribution:

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17] 	R4-2415510 On methods for reducing the number of UE Rx chains for fragmented carriers, Apple, RAN4#112bis

[18]	R4-2502873 WF on NR FR1 DL fragmented carriers study, RAN4#114