3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504289

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              Discussion on RRM requirements for NES OD-SSB

Agenda item:			7.25.2.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES OD-SSB. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update (SSB related)

RAN 1# 120 From Tuesday session Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following. - Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-C: The specification has no restriction with regards to overlapping From Wednesday session Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list Agreement For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission. Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain. From Thursday session Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB From Friday session Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Agreement For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options - Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication - Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB” - Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

3 RAN2 agreements update

Agreements on OD-SSB - RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

4 Discussion

4.1 Topic 1: T2 analysis

4.1.1 The necessity, the rule and the conditions

From our understanding, the time interval is not imperative. During the time interval, it is unnecessary to define any measurement requirements from UE side, and NW is not expected to re-trigger another OD-SSB transmission as well. To meet above expections, a rule in terms of the maximum length of such interval, i.e., [X]s and the corresponding conditions can be defined and aligned between NW and UE. We would like give some analyzes regarding T2 based on the following three cases.

- CASE 1: Only FMW-based fast deactivated SCell measurement, below

Fig. 1 Fast deactivated SCell measurement based on FMW

In case 1, the time interval can exist, but what we’d better to focus on are:

- The length of the time interval T2 and the corresponding conditions. From our understanding, the length of the time interval can be limited as [X]s with the following general conditions during [X]s be met:

- The OD-SSB measured remains detectable/The cell remains detectable and measured

- The available measurement results remain valid/the reported measurement results satisfy measurement accuracy (As per RAN4 agreement guidance: The legacy measurement accuracy is applied for OD-SSB based measurement)

In this sense, based on the inspiration from condition of known cell, suggest to consider the following values as baseline:

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [max(5*TOD-SSB, 5*DRX cycles)] s

- Accordingly, based on the [X]s, the following rule can be defined and aligned between NW and UE: NW is not expected to re-trigger another OD-SSB transmission, UE is not expected to perform measurement/detection if the following conditions during [X] s are satisfied :

  - The OD-SSB measured remains detectable/The cell remains detectable and measured

  - The reported measurement results satisfy measurement accuracy

- Subsequently, UE is required to perform fast deactivated SCell measurement based on FMW after T2 when UE re-receive the OD-SSB activation command and the FMW is triggered again. The same requirements defined for the first FMW-based measurement can be reused.

- CASE 2: FMW-based fast deactivated SCell measurement + fast NES SCell activation

In case 2, there is a possibility that T2 is not needed when the FMW-based fast deactivated SCell measurement is anticipated to be used for the subsequent fast NES SCell activation directly, below

Fig. 2 Fast known NES SCell activation based on FMW-based fast deactivated SCell measurement (FR1 as example)

- From our understanding, as one of the probabilities, taking advantage of dense periodicity of OD-SSB, and considering NWs’ energy saving demand and signalling overhead brought by triggering such ON/OFF RS, there could be an expectation from NW that the OD-SSB transmission is activated in scenario #2, and then to be deactivated in the next scenario #2 to achieve fast deactivated SCell measurement and fast SCell activation comprehensively for the bursty data traffic. From this, all the involved procedures, e.g., fast deactivated SCell measurement, NES SCell activation, CSI measurement and reporting are associated with OD-SSB (as per RAN1 agreement guidance: For L1 measurement based on on-demand SSB, periodic, semi-persistent, and aperiodic L1 measurement reports based on existing CSI framework are supported), and naturally, fast NES SCell activation based on FMW-based fast deactivated SCell measurement can be carried out.

  - To be discussed further, if

- one valid measurement reporting based on OD-SSB is sent before receiving NES SCell activation command,

- known condition satisfies,

known NES SCell activation can be completed within more shorten activation delay

  - Fallback to a general case: UE to perform fast+slow mode deactivated SCell measurement, and the known condition can not be met when receiving the NES SCell activation command, the OD-SSB can still be used to quickly activate the SCell since the OD-SSB deactivation command is not sent.

- More meaningful, CASE 3: OD-SSB based direct NES SCell activation. The NES SCell addition/modification with the initial state set to “activated”, OD-SSB based direct NES SCell activation at SCell addition with OD-SSB parameters configured by RRC is considered. In this sense, all the deactivated measurements no matter whether is under fast+slow mode or not can be skipped, T2 is not needed anymore.

Based on the discussions above, we have the following proposals regarding T2.

Proposal 1: T2 is not imperative in the following cases when OD-SSB transmission is activated in scenario #2, and then to be deactivated in the next scenario #2:

- OD-SSB based direct NES SCell activation

- Fast NES SCell activation based on FMW-based fast deactivated SCell measurement

Proposal 2: If T2 is needed, suggest to define the maximum length of the time interval T2, i.e., [X]s, following values to limit T2 can be considered as baseline (inspiration from condition of known cell):

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [max(5*TOD-SSB, 5*DRX cycles)] s

  - Wait for RAN2 conclusions on whether new SMTC period corresponding to OD-SSB period can be used for FMW-based deactivated SCell measurement

Proposal 3: The rule in terms of [X]s and the corresponding conditions can be defined and aligned between NW and UE, and if the following conditions during [X] s are satisfied, NW is not expected to re-trigger another OD-SSB transmission and UE is not expected to perform measurement/detection during [X] s:

  - The OD-SSB measured remains detectable/The cell remains detectable and measured

  - The reported measurement results satisfy measurement accuracy

Proposal 4: UE is required to perform fast deactivated SCell measurement based on FMW after T2 when OD-SSB is reactivated. The same requirements defined for the first FMW-based measurement can be reused

Proposal 5: RAN4 to consider the following scenario: Fast NES SCell activation based on FMW-based fast deactivated SCell measurement.

- FFS: Known condition

4.2 SCell activation

Considering UE receive SCell activation command and OD-SSB activation indication command at the same time, and a separate MAC CE to activate SSB adaptation along with SCell activation.

- For Case 1: There is no always-on SSB transmitted on the SCell supporting OD-SSB operation, but UE can obtain the related parameters information (e.g., time location, frequency and transmit power, etc ) from the specific on-demand SSB configuration, agreed in RAN1. In this sense, OD-SSB can enable faster SCell activation compared to that based on always-on SSB. And from our understanding, if based on the assumption that there is no active FR2 serving cell supporting OD-SSB, the assumption of using OD-SSB resources rather than SMTC periodicity to perform AGC adjustment is also acceptable, that is TSMTC_MAX is not required for AGC in unknown SCell activation, which is in terms of all the active serving cells in the band

Proposal 6: For FR2 unknown SCell activation in case 1, if there is no active FR2 serving cell supporting OD-SSB operation in the band, it can be assumed that the OD-SSB resources can be used to active the NES SCell (w.r.t. obtain AGC) instead of SMTC.

- For Case 2: There is always-on SSB which is periodically transmitted on the cell. RAN1 has the following agreements:

- The frequency location of OD-SSB is the same as the frequency location of AO-SSB at least for the case where always-on SSB is not CD-SSB. But the time domain location can be overlapped and categorized into Time-C1 and Time-C2.

From our understanding, the aim of SSB adaptation is to improve the periodic behavior of AO-SSB, in this sense, SSB adaptation in time domain with denser periodicity after can be deemed as OD-SSB transmission case 2 Time-C1 non-overlapping with only periodicity changes is considered. Both sets of SSBs are NCD-SSBs, allocated at the same frequency location and off sync raster. From this, L1/L3 measurement and T/F tracking during the activation procedure can be further enhanced by using both sets of SSB with smaller periodicity prior to the original AO-SSB. Another feasibility verification is that

RAN1 #119 - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

However, the CSI reporting configuration for the OD-SSB resource configuration is ongoing discussion in RAN1, we’d better to wait for their further conclusions.

Observation 1: SSB adaptation in time domain with denser periodicity after can be deemed as OD-SSB transmission case 2 Time-C1 non-overlapping with only periodicity changes is considered. Both sets of SSBs are NCD-SSBs, allocated at the same frequency location and off sync raster

- Wait for RAN1 conclusions on L1 measurements based on on-demand SSB

- The AO-SSB on a sync raster and is CD-SSB, OD-SSB is not on sync raster and is NCD-SSB. AO-SSB and OD-SSB are located in the same BWP, and UE is not required to measure both AO-SSB and OD-SSB.

In this case, time for RF tuning/retuning to the frequency location of OD-SSB from the location of AO-SSB and the corresponding interruption might be happened and considered during SCell activation procedure.

Proposal 7: If AO-SSB on a sync raster and is CD-SSB, OD-SSB is not on sync raster and is NCD-SSB. Due to the frequency location of two sets of SSBs is different, time for RF tuning/retuning and the corresponding interruption might be happend and considered during SCell activation procedure.

Proposal 8: For FR2 unknown SCell activation in case 2, when AO-SSB and OD-SSB are NCD-SSBs, allocated at the same frequency location, off sync raster with time location Time-C1, RAN4 to consider the following feasibility to define NES SCell activation delay requirements:

- Combining AO-SSB and OD-SSB

- For other cases: Priority-based rules for handling between OD-SSB and AO-SSB

4.3 Cell identification

When there is no AO-SSB in the cell, i.e. Case #1, the UE need to know how to/where to receive the first OD-SSB, some functionality of the existing configurations e.g., referenceCell or ca-slotoffset parameter in the Servingcellconfig can be naturally considered for issue-solving. However, based on RAN1 discussions, SSB position within the burst and locations (e.g., SFN offset and HF index) in the time domain of the first possible burst will be configured and provided by RRC. In this sense, UE will know the timing informaion, and the blind detection overhead of OD-SSB after UE receives the OD-SSB activation command can be reduced.

Proposal 9: No need to extend referenceCell to OD-SSB SCell to skip cell identification

4.4 (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

From our understanding,”known” or ”unkown” is a pure UE-side definition, when SCell being activated is known, that means the UE had identified the SCell and the symbol timing is already acquired, while the NW is not aware of whether itself is a known cell or not to the served UE. Besides, SCell activation/deactivation is UE specific, in case 2, the AO-SSB with large SSB periodicity is prone to be used for energy saving, but the triggered on-demand SSB with dense periodicity is tailored for quick SCell operation for different UE experienced different scenarios. And based on the FMW-based fast deactivated SCell measurement, UE can to report the valid results to NW to assist on determining whether the NES SCell has sufficiently good channel condition to quickly activate for bursty data traffic scheduling. In this sense, there is no difference on the intention of triggering OD-SSB transmission in case 1 and case 2.

Proposal 10: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2): The same approach in Case 1 can be reused

5 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES OD-SSB. The following observations and proposals are obtained:

Observation 1: SSB adaptation in time domain with denser periodicity after can be deemed as OD-SSB transmission case 2 Time-C1 non-overlapping with only periodicity changes is considered. Both sets of SSBs are NCD-SSBs, allocated at the same frequency location and off sync raster

- Wait for RAN1 conclusions on L1 measurements based on on-demand SSB

Proposal 1: T2 is not imperative in the following cases when OD-SSB transmission is activated in scenario #2, and then to be deactivated in the next scenario #2:

- OD-SSB based direct NES SCell activation

- Fast NES SCell activation based on FMW-based fast deactivated SCell measurement

Proposal 2: If T2 is needed, suggest to define the maximum length of the time interval T2, i.e., [X]s, following values to limit T2 can be considered as baseline (inspiration from condition of known cell):

- FR2: [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4

- FR1: [max(5*TOD-SSB, 5*DRX cycles)] s

  - Wait for RAN2 conclusions on whether new SMTC period corresponding to OD-SSB period can be used for FMW-based deactivated SCell measurement

Proposal 3: The rule in terms of [X]s and the corresponding conditions can be defined and aligned between NW and UE, and if the following conditions during [X] s are satisfied, NW is not expected to re-trigger another OD-SSB transmission and UE is not expected to perform measurement/detection during [X] s:

  - The OD-SSB measured remains detectable/The cell remains detectable and measured

  - The reported measurement results satisfy measurement accuracy

Proposal 4: UE is required to perform fast deactivated SCell measurement based on FMW after T2 when OD-SSB is reactivated. The same requirements defined for the first FMW-based measurement can be reused

Proposal 5: RAN4 to consider the following scenario: Fast NES SCell activation based on FMW-based fast deactivated SCell measurement.

- FFS: Known condition

Proposal 6: For FR2 unknown SCell activation in case 1, if there is no active FR2 serving cell supporting OD-SSB operation in the band, it can be assumed that the OD-SSB resources can be used to active the NES SCell (w.r.t. obtain AGC) instead of SMTC.

Proposal 7: If AO-SSB on a sync raster and is CD-SSB, OD-SSB is not on sync raster and is NCD-SSB. Due to the frequency location of two sets of SSBs is different, time for RF tuning/retuning and the corresponding interruption might be happend and considered during SCell activation procedure.

Proposal 8: For FR2 unknown SCell activation in case 2, when AO-SSB and OD-SSB are NCD-SSBs, allocated at the same frequency location, off sync raster with time location Time-C1, RAN4 to consider the following feasibility to define NES SCell activation delay requirements:

- Combining AO-SSB and OD-SSB

- For other cases: Priority-based rules for handling between OD-SSB and AO-SSB

Proposal 9: No need to extend referenceCell to OD-SSB SCell to skip cell identification

Proposal 10: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2): The same approach in Case 1 can be reused

6 References

[1] R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson