3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussions on 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Background

WF R4-2502275 [1] was agreed in RAN4-114 meeting, this contribution provides our views on related open issues.

- General

Definition of MMSE-IRC receiver capability for 8Rx UE

One issue is whether to update the definition of MMSE-IRC receiver capability for 8Rx UE. Following options were made:

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

Current definition for 8Rx receiver is shown as following:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

It is true that current receiver definition and description are limited to SU-MIMO scenario. But MMSE-IRC algorithm is same for SU-MIMO and MU-MIMO scenario. I.e. UE treats co-scheduled UE(s) as noise, which leads to that UE declaring current capability, i,e, Baseline/Simplified SU-MIMO 8Rx Receiver will be able to pass MU-MIMO tests with assumption of  MMSE-IRC algorithm. Therefore, we don’t see the necessary to modify the current receiver capability definition.

Proposal 1: Keep current 8Rx receiver capability definition.

Furthermore, there is an ambiguity that “support of up to x layers” implies that there are two traditional schemes to process the interference layers under MU-MIMO scenario:1) UE detects the co-scheduled layers and acquire the estimated symbols, 2) UE treats the MIMO layers as interference and reject it by IRC algorithm. For the first scheme, the total number of layers is number of Rx, for the second scheme, the total number of layers may exceed number of Rx with advanced precoding schemes from BS side. Hence, we propose to replace “support of up to x layers” by “detect up to x layers” to avoid such ambiguity.

Proposal 2: If it is agreed to modify the capability, RAN4 to modify the capability to following:

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with detecting up to 8 layers with joint 8Rx MIMO detector. Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with detecting only up to 4 layers with two joint 4Rx MIMO detectors.

One issue is whether to define two sets of requirements for MMSE-IRC receiver. The candidate options are:

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

Our view is that it’s a bad precedent to use one requirement to cover two defined receiver types, which is an unfair test since baseline receiver’s requirements will be relaxed.  RAN4 shall respect every UE implementation defined in the spec and define separate requirement to reflect and verify the implementation. Given the criteria has been agreed, we propose option 1 to minimize the unfairness.

Proposal 3: RAN4 to use 0.5dB as threshold for criteria for number of set of requirements definition

- PDSCH for MU-MIMO

The only open issue is whether to consider Rank 4+4:

Issue 1-3-2: Rank 4+4 Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

Rank 4+4 is not typical scheduling in MU-MIMO scenario based on our investigation. MMSE-IRC algorithm has been fully verified under Rank 2+2 test. More interference layers will not change the UE algorithm. We don’t think Rank 4+4 is needed.

Proposal 4: RAN4 doesn’t consider Rank 4+4.

- CQI for inter cell interference

Rank

One of issues are rank for serving cell, it was agreed to choose Rank2 as first priority:

Issue 1-4-2: Rank for serving cell Agreement: - If simulation results show both rank 1 and rank 2 to be feasible for requirement definition, then Rank 2 will be selected.

Based on our simulation results shown in [2], gama values corresponding to all SNR values are larger than 2, which is enough for requirements definition. Hence Rank2 should be selected.

SINR

Our simulation results shows that gama value is sufficient for requirements definition when SINR=-2 dB, hence we propose SINR=-2dB.

Gama

Based on our simulation results shown in [2], gama value is about 2.50, we propose 2.1 considering some margin.

Proposal 5: Adopt following test setups for CQI requirements:

Rank2

SINR=-2dB

Gama=2.1

- Applicability rules

Following options for applicability rules to skip 2/4Rx requirements were made:

Issue 1-5-1: Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers.

Rel-17 2/4Rx and Rel-18 8Rx tests have different test setups especially for Rank, which make it quite unreasonable to define “corresponding 8Rx requirements”. Furthermore, due to the complexity of matrix inversion, performing interference plus noise covariance matrix calculation with 8Rx is much more complicate than that with 2/Rx. 8Rx UE doesn’t need to pass the test that which is easier to the UE if the UE has passed the test which is complicate to the UE.

Proposal 6: Adopt Option1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Following options related independency were made:

Issue 1-5-3: Release independency Proposals: - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17. - Option 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18. - Option 5: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

Noted that support of 8Rx is introduced in Rel-18 but release independent from Rel-17. Considering there is short period between Rel-18 and Rel-19 and MMSE-IRC receiver is basic feature which is always be assumed as baseline receiver for demodulation requirements. It’s reasonable to define the new requirements release independent from Rel-18. To be conservative, the requirements could be optional for Rel-17.

Proposal 7:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Conclusion

This contribution provides our simulation results for 8Rx requirements with interference. The proposals are:

Proposal 1: Keep current 8Rx receiver capability definition.

Proposal 2: If it is agreed to modify the capability, RAN4 to modify the capability to following:

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with detecting up to 8 layers with joint 8Rx MIMO detector. Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with detecting only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 3: RAN4 to use 0.5dB as threshold for criteria for number of set of requirements definition

Proposal 4: RAN4 doesn’t consider Rank 4+4.

Proposal 5: Adopt following test setups for CQI requirements:

Rank2

SINR=-2dB

Gama=2.1

Proposal 6: Adopt Option1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 7:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Reference

[1]  R4-2419784 Way forward for [114][324] NR_demod_Ph5_Part2_UE. Nokia