3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Discussions on BS MMSE-IRC requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.18.3

Document for:	Discussion

Background

This contribution provides our views on open issues for BS MMSE-IRC requirements according to [1], which was agreed in last meeting.

- General

Number of interference for 2Rx

One of issues are number of interference for 2Rx, candidate options are:

Number of interferers for 2Rx - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx. - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following: - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario. - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Based on our simulation results in [1], gain of MMSE-IRC over MMSE-MRC are 1.9dB for HomNet with 2 interference cells and 3.7dB for HetNet with 2 interference cells, which are enough for requirements definition. Noting in Rel-17 performance enhancement WI, according to the evaluation from companies, the gain of CRS-IM receiver over baseline receiver is only 1.5dB, hence we don’t see any technical issues for test setup which shows 1.9dB gain.

Proposal 1: Introduce both HomNet and HetNet scenario with 2 interference cells for 2Rx requirements.

Propagation condition for the serving cell

One of issues are propagation conditions for the serving cell, candidate options are:

Propagation condition for the serving cell - Select between the following options: - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2) - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

Considering large delay spread is suitable for HetNet scenario and small delay spread is suitable for HomNet scenario. Option 3 could be a good choice. For TDLB100-400, the SNR for 2Rx is 15dB according to our simulation results, which is too high for cell edge scenario.

Proposal 2: RAN4 to use TDLA30-10 for HetNet scenario and TDLC300-100 for HomNet.

Manufacturer declaration on MMSE-IRC receiver

One of issues are manufacturer declaration on MMSE-IRC receiver, candidate options are:

Manufacturer declaration on MMSE-IRC receiver - Candidate options: - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

We support proposal 1, in LTE, the similar applicability rules were defined as following:

The requirements apply to the BS supporting the enhanced performance requirements type A.

BS declaration plays a same role as LTE applicability rules, hence it’s proposed to follow the same logic and go with proposal 1.

Proposal 3: RAN4 to introduce new manufactory declaration for BS MMSE-IRC receiver.

Applicability

One of issues are applicability rules, candidate options are:

Applicability - Candidate options on test requirement applicability for different BS classes and test scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded. - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed - Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

Three types Wide Area BS, medium Range BS and Local Area BS are defined in NR. Additionally, one more BS type Home BS is defined in LTE, the applicability rules for LTE BS MMSE-IRC requirements are: “HomNet scenario is not applicable for Local Area BS and Home BS”, RAN4 shall follow this to define the rules as ” HomNet requirements are not applicable for Local Area BS” considering Home BS is not defined in NR spec. LTE didn’t define any applicability rules for HetNet scenario, so we don’t see any reason to do that in NR.

Proposal 4: RAN4 to define the applicability rules that HomNet requirements are not applicable for Local Area BS and HetNet requirements

For the test requirements applicability,  considering  different DIP sets have no impact on baseband processing, it’s enough for BS to pass test with one DIP. Hence proposal 1 is preferred.

Proposal 5: RAN4 to choose proposal1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Conclusion

This contribution provides our views on BS MMSE-IRC requirements with inter-cell interference. The proposals are:

Proposal 1: Introduce both HomNet and HetNet scenario with 2 interference cells for 2Rx requirements.

Proposal 2: RAN4 to use TDLA30-10 for HetNet scenario and TDLC300-100 for HomNet.

Proposal 3: RAN4 to introduce new manufactory declaration for BS MMSE-IRC receiver.

Proposal 4: RAN4 to define the applicability rules that HomNet requirements are not applicable for Local Area BS and HetNet requirements

Proposal 5: RAN4 to choose proposal1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Reference

[1]  R4-2419784 Way forward for [114][324] NR_demod_Ph5_Part2_UE. Nokia