3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504366

Wuhan, China, 07th – 11th April, 2025

Agenda item:	9.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the simplification for BS spec

Document for:	Approval

Introduction

In the RAN#106 meeting, new RAN task is given for RAN4 to investigate how to simplify the BS spec especially from BS transmitter co-existence and co-location spurious emission and co-location blocking requirement perspective. In this contribution, we would like to share some initial views on these requirement.

Focus on the following options: - Maintain tables for co-existence and co-location in each specification but simplify them, e.g., according proposals in [3, 4]. - Create a new standalone specification, capturing all the network-node requirements for co-existence and co-location. - This option does not exclude simplification of existing tables as the same time, as in bullet 1. Other options are not precluded. Companies are encouraged to present the possible implementations of these options during next meetings, e.g., draft CR, etc. Companies are encouraged to identify pros and cons of the above options, considering (but not limited to) the following aspects: - Workload for introduction of a new operating band, - Future-proof consideration, especially considering 6G introduction, - Repetitions reduction among multiple specifications, - Amount of specifications that contain together the complete information necessary to perform conformance testing in the lab. Final solution shall cover network node specifications, including the following aspects: - BS, IAB, and Repeater requirements, - All RATs of interest, - SRAT, and MSR requirements, - Core and Performance parts to be covered, - Conducted and OTA requirements.

Discussion

## 2.1. Coexistence spurious emission requirement

First of all, co-existence spurious emission requirement was defined with the following basic logic from 3G phase, however this basic logic was questioned by ETSI TFES already and RAN4 agreed to have additional objective to further investigate how to specify more appropriate value for these co-existence spurious emission requirement. Basic background to coexistence spurious requirement with E-UTRA or NR is summarized as following:

If the co-existence spurious emission requirement is to protect the uplink receiver performance at BS side, then it is necessary to make sure the received spurious emission at victim node 7dB lower than its noise figure

Figure 2.1. the network layout assumption for 3G phase from our interpretation in 7.4.1.2.3 of TR 25.942

MCL=FreeSpace-Tx_gain-Rx_gain= 87-10-10=67dBc

Where:

- Freespace is is corresponding to the pathloss of 577/2 meter;

- Tx gain and Rx gain is assumed as 10dB instead of 13dBi for some mechanical downtilt considered.

Based on the above information, then for the protection of the victim BS receiver performance, then it is equal to -174dBm/Hz+60+5+67-7=-49dBm;

If the co-existence spurious emission requirement is to protect the UE receiver performance in the downlink direction, then it could be calculated as following:

-174dBm/Hz+60+9+53=-52dBm

Where:

- Freespace is is assumed as 53dBc as worst case considering the MCL for Medium range BS class

- UE noise figure is assumed as 9dB;

- In Rel-19 BS RF enhancement, the MCL is still under the discussion since 67dBc between different BSs are questionable and also under the discussion in this RAN4 meeting and 52dbc between aggressor BS and victim UE are needed to be further investigated. Please find the following initial proposals on how to do the simplification for co-existence spurious emission requirement.

| - Freq offset between coexistence bands edge | - MCL between BSs | - MCL between BS and UE | - Coexistence spurious emission for UL protection - dBm/1MHz | - Coexistence spurious emission for DL protection - dBm/1MHz |
| --- | --- | --- | --- | --- |
| - <= X MHz | - MCL1,1 | - MCL1,2 | - -174dBm/Hz+60+5-7+MCL1,1 | - -174dBm/Hz+MCL1,2+9+60 |
| - >X MHz & <=YMHz | - MCL2,1 | - MCL2,2 | - -174dBm/Hz+60+5-7+MCL2,1 | - -174dBm/Hz+MCL2,2+9+60 |
| - > YMHz | - MCL3,1 | - MCL3,2 | - -174dBm/Hz+60+5-7+MCL3,1 | - -174dBm/Hz+MCL3,2+9+60 |
| - NOTE 1: for NR coexisting with FDD bands, both Coexistence spurious emission for UL protection and DL protection should be fulfilled. - NOTE 2: for NR coexisting with FDD bands, more stringent requirement between Coexistence spurious emission for UL protection and DL protection should be complied with. - NOTE 3: if coexistence bands with its frequency gap less than TBD MHz or overlapping or partially overlapping with coexistence bands, then this requirement is not applicable. - NOTE 4: the default assumption of noise figure for UL protection is 5dB and the default assumption of noise figure for DL protection is 9dB, otherwise the value of nXX band is [TBD]. |  |  |  |  |

## 2.2. Co-location spurious emission requirement

The background information on how this co-location spurious emission requirement is clarified as following.

- The Spurious Emission Level of WA BS should be:

- Spurious Emission LevelWA ＜ kT0 + NFWA  + 10*log10(100kHz) －7dB + MCL

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL

- Where :

- MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, we assume that   the WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

- Please find the following initial proposals on how to do the simplification for co-location spurious emission requirement.

| - Freq offset between coexistence bands edge | - MCL between BSs | - Co-located spurious emission for WA UL protection - dBm/100KHz | - Co-located spurious emission for MR UL protection - dBm/100KHz | - Co-located spurious emission for LA UL protection - dBm/100KHz |
| --- | --- | --- | --- | --- |
| - <= X MHz | - MCL1 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL1 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL1 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL1 |
| - >X MHz & <=YMHz | - MCL2 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL2 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL2 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL2 |
| - > YMHz | - MCL3 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL3 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL3 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL3 |
| - NOTE 1: if coexistence bands with its frequency gap less than TBD MHz or overlapping or partially overlapping with coexistence bands, then this requirement is not applicable. - NOTE 2: to indicate that bands with different noise figure than above. |  |  |  |  |

Conclusions

In this contribution, we shared some initial views on OTA test scope cases in RAN4 and proposals are made as following:

Proposal 1: propose to consider the following framework to simplify the co-existence spurious emission requirement.

| - Freq offset between coexistence bands edge | - MCL between BSs | - MCL between BS and UE | - Coexistence spurious emission for UL protection - dBm/1MHz | - Coexistence spurious emission for DL protection - dBm/1MHz |
| --- | --- | --- | --- | --- |
| - <= X MHz | - MCL1,1 | - MCL1,2 | - -174dBm/Hz+60+5-7+MCL1,1 | - -174dBm/Hz+MCL1,2+9+60 |
| - >X MHz & <=YMHz | - MCL2,1 | - MCL2,2 | - -174dBm/Hz+60+5-7+MCL2,1 | - -174dBm/Hz+MCL2,2+9+60 |
| - > YMHz | - MCL3,1 | - MCL3,2 | - -174dBm/Hz+60+5-7+MCL3,1 | - -174dBm/Hz+MCL3,2+9+60 |
| - NOTE 1: for NR coexisting with FDD bands, both Coexistence spurious emission for UL protection and DL protection should be fulfilled. - NOTE 2: for NR coexisting with FDD bands, more stringent requirement between Coexistence spurious emission for UL protection and DL protection should be complied with. - NOTE 3: if coexistence bands with its frequency gap less than TBD MHz or overlapping or partially overlapping with coexistence bands, then this requirement is not applicable. - NOTE 4: the default assumption of noise figure for UL protection is 5dB and the default assumption of noise figure for DL protection is 9dB, otherwise the value of nXX band is [TBD]. |  |  |  |  |

Proposal 2: propose to consider the following framework to simplify the co-location spurious emission requirement.

| - Freq offset between coexistence bands edge | - MCL between BSs | - Co-located spurious emission for WA UL protection - dBm/100KHz | - Co-located spurious emission for MR UL protection - dBm/100KHz | - Co-located spurious emission for LA UL protection - dBm/100KHz |
| --- | --- | --- | --- | --- |
| - <= X MHz | - MCL1 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL1 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL1 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL1 |
| - >X MHz & <=YMHz | - MCL2 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL2 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL2 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL2 |
| - > YMHz | - MCL3 | - -174dBm/Hz + 5dB + 50dB－7dB + MCL3 | - -174dBm/Hz + 10dB + 50dB－7dB + MCL3 | - -174dBm/Hz + 13dB + 50dB－7dB + MCL3 |
| - NOTE 1: if coexistence bands with its frequency gap less than TBD MHz or overlapping or partially overlapping with coexistence bands, then this requirement is not applicable. - NOTE 2: to indicate that bands with different noise figure than above. |  |  |  |  |

Proposal 3: for the spec simplification update, propose to wait for the outcome of Rel-19 BS RF enhancement.

Proposal 4: for the spec simplification update in NCR or IAB spec, propose to either capture the same simplified table or reference approach to BS spec.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2502306, Way Forward for [114][331] BSRF_Simplify_CoLo_Coex, Ericsson, approved.