3GPP TSG-RAN4 Meeting #114	        R4-2504484 Wuhan, China, 7th April – 11th April 2025

Source: 	Qualcomm Inc.

Title: 	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth

Agenda Item:	7.8.2

Document for:	Discussion

WI Code:	NR_FR1_7MHz_BW-Core

# Introduction

RAN4 agreed that the signalling labels NS_12 and NS_15 should be supported in band n26 for a 7 MHz channel bandwidth [1]. In this proposal, we present a simulation study performed in band n26 with a power class 3 UE highlighting A-MPR behavior.

# Background

The WI objective is to support 7 MHz channel bandwidth in bands n5 and n26 for which the network signalling labels are defined as shown below. It was agreed; there’s no impact on A-MPR due to NS_100 [1].

| NR band | Value of additionalSpectrumEmission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| n5 | NS_100 |  |  |  |  |
| n26 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |

Out of the remaining four network signalling labels applicable to band n26, NS_13 and NS_14 are applied for channel bandwidths up to maximum of 5 MHz and minimum of 10 MHz respectively. Therefore, A-MPR study based only on network signalling labels NS_12  and NS_15 is presented.

Additional requirements to be met by the UE when NS_12 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Additional requirements to be met by the UE when NS_15 is indicated in the cell are tabulated below.

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 7MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# Simulation Study

For analyzing the impact on Tx performance, we simulated the impact on achievable output power. A power class 3 (PC3) transmitter was calibrated to just meet the ACLR with DFT-s-OFDM waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. Other parameters in the simulation set up are as follows :

- Channel Bandwidth = 7 MHz

- SCS = 15 kHz

- Maximum # of RBs = 35

- Band of Operation : n26 (Uplink : 814 – 849 MHz)

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Network Signalling Labels : NS_12 and NS_15

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK

Preliminary simulated results are presented below, further corroboration with bench measurements will be performed.

NS_12 :

Carrier frequency of 817.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. The results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 1: A-MPR behavior for NS_12 @ Fc = 817.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Based on these initial results Table 1 and Table 2 are updated along with the chosen carrier frequency. Further study is required to complete the analysis over the range of carrier frequencies and other sets of waveforms as highlighted in the tables.

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 4 dB and ≤ 6 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 4 dB and ≤ 6 dB respectively.

Proposal 2: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_12.

Table 1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | Fc = 817.5 MHz | TBD | TBD | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | TBD |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | TBD |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | TBD |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | TBD |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | TBD |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | TBD |
| CP-OFDM 256 QAM | ≤ 9.5 |  | TBD |

NS_15 :

Similar analysis is performed for NS_15 with a carrier frequency of 847.5 MHz over the entire allocation space of 7 MHz channel bandwidth. The A-MPR results are presented below for DFT-s-OFDM and CP-OFDM QPSK waveforms.

Figure 2: A-MPR behavior for NS_15 @ Fc = 845.5 MHz for (a) DFT-s-OFDM  and (b) CP-OFDM

Based on these initial results Table 3 and Table 4 are updated with the chosen carrier frequency. Further study is required to complete the analysis over the range of carrier frequencies and other sets of waveforms as highlighted in the tables.

Observation 2: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_15 is ≤ 6.5 dB and ≤ 8 dB respectively.

Proposal 3: A-MPR for NS_15 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 6.5 dB and ≤ 8 dB respectively.

Proposal 4: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_15.

Table 3: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7 MHz | Fc = 845.5 MHz | TBD | TBD | A7 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |

Table 4: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | TBD |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 6.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | TBD |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | TBD |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | TBD |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 8 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | TBD |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | TBD |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | TBD |

# Conclusions

In this contribution A-MPR analysis with network signalling labels NS_12 and NS_15 are presented in band n26 for 7 MHz channel bandwidth. Following observations and proposals are made:

Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 4 dB and ≤ 6 dB respectively.

Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 4 dB and ≤ 6 dB respectively.

Proposal 2: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_12.

Observation 2: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_15 is ≤ 6.5 dB and ≤ 8 dB respectively.

Proposal 3: A-MPR for NS_15 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 6.5 dB and ≤ 8 dB respectively.

Proposal 4: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_15.

# References

[1] 	R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson