3GPP RAN4 Meeting #114bis		R4-2504578

Wuhan, China, April 7 – 11, 2025

Agenda item:	7.14.2.2

Source:	Apple

Title:	On NTN TRP TRS test methodology

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN #103 meeting a new Work Item on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 was approved with the following scope related to IoT/NR NTN devices:

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN) - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop preliminary Measurement Uncertainty (MU) (RAN5)

During the RAN4 #112bis meeting preliminary agreements were reached on the sub-topic related to NTN TRP/TRS methodology [2].  During the RAN4 #113 meeting these were supplemented with the following agreements [3]:

Issue 3-1-2: General view on Usage scenarios for handheld IoT-NTN and NR-NTN Agreements: - For IoT-NTN and NR-NTN, typical usage scenarios are: - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode. - Scenario 2: browsing mode (hand only) - For IoT-NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. - UEs supporting Scenario 1 are tested with head+hand and hand only modes - UEs supporting Scenario 2 only are only tested with hand only mode Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric Issue 3-1-4: non-Handheld IoT-NTN performance metric Agreements: - FFS on partial sphere metrics, such as the radiated power and receive sensitivity within a certain angular range, should be performance metric for non-handheld IoT NTN devices. - This can be considered as a package with issue 3-1-3. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements: - For UEs support both GSO and NGSO, single set of performance metric is required for GSO and NGSO handheld UEs. - For UE support only GSO or NGSO, FFS performance metric should be the same. Issue 3-1-6: How to determine hand only mode for NR/IoT NTN mobile handheld UE (if we call it as Type 1 NTN UEs, usage-mode restriction) Agreements: - RAN4 to discuss how to distinguish UEs which support Scenario 1 (talk and browsing modes) vs. Scenario 2 only (browsing mode only) Issue 3-2-1: detailed test parameters for NR-NTN band n254 Agreements: - The test parameters in Issue 3-3-1 in WF R4-2416590 for NR-NTN n254 OTA testing is confirmed. CR will be provided next meeting. Issue 3-2-2: Detailed test parameters for IoT-NTN Agreements: - The test parameters in Issue 3-3-2 in WF R4-2416590 for IoT-NTN OTA testing is confirmed. CR will be provided next meeting. Issue 3-2-3: Doppler conditions for NTN Agreements: - The following setting can be a starting point for further checking: - NR/IoT NTN radiated requirements shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites. Issue 3-2-4: How to resolve UE antenna circular polarization issue Proposals - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system. - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments. - WF - FFS Issue 3-2-6: Testing time reducing method for NTN Agreements: - FFS on the NTN RTT (Round Trip Time) to reduce TRS measurement time. Feedback from NTN operators and TE vendors requested.

RAN4 #114 made further agreements with the following [4]:

Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE Agreements: - OEM declaration of scenario applicability can be used to declare the supported usage scenario. - Details on declaration and spec language can be further discussed. Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: - RAN4 further discuss the performance metric for UE declared only support scenario 2. - Details on performance metric data processing should also be discussed. Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN) Agreements - Same performance metric for GSO and NGSO handheld UEs. Issue 3-2-1: NTN OTA test system, LP or CP for measurement antenna Agreements: - From a test system perspective, the baseline test method for FR1 NTN is LP-based OTA test system. - For the full sphere performance metric TRP/TRS for NTN scenario 1, it is same as that defined in TR 38.870. Issue 3-2-3: Whether MU is impacted if NTN OTA test system go with LP architecture Agreements: - No additional MU contributor is identified for NTN OTA test system.

This contribution provides our views related to this objective.

# 2	Discussion

Given the clear delineation of the scenarios applicable to NTN devices (see Issue 3-1-2 in [3]) and the agreement by RAN4 to introduce the OEM declaration (see Issue 3-1-1 in [4]), the OEM declaration of scenario applicability can be formulated for the TR.

Proposal 1:	For handheld IoT/NR NTN devices a band-specific OEM declaration of scenario applicability is needed; the declaration is proposed as follows: {Scenario 1: UE supports both talk mode (head+hand, and hand only) and browsing mode; Scenario 2: UE supports browsing mode (hand only)}.  The UE shall report to the test lab the applicability of the DUT to only one scenario.

The applicability of the OTA performance metric for IoT NTN and NR NTN UEs is also determined by the scenario declaration (see Proposal 1).  For handheld UEs declaring support of Scenario 1, the performance metric assumes AoA is likely from any direction / orientation of the device, and RAN4 has already agreed that the full sphere TRP and TRS performance metrics apply.

Observation 1:	For Scenario 2 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

For handheld UEs declaring support of Scenario 2, our past contribution described potential design optimizations that can be applicable [5].  Based on these observations, an argument can be made for defining the Scenario 2 radiated power and radiated sensitivity metrics as partial integrals over a subset of elevation angles, as an example, or other similar schemes.  Given this observation, it follows that the design and performance of such implementation-specific schemes can vary widely depending on form factor design, region, band, and satellite constellation.  Additional band-specific aspects, such as the impact of duplexer separation on the accuracy of such device positioning algorithms, should also be considered by RAN4.

Proposal 2:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

We note that with Issue 3-2-1 [1] RAN4 agreed to a baseline of a full-sphere measurement, and it is our intention to follow this baseline to move the discussion forward.

Considering the implementation-specific nature of the related optimizations, the range of elevation and azimuth angles which have dominant impact on radiated performance of the device under the assumed user interaction scenario can vary by device and band.  RAN4 should consider an additional OEM declaration of the peak direction and range of elevation and azimuth angles over which the Scenario 2 test metric is calculated.

Proposal 3:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration of the initial DUT position and the range of elevation and azimuth angles relative to the initial position over which the performance metric is calculated.  Angular granularity of this declaration can be further discussed.

# 3	Conclusions

This contribution provides our views on the topic of TRP/TRS methodology development for IoT/NR NTN devices.  The following observation and proposal are made:

Observation 1:	For Scenario 2 UEs, RAN4 has already agreed that the full-sphere TRP and TRS metrics apply.

Proposal 1:	For handheld IoT/NR NTN devices a band-specific OEM declaration of scenario applicability is needed; the declaration is proposed as follows: {Scenario 1: UE supports both talk mode (head+hand, and hand only) and browsing mode; Scenario 2: UE supports browsing mode (hand only)}.  The UE shall report to the test lab the applicability of the DUT to only one scenario.

Proposal 2:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles.

Proposal 3:	For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration of the initial DUT position and the range of elevation and azimuth angles relative to the initial position over which the performance metric is calculated.  Angular granularity of this declaration can be further discussed.

# 4	References

RP-240841, “New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3,” Qualcomm, 3GPP RAN #103, March 2024

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #112bis, October 2024

R4-2419808, “WF for [113][325] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #113, November 2024

R4-2502370, “Way Forward for [114][327] TRP_TRS_Ph3,” vivo, 3GPP RAN4 #114, February 2025

R4-2417757, “On NTN TRP TRS test methodology,” Apple, 3GPP RAN4 #113, November 2024

TR38.821, “Solutions for NR to support non-terrestrial networks (NTN)”