3GPP TSG-RAN WG4 Meeting #114bis	R4-2504750

Wuhan, China, Apr 7th – 11th, 2025

Title: 	Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx

Agenda Item:	7.1.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1: Single carrier test

- Agreement

- Only define single carrier test for 6Rx, i.e. no-CA related tests.

Issue 1-2: FR1 TDD test

- Agreement

- Only define test cases for FR1 TDD

    - 30kHz SCS

    - 40MHz CBW

    - TDD UL-DL pattern: 7DS2U, S: 6D+4G+4U

Issue 1-3: PDCCH test

- Agreement

- Not define PDCCH demodulation requirements

Issue 1-4: PBCH test

- Agreement

- Not define PBCH demodulation requirements

Issue 1-5: Test applicability rule for 6Rx test cases

- Way forward

- Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification.

- Option 2:

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Issue 1-6: Specification structure

- Proposals

- Option 1: Add a new clause for 6Rx

- Other options

- Recommended WF

- Postpone the discussion to the CR drafting stage

2 PDSCH requirements for 6Rx

Issue 2-1: Receiver type assumed for UE performance requirements definition

- Agreement

- Single receiver type of MMSE-IRC with joint 6Rx MIMO detector

Issue 2-2: Test scenarios

- Way forward

- Option 1: Test scenarios without interference: PDSCH Mapping Type A

- Option 2: Test scenarios with interference

  - PDSCH demodulation requirements with inter-cell interference

  - PDSCH demodulation requirements with intra-cell inter-user interference

Issue 2-3: Test cases for 6 MIMO layers for FWA

- Agreement

- A single test case for 6 MIMO layers for FWA

Issue 2-4: Test cases for 4 MIMO layers for FWA and Handheld UE

- Way forward

- Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

- Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE

  - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE

  - Other options

Issue 2-5: Tx EVM assumption

- Agreement

- 6% for QPSK/16QAM/64QAM

- 3% for 256QAM

Issue 2-6: Propagation channel and MIMO correlation for 6 MIMO layers

- Way forward

- Option 1: TDLA 30-10 ULA Low

- Option 2: TDLD 30-5 ULA Low

Issue 2-7-1: Propagation channel for 4 MIMO layers

- Way forward

  - Option 1: TDLA30-10

  - Option 2: TDLC300-100

  - Option 3: TDLA30-10 and TDLC300-100

Issue 2-7-2: MIMO correlation for 4 MIMO layers

- Way forward

  - Option 1: ULA Medium

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

    - Option 1a: with β = 0.00718

    - Option 1b: with β = 0.005154

  - Option 2: ULA Low

Issue 2-8: DMRS configuration for 6 MIMO layers case

- Way forward

- Option 1: Rel-15 Type 1 DMRS 2+2

- Option 2: Rel-15 Type 1 DMRS 2+0

- Option 2: Rel-18 eType1 DMRS 1+1

Issue 2-9: DMRS configuration for 4 MIMO layers case

- Agreement

- Rel-15 Type 1 DMRS 1+1 Type 1

Issue 2-10: Antenna configuration for 6 MIMO layers case

- Agreement

- 8Tx6Rx

Issue 2-11: Antenna configuration for 4 MIMO layers case

- Way forward

- Option 1: 4Tx6Rx

- Option 2: 8Tx6Rx

Issue 2-12: MCS for 6 MIMO layers case

- Way forward

- Option 1: MCS13 (Table 2)

- Option 2: MCS 13 as starting point (Table 1)

- Option 3: 256QAM

- Option 4: 64QAM (MCS Table 1) with the same MCS for both CW

- Option 5: MCS17 as starting point (Table 1)

Issue 2-13: MCS for 4 MIMO layer case

- Way forward

- Option 1: MCS13 (Table 2) : 64QAM with CR = 567/1024

- Option 2: MCS 13 as starting point (Table 1) : 16QAM with CR = 490/1024

- Option 3: 256QAM

- Option 4: 64QAM (MCS Table 1)

- Option 5: MCS17 as starting point (Table 1): 64QAM with CR = 438/1024

- Option 6: MCS13 and MCS 17 as starting point

Issue 2-14: N1, N2 for codebook

- Agreement

- 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1)

- 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) (if 4Tx6Rx configuration for 4 MIMO layers case is agreed)

Issue 2-15: Other test parameters for PDSCH test

- Way forward

  - Option 1:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |

  - Other options

3 SDR requirements for 6Rx

Issue 3-1: SDR test

- Agreement

- Define SDR test for 6Rx

Issue 3-2: Tx EVM

- Agreement

- 6% for up to 64QAM

- 3% for 256QAM

- 2.5% for 1024QAM

Issue 3-3: Static channel and precoding for 6Rx

- Proposals

- Option 1: Introduce new 6Rx static channel. The effective channel matrix, which is a combination of the introduced 6Rx static channel and precoding matrix, should maintain good orthogonality across multi-layers. The effective channel matrices should be full rank and leading no zero input at the receiving antenna.

- Option 2:

- For 1T6R (Random precoding):

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

- H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Issue 3-4: SDR requirements for 64QAM and 256QAM

- Agreement

- Reuse the MCS values for 2 and 4 layers

- Define new requirements for 6 layers and FFS the MCS values

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | TBD |
| 6 | 8 | 0.75 | TBD |
| 6 | 8 | 0.4 | TBD |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | TBD |
| 6 | 6 | 0.75 | TBD |
| 6 | 6 | 0.4 | TBD |
| 6 | 4 | 1 | TBD |
| 6 | 4 | 0.8 | TBD |
| 6 | 4 | 0.75 | TBD |
| 6 | 4 | 0.4 | TBD |
| 6 | 2 | 1 | TBD |
| 6 | 2 | 0.8 | TBD |
| 6 | 2 | 0.75 | TBD |
| 6 | 2 | 0.4 | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Issue 3-5: SDR requirements for 1024QAM

- Agreement

- Define SDR requirements for 1024QAM with 2 layers

- FFS Define SDR requirements for 1024QAM with 4 layers

4 CSI requirements for 6Rx

Issue 4-1: CQI test for 6Rx

- Way forward

- Option 1: Not define CQI test

- Option 2: Define CQI test

  - Option 2a: Only define CQI requirements with rank 4 under static channel

  - Option 2b: Define the following CQI test

    - Periodic CQI reporting requirements under AWGN

    - Wideband CQI reporting requirements with inter-cell interference under fading conditions

Issue 4-2: PMI test

- Agreement

- Not define PMI test

Issue 4-3: RI test

- Way forward

- Option 1: Not define RI test for 6 layers

- Option 2: Define RI test for 6 layers