3GPP TSG-RAN WG4 Meeting # 114bis														R4-2505097

Wuhan, China, 7th -11th April, 2025

Agenda item:			7.24.5

Source:	Moderator (Huawei)

Title:	WF on requirements for A-IoT BS and CW

Document for:	Approval

Introduction

This document captures the agreements about the RF requirements for A-IoT BS and CW on Rel-19 WI solutions for Ambient IoT in NR (RP-243326). The topic summary is R4-2500687 (RAN4#114), R4-2504688 (RAN4#114bis). The previous WF is R4-2502859 (RAN4 #114).

Topic #1: A-IoT BS TX

- Issue 1-2-1: RE power control dynamic range

- Agreement:

  - No need to define.

- Issue 1-2-2: Total power dynamic range

- Agreement:

  - No need to define.

- Issue 1-3-1: Transmitter OFF power

- Agreement:

  - Transmitter OFF power spectral density shall be less than [-85] dBm/MHz per antenna connector.

- Issue 1-3-2: Transmitter transient period

- Agreement:

  - Reuse 1-C NR TDD BS Transmitter transient period requirements, that is, 10µs

- Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

    - Option 1: ACLR1 is 40dB and ACLR2 is 50dB for A-IOT BS 1PRB transmission BW with 100kHz offset

    - Option 2: ACLR 1 is 30dB and ACLR2 is 40dB for A-IoT BS 1PRB transmission BW with Foffset=100kHz.

  - FFS on whether frequency offset is needed or not for other PRB numbers

- Issue 1-5-4: Transmitter spurious emissions

- Agreement:

  - Reuse the FR1 NR BS transmitter spurious emission requirement for A-IoT BS

  - Reuse the co-existence and co-location requirements of NR Micro BS

Topic #2: A-IoT BS RX

- Issue 2-1-1: Reference sensitivity level

Copied from TR38.769:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

Copied from TR38.769:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

- Agreement:

  - Consider reference sensitivity requirements for the reader [without frequency shift (R=1)]

  - FFS on side condition of CW for sensitivity requirement

- Issue 2-1-2: LLS assumptions and parameters for SNR

- Agreement:

  - Take the following assumptions for LLS

  - Assumptions adapted can be reported.

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz as baseline |
| Block structure | Depend on RAN1 |
| Channel model | AWGN |
| Number of Tx/Rx chains for Ambient IoT device | 1 |
| Number of BS antenna elements | 1 |
| Reference data rate | 1 kbps (M) |
| Message size | Option 1： {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size Option 2： 96 bits |
| BLER target | 10% |
| Sampling frequency（SFO） | Option 1: Randomly select a value from the range of [104 ppm~105 ppm] for device 1 Option 2: Use Residual SFO based on companies report |
| [OOK/BPSK chip rate] | 7.5 kcps for 2SB 15 kHz Transmission bandwidth |
| Receiver bandwidth | Option1: the same as transmission bandwidth. Option 2：18 kHz (consider SFO and BS filter) for 2SB 15 kHz transmission bandwidth Option 3: first sidelobes as receiver bandwidth Companies can report |
| Waveform (CW) | unmodulated single tone |
| Modulation | Option1：OOK Option 2：BPSK Companies can report |
| Line code | Manchester encoding |
| FEC | 1/3 Convolutional Code |
| ADC bit width | Ideal |
| D2R receiver | coherent receiver |
| Repetition | None |
| Preamble configuration | Companies can report |
| Midamble configuration | companies can report |

- Issue 2-3: ACS

- Agreement:

  - For ACS requirements, use legacy NB-IoT requirements as starting point

  - FFS on the frequency offset for ACS requirement

- Issue 2-4-1: general blocking

- Agreement:

  - FFS whether or not to reuse the in-band blocking requirement from NR Micro BS.

- Issue 2-7-2: Narrowband intermodulation

- Agreement:

  - FFS whether or not to reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement in the licensed band for A-IoT BS Rx narrowband intermodulation requirement.

Topic #3: CW

- Issue 3-1: CW Output power

- Agreement:

  - CW output power <=33dBm, and can be declared by manufacture.

- Issue 3-2-1: ON/OFF power

- Agreement:

  - Define CW OFF power

  - Value TBD

- Issue 3-4: Phase noise

- Agreement:

  - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI.

  - Encourage companies provide phase noise profile

- Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

- Issue 3-6: Spurious emissions

- Agreement:

  - FFS on spurious emissions boundary and corresponding values:

  - Option 1: The spurious emission of CW starts from the boundary of D2R channel bandwidth.

  - Option 2: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz

  - Other options are not precluded.

- Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

    - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

Appendix_TS38.914 work split：

| No. | Section in TS38914 | Volunteer company |
| --- | --- | --- |
| 1 | 5.3 BS channel bandwidth 5.4 Channel arrangement | CMCC |
| 2 | 6.1 General 6.2 Base station output power 6.3 Transmit ON/OFF power | CATT, LGE |
| 3 | 6.4 Transmitted signal quality | Huawei，ZTE |
| 4 | 6.5 Unwanted emissions 6.6 Transmitter intermodulation | CMCC，CATT |
| 5 | 7.1 General 7.2 Reference sensitivity level 7.3 Dynamic range | ZTE, Huawei |
| 6 | 7.4 In-band selectivity and blocking 7.5 Out-of-band blocking | Ericsson |
| 7 | 7.6 Receiver spurious emissions 7.7 Receiver intermodulation 7.8 In-channel selectivity | Ericsson, ZTE |
| 8 | 8.1 General 8.2 CW Output power | Huawei, OPPO |
| 9 | 8.3 Frequency error 8.4 Unwanted emission | Vivo，ZTE |