3GPP RAN WG4 Meeting #115		R4-2505443

Malta, May 19th – 23rd, 2025

Agenda item:	4.2.2

Source:	Apple, Skyworks Solutions Inc.

Title:	Corrections on RedCap UE PC3 FDD band 1Rx REFSENS

WI/SI:	NR_redcap-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

NR RedCap UE was first introduced in Rel-17 [1] where both 1Rx and 2Rx can be supported by all NR bands. For 2Rx RedCap UE, the NR FR1 non-RedCap UE RF requirements are mostly reused, while for 1Rx RedCap UE, the REFSENS were exclusively specified. The REFSENS between 1Rx and 2Rx implementations in general is differed by the so-called Rx diversity gain which is the increase in signal-to-noise/interference ratio by a certain Rx diversity scheme, normally through Maximum Ratio Combining (MRC). The ideal MRC with equal SNR in both main and diversity Rx paths would result in a 3dB gain for 2Rx as compared to 1Rx implementation. However, in practice the SNR in Rx main and diversity paths may not always be equal, especially for FDD bands where the main path is usually subject to higher insertion loss from duplexer as compared to single bandpass filter for diversity path. On the other hand, the Rx noise impact from Tx interference may also be different between the main and diversity paths which would depend on the performance of duplexer/filter and antenna isolation. Therefore, the REFSENS for 1Rx as compared to that for 2Rx in principle should be evaluated on a per band basis. However, considering that there were more than 40 NR bands to be defined for FR1 RedCap UE, RAN4 took a shortcut by leveraging the approach taken for LTE category 1bis UE to apply a constant allowance ΔR1R between 1Rx and 2Rx for all NR bands and all channel bandwidths up to 20 MHz which unfortunately had overlooked and underestimated the Tx noise impact to 1Rx REFSENS for certain FDD bands at 15 MHz and 20 MHz. In this contribution, we propose to correct the RedCap UE 1Rx REFSENS allowance ΔR1R for PC3 FDD bands which are subject to noticeable Tx self-interference.

# Discussion

NR FDD bands subject to noticeable Tx self-interference can be first identified by the UL RB number restriction in REFSENS requirement UL configuration as highlighted in green in Table 2-1 which is intended to minimize the Tx induced noise impact to Rx intrinsic noise. However, for certain FDD bands with narrow duplex spacing and wider channel bandwidths, even with UL RB number restriction, the Tx noise impact to REFSENS may still exist which can be realized if the REFSENS for wider channel bandwidth does not scale with REFSENS at 5MHz by 10*log(NRB/25) and characterized as MSD = REFSENS_BW – (REFSENS_5MHz + 10log(NRB/25)), as also shown in Table 2-1.

For NR FDD bands subject to noticeable Tx self-interference, due to that Tx noise impact to main and diversity Rx paths are not balanced in typical UE implementation, the REFSENS between 1Rx and 2Rx (ΔR1R) would no longer be 3dB and has to be evaluated via reverse engineering based on the Tx noise imbalance amount between main and diversity Rx path.

In this contribution, we provide our 1Rx REFSENS ΔR1R analysis for FDD bands with non-zero MSD as shown in Table 2-1. Our analysis was carried out based on the assumption that the Tx noise in primary Rx path is effectively 6dB higher than the diversity Rx path. Table 2-2 presents the analysis for n5 at 20MHz (which is subject to 4.9dB MSD at PC3 REFSENS) to derive ΔR1R. The resulted ΔR1R is 5.6 dB which is substantially higher than 3 dB as currently specified without the analysis [2].

| Band |  | Channel BW (MHz) / NRB / UL Configurations for REFSENS / MSD (dB) (PC3) with SCS = 15 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | NRB | 25 | 52 | 79 | 106 |
| n1 | NRB_UL | 25 | 50 | 75 | 100 |
|  | MSD | 0 | 0 | 0 | 0 |
| n2 | NRB_UL | 25 | 50 | 50 | 50 |
|  | MSD | 0 | 0 | 0 | 0 |
| n3 | NRB_UL | 25 | 50 | 50 | 50 |
|  | MSD | 0 | 0 | 0 | 0 |
| n5 | NRB_UL | 25 | 25 | 20 | 20 |
|  | MSD | 0 | 0 | 0 | 4.9 |
| n7 | NRB_UL | 25 | 50 | 75 | 75 |
|  | MSD | 0 | 0 | 0 | 0 |
| n8 | NRB_UL | 25 | 25 | 20 | 20 |
|  | MSD | 0 | 0 | 0.6 | 4.9 |
| n12 | NRB_UL | 20 | 20 | 20 |  |
|  | MSD | 0 | 0 | 8.0 |  |
| n13 | NRB_UL | 20 | 20 |  |  |
|  | MSD | 0 | 0 |  |  |
| n14 | NRB_UL | 20 | 20 |  |  |
|  | MSD | 0 | 0 |  |  |
| n18 | NRB_UL | 25 | 25 | 25 |  |
|  | MSD | 0 | 0 | 0 |  |
| n20 | NRB_UL | 25 | 20 | 20 | 20 |
|  | MSD | 0 | 0 | 1.0 | 1.0 |
| n24 | NRB_UL | 25 | 50 |  |  |
|  | MSD | 0 | 0 |  |  |
| n25 | NRB_UL | 25 | 50 | 50 | 50 |
|  | MSD | 0 | 0 | 0 | 0 |
| n26 | NRB_UL | 25 | 25 | 25 | 25 |
|  | MSD | 0 | 0 | 0 | 3.6 |
| n28 | NRB_UL | 25 | 25 | 25 | 25 |
|  | MSD | 0 | 0 | 0 | 1.4 |
| n30 | NRB_UL | 20 | 20 |  |  |
|  | MSD | 0 | 0 |  |  |
| n31 | NRB_UL | 5 |  |  |  |
|  | MSD | 0 |  |  |  |
| n65 | NRB_UL | 25 | 50 | 75 | 100 |
|  | MSD | 0 | 0 | 0 | 0 |
| n66 | NRB_UL | 25 | 50 | 75 | 100 |
|  | MSD | 0 | 0 | 0 | 0 |
| n70 | NRB_UL | 25 | 50 | 75 | 75 |
|  | MSD | 0 | 0 | 0 | 0 |
| n71 | NRB_UL | 25 | 25 | 20 | 20 |
|  | MSD | 0 | 0 | 0.6 | 4.9 |
| n72 | NRB_UL | 5 |  |  |  |
|  | MSD | 0 |  |  |  |
| n74 | NRB_UL | 25 | 25 | 25 | 25 |
|  | MSD | 0 | 0 | 0 | 3.9 |
| n85 | NRB_UL | 20 | 20 | 20 |  |
|  | MSD | 0 | 0 | 8.0 |  |
| n100 | NRB_UL | 25 |  |  |  |
|  | MSD | 0 |  |  |  |
| n105 | NRB_UL | 25 | 25 | 20 | 20 |
|  | MSD | 0 | 0 | 0.6 | 4.0 |

Table 2-1 FDD bands subject to noticeable Tx self-interference

Observation: For NR FDD bands subject to noticeable Tx self-interference, the REFSENS between 1Rx and 2Rx (ΔR1R) would no longer be 3dB and has to be evaluated via reverse engineering based on the Tx noise imbalance amount between main and diversity Rx paths.

| NR Band n5 | 20 MHz |
| --- | --- |
| PC3 2Rx REFSENS (dBm) | -86.8 |
| PC3 MRC noise (dBm) | -85.8 |
| PC3 primary Rx noise (dBm) | -87.7 |
| PC3 diversity Rx noise (dBm) | -87.7 |
| PC3 Primary Tx noise (dBm) | -81.1 |
| PC3 Diversity Tx noise (dBm) | -87.1 |
| PC3 Primary total noise (dBm) | -80.2 |
| PC3 Diversity total noise (dBm) | -84.4 |
| PC3 MRC noise (dBm) | -85.8 |
| PC3 ΔR1R | 5.6 |

Table 2-2 n5 20MHz ΔR1R analysis

Table 2-3 summarizes the ΔR1R for all FDD bands subject to noticeable Tx self-interference to REFSENS where empty value means the allowance is the same as in Table 7.3I.2-1 in TS 38.101-1.

| Band | 15 MHz | 20 MHz |
| --- | --- | --- |
| n5 |  | 5.6 |
| n8 | 3.4 | 5.6 |
| n12 | 6.3 | N/A |
| n20 | 3.6 | 3.6 |
| n26 |  | 5.0 |
| n28 |  | 4.0 |
| n71 | 3.4 | 5.6 |
| n74 |  | 5.2 |
| n85 | 6.3 | N/A |
| n105 | 3.4 | 5.2 |

Table 2-3 ΔR1R for FDD bands subject to noticeable Tx self-interference to REFSENS

Proposal: Capture the ΔR1R values in Table 2-3 above as exceptions to the requirements in Table 7.3I.2-1 in TS 38.101-1 for RedCap UE single antenna port reference sensitivity allowance.

# 3	Conclusion

In this contribution, we propose to correct the RedCap UE 1Rx REFSENS allowance ΔR1R for PC3 FDD bands which are subject to noticeable Tx self-interference.

Observation: For NR FDD bands subject to noticeable Tx self-interference, the REFSENS between 1Rx and 2Rx (ΔR1R) would no longer be 3dB and has to be evaluated via reverse engineering based on the Tx noise imbalance amount between main and diversity Rx paths.

Proposal: Capture the ΔR1R values in the table below as exceptions to the requirements in Table 7.3I.2-1 in TS 38.101-1 for RedCap UE single antenna port reference sensitivity allowance.

| Band | 15 MHz | 20 MHz |
| --- | --- | --- |
| n5 |  | 5.6 |
| n8 | 3.4 | 5.6 |
| n12 | 6.3 | N/A |
| n20 | 3.6 | 3.6 |
| n26 |  | 5.0 |
| n28 |  | 4.0 |
| n71 | 3.4 | 5.6 |
| n74 |  | 5.2 |
| n85 | 6.3 | N/A |
| n105 | 3.4 | 5.2 |

# 4	Reference

RP-210918 “Revised WID on support of reduced capability NR devices” Nokia, Ericsson, 3GPP TSG RAN Meeting #91-e, March 22nd – 26th, 2021

3GPP TS 38.101-1 V19.1.0 (2025-03)