3GPP TSG-RAN WG4 Meeting #115	R4-2505499

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on 6 Layer MIMO Specification Impact

Agenda item:	7.1.2.3.4

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [2] includes the following objective (highlighting added for emphasis):

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During RAN4#114-bis, an LS was sent to RAN2 from RAN4 [3].

RAN4 discussed the support of DL MIMO layers operation for UEs equipped with 6Rx chains for FWA and handheld devices and reached the following conclusions in RAN4 #114: - 6-layer performance requirements will be only defined for FWA (Fixed Wireless Access) UE devices. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications During the following RAN#107 meeting it was agreed to: - Introduce 6 MIMO layers support as an optional feature only for FWA, which is captured as the objective in the latest WID RP-250780. During the RAN4#114bis meeting the following further conclusions have been reached: - Introduce UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH in addition to the existing capability of 2, 4 and 8 maximum DL MIMO layers. - The capability signalling shall be defined as optional. - The signalling details are up to RAN2. Note: In the scope of this WI RAN4 will introduce requirements for both FWA and handheld type of 6Rx devices in RAN4 specifications per agreement in RAN4 #114.

Discussion

As the LS has been sent to RAN2 during RAN4#114-bis and RAN4 will await the reply LS, Nokia would like to make some observations, but no explicit proposals in this document.

As specified in TS 38.101-1, 8 Rx operation is supported exclusively for device form factors such as Fixed Wireless Access (FWA), Customer Premises Equipment (CPE), vehicular, and industrial UEs. In such, 8 Rx capability is not applicable to handheld UEs.

Moreover, it is evident that the maximum number of downlink (DL) MIMO layers that may be utilised is constrained by the number of Rx supported by the UE. Consequently, a UE equipped with a lower Rx count cannot support a greater number of MIMO layers. For instance, a handheld 4Rx UE is not capable of supporting 8 DL MIMO layers. Therefore, 8-layer MIMO operation is likewise only applicable to FWA-type devices, based upon the table in TS 38.101-1.

8 Layer MIMO is already intrinsically defined for FWA-type devices only.

Observing both 38.306 and 38.331 signalling is defined for 2,4, and 8 layers without specifying FWA or handheld UE:

#### – MIMO-Layers The IE MIMO-Layers is used to convey the number of supported MIMO layers. MIMO-Layers information element -- ASN1START -- TAG-MIMO-LAYERS-START MIMO-LayersDL ::= ENUMERATED {twoLayers, fourLayers, eightLayers} MIMO-LayersUL ::= ENUMERATED {oneLayer, twoLayers, fourLayers} -- TAG-MIMO-LAYERS-STOP -- ASN1STOP

Clearly RAN2 will make the final decision for how to include in appropriate specifications; but it seems unnecessary to include information about whether the UE is FWA or handheld in 38.306 or 38.331.

For other MIMO layer definitions in 38.306 or 38.331, it is not defined whether the UE is handheld or FWA.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4, and presents some of the Nokia views on this topic for discussion during RAN4#115.

The following Observations and Proposals were made:

Observation 1: 8 Layer MIMO is already intrinsically defined for FWA-type devices only.

Observation 2: For other MIMO layer definitions in 38.306 or 38.331, it is not defined whether the UE is handheld or FWA.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024

- R4-2505225 LS on DL MIMO layers capabilities for 6Rx UEs