3GPP TSG-RAN WG4 Meeting # 115											                 R4-2505550

Malta, MT, 19th – 23rd May, 2025

Agenda item:			4.5.1

Source:	Moderator (Xiaomi)

Title:	Topic summary for [115][201] Maintenance_up_to_R16

Document for:	Information

Introduction

This document provides summary for RRM related Tdocs submitted to the following AI

4.5.1		Rel-15/16 RRM requirements 	[WI code]

Please kindly take following notes for Tdoc handling in this topic thread.

- Open issues are based on Discussion papers.

- Based on Chair’s guidance, all CRs in this email thread will be first handled in NWM flagging procedure which will be triggered separately.

- Tdocs that are withdrawn or revised in the Tdocs list will not be handled in the summary document or the NWM flagging procedure.

- Cat-A CRs will not be handled in the summary document or the NWM flagging procedure.

Topic #1: Timing adjustment for CA and DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505349 | Qualcomm | Observation 1: There is agreement in all proposals that UL transmission on SCell is stopped in case MTTD is exceeded by a TA command. Observation 2: However, in case that MTTD is exceeded by gradual timing adjustment, the proposals by Apple on the one side and Ericsson and Nokia on the other side seem to differ. Observation 3: Both proposals, i.e., a) capping the gradual timing adjustment and continuing UL transmission on SCell and b) stopping the UL transmission on SCell, should work technically. Observation 4: The proposals differ in which release the problem should be addressed, i.e., Rel-15, Rel-16 and Rel-18. Observation 5: Since existing implementations have handled the lack in specification, there does not seem to be a need for clarification in Rel-18 or earlier. Proposal 1: The lack in specification, if at all, should be addressed in Rel-19. |
| R4-2505894 | Apple | - Defining a behavior based on the condition of meeting a certain requirement is not feasible. - The behavior defined should not restrict UEs that can handle TTD>MTTD. - The UE shall be allowed to continue adjustment if it doesn’t cause UE TX EVM degradation. - If UE stops transmission, NW might not know the reason and assume its link failure. - In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG. - Introducing signaling for this might not be feasible from earlier release. - A more practical solution could be to introduce UE behavior to stop adjustment when TTD exceeds the limit. - In case TTD exceeds MTTD during gradual timing adjustment on any TAG, UE shall continue gradual timing adjustment unless it impacts UE TX EVM, otherwise UE shall stop gradual timing adjustment on that TAG. - In case TTD exceeds MTTD after receiving new TA command, UE may apply the new TA command if UE TX EVM is not degraded, otherwise UE shall stop transmission on SCell or PSCell. - UE shall stop transmission when TAC is received/ stop gradual timing adjustment if TTD exceeds MTTD in the following cases: Inter-band CA Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation Intra-band EN-DC if UE doesn’t support asynchronous operation Inter-band NE-DC if UE doesn’t support asynchronous operation Inter-band NR-DC if UE only supports synchronous operation - Further discuss if the clarification to UE behavior with TTD exceeds a limit is introduced from Rel-15 or Rel-16. |

Open issues summary

Issue 1-1: Applicable release for updated Timing adjustment requirement

- Proposals

  - Option 1 (Apple):

    - Further discuss if the clarification to UE behavior with TTD exceeds a limit is introduced from Rel-15 or Rel-16.

  - Option 2(Qualcomm):

    - The lack in specification, if at all, should be addressed in Rel-19

- Recommended WF

  - The main difference lies in which release the modification may apply. The issue has existed since Rel-15 and lasts for many years.

  - Suggest to further discuss the impact of introducing changes in Rel-15/16 Vs. Rel-19:

    - Assess backward compatibility risks for deployed UEs if clarified in earlier release.

    - Evaluate possible impact to network (e.g. NW adaptation to new UE behaviours)

Issue 1-2: UE timing adjustment after receiving the TA command

Background:

Agreement in RAN4 #114: Issue: UE timing adjustment after receiving the TA command RAN4 to further discuss: As starting point, capture in the specification that the UE may stop transmission on SCell or PSCell If MTTD is exceeded following the TAC. - FFS other details or any other conditions - FFS the applicable release

- Proposals

  - Option 1 (Apple CR R4-2505895):

    - if the UE receives a TA command that causes uplink transmission timing difference to exceed the value in Table 7.5.2.1-1, the UE can apply the TA command if UE TX EVM is not degraded, otherwise the UE shall stop UL transmission on PSCell.

  - Option 2 (Nokia CR R4-2507274):

    - The UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal.

- Recommended WF

  - For both options, the common part is that UE may stop transmit the UL. The difference lies in the condition.

  - Suggest to further discussion the condition if UE can continue to transmit UL.

    - Option 1: UE can continue to transmit if UE TX EVM is not degraded.

    - Option 2: UE can continue to transmit if UE meets all the transmit timing requirements defined in clause 7.1.2

Issue 1-3: UE gradual timing adjustment

Proposals

  - Option 1 (Apple):

    - The UE shall be allowed to continue adjustment if it doesn’t cause UE TX EVM degradation.

    - In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG.

    - In case TTD exceeds MTTD during gradual timing adjustment on any TAG, UE shall continue gradual timing adjustment unless it impacts UE TX EVM, otherwise UE shall stop gradual timing adjustment on that TAG.

  - Option 2 (Nokia CR R4-2507274):

    - If the transmission timing difference after such adjustment is bigger than the maximum transmission timing difference UE may continue adjustment in this TAG. The UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal.

- Recommended WF

  - The main difference between option 1 and option 2 are as below:

    - Issue 1: The condition if UE can continue to transmit UL is different.

      - Option 1: UE can continue to transmit if UE TX EVM is not degraded.

      - Option 2: UE can continue to transmit if UE meets all the transmit timing requirements defined in clause 7.1.2

    - Issue 2: UE behaviour if UE stops UL:

      - Option 1: In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG.

  - For issue 2, as states in observation #4(R4-2505894), it may not be feasible to design new signaling. Therefore, we suggest not to discuss for issue 2 and only discuss for issue 1, which is similar as issue 1-3.

Issue 1-4: Applicable condition for UE gradual timing adjustment

- Proposals

  - Option 1 (Apple):

    - UE shall stop transmission when TAC is received/ stop gradual timing adjustment if TTD exceeds MTTD in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation 
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation

- Recommended WF

  - Further discussion

Topic #2: Clarification on mandatory gap patterns

Companies’ contributions summary

| R4-2506898 | Ericsson | Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch will happen in real field. Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting in both Rel-16 supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns if UE has already implemented such mismatch reporting. Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting. |
| --- | --- | --- |

Open issues summary

Issue 2-1: Clarification on mandatory gap patterns

- Proposals

  - Option 1 (Ericsson):

    - Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch will happen in real field.

    - Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting in both Rel-16  supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns if UE has already implemented such mismatch reporting.

    - Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting.

- Recommended WF

  - Suggest to discuss Proposal 1 first, i.e. whether such conflicting happens in real networks. If Yes, then continue to discuss Proposal 2 and Proposal 3.