3GPP TSG-RAN WG4 Meeting # 115	R4-2505599

Saint Julian’s, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	7.25.6

Source:	MediaTek Inc.

Title:	Discussion on SBFD UE demodulation and CSI requirements

Document for:	Discussion

# Introduction

In Rel-19 subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier has been introduced targeting to better UL coverage compared to TDD and improved UL/DL latency. In Rel-19 solution gNB can perform simultaneous Tx and Rx on the DL and UL subbands while UE remains half duplex. RAN4 needs to specify necessary BS and/or UE demodulation performance requirements to support SBFD operation [1].

# Discussion

In this discussion paper we will focus on UE side impacts of the SBFD feature. In Rel-19 SBFD solution gNB can perform simultaneous Tx and Rx on the DL and UL subbands while UE remains half duplex. We can have some UE related assumptions from TR 38.858 [2].

- FR1 and FR2-1 are both applicable.

- Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

- Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side.

- Resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols.

- Resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots.

- CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots.

Figure 1 shows high level concept of duplex operations in gNB and UE side.

UL

SBFD gNB

HD-UE

DL

HD-UE

Figure 1: Duplex operations of gNB and UE.

Observation #1: In Rel-19 SBFD operation UE remains in half duplex processing.

Figure 2 shows different interference scenarios impacting SBFD operation. Interference scenarios can be identified to 3 categories that are Self-interference (SI) at gNB, Inter-gNB CLI (DL-to-UL) and Inter-UE CLI (UL-to-DL). In UE perspective only the last one, Inter-UE CLI, has an impact.

Observation #2: In Rel-19 SBFD operation UE can be impacted of Inter-UE Cross link interference (CLI).

Figure 2: Interference scenarios of SBFD.

In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier (Figure 3, Left). Uplink subband is defined per symbol granularity in time domain where SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period and exact start and end symbols are defined. Therefore, a slot can consist of SBFD symbols and non-SBFD symbols (Figure 3, Right).

As of now, SBFD subband configuration is agreed to be cell specific, and UE-specific configuration SBFD subband configuration is still under discussion. In SBFD slots/symbols, some UEs may be configured to transmit in the UL-SB while other UEs are configured to receive in the DL-SB and this causes Inter-UE Cross link interference due to UL signal leakage to neighbouring RBs.

Observation #3: In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier.

Observation #4: In Rel-19 SBFD operation some UEs may be configured to transmit in the UL-SB while other UEs are configured to receive in the DL-SB and this causes Inter-UE Cross link interference due to UL signal leakage to neighbouring RBs.

|  | StartSlot EndSlot StartSymbol EndSymbol |
| --- | --- |

Figure 3: Frequency domain allocation of downlink and uplink in Rel-19 SBFD.

In Rel-19 SBFD operation the uplink subband cannot be used for downlink purpose. PDSCH frequency domain allocation is signalled for every slot and in the usual scenario gNB should not indicate PDSCH allocation in uplink subband part. However, if PDSCH allocation would overlap with uplink subband part, assigned PRBs that fall outside DL usable PRBs are invalid and should not be used for PDSCH resource mapping. Also, the number of PRBs for TBS determination is based assigned PRBs within DL usable PRBs only. See Figure 4 left part.

Observation #5: In Rel-19 SBFD operation PDSCH frequency domain should be signalled to DL usable PRBs in typical scenario.

In Rel-19 SBFD operation CSI-RS resource allocation can be done in one full bandwidth frequency allocation for signalling simplicity, and it is UEs responsibility to exclude CSI-RS frequency resources outside DL usable PRBs (Figure 4, right).

Observation #6: In Rel-19 SBFD operation it is UEs responsibility to exclude CSI-RS frequency resources outside DL usable PRBs.

In Rel-19 SBFD operation in CSI reporting the current working assumption is that one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and a second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. The main difference between these two CSI-ReportConfigs is that when CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only, this measurement can observe possible Inter-UE Cross link interference, whereas when associated to non-SBFD symbols only, that measurement do not observe Inter-UE Cross link interference.

Observation #7: In Rel-19 SBFD operation if UE is configured with two CSI-ReportConfig, one will likely observe Inter-UE CLI, whereas the other does not.

| PDSCH frequency domain allocation | CSI-RS frequency domain allocation |
| --- | --- |

Figure 4: PDSCH and CSI-RS frequency domain allocation

In RAN4 scope we can find two possible justifications to introduce UE requirements.

- Demodulation performance impact due to Inter-UE Cross link interference.

- CSI reporting test to exclude CSI-RS frequency resources outside DL usable PRBs.

For the 1) Demodulation performance impact due to Inter-UE Cross link interference, we see that the existing RAN4 specification already implements demodulation tests with MMSE-IRC receiver to check UE can suppress interference.

Observation #8: RAN4 already includes tests to verify interference suppression of UE with MMSE-IRC receiver.

The cross-link interference in SBFD is caused by signal leakage from UL-SB to DL-SB. To model it for demodulation testing would require defining a commonly agreed leakage model. A further complication is that the leakage is affected by the RF functionality of both the interfering and the interfered UE. Furthermore, we believe that the performance is dominated by the RF impacts rather than potential baseband interference suppression.

Observation #9: Testing cross-link interference suppression with baseband demodulation requires first defining a common signal leakage model, which currently does not exist.

Observation #10: The demodulation performance under Cross link interference is likely dominated by the RF impacts rather than potential baseband interference suppression.

Proposal #1: We propose not to introduce demodulation test for interference suppression of UE with MMSE-IRC receiver.

For the 2) CSI reporting test to exclude CSI-RS frequency resources outside DL usable PRBs, we see this strictly as functional test and little value.

Observation #11: Testing UE ability to exclude CSI-RS frequency resources outside DL usable PRBs is functional test.

Proposal #2: We propose not to introduce functional CSI reporting test to verify UE can exclude CSI-RS frequency resources outside DL usable PRBs.

# Conclusion

In this contribution we provide the view on the SBFD UE demodulation and CSI requirements. The following observations and proposals are made:

Observation #1: In Rel-19 SBFD operation UE remains in half duplex processing.

Observation #2: In Rel-19 SBFD operation UE can be impacted of Inter-UE Cross link interference (CLI).

Observation #3: In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier.

Observation #4: In Rel-19 SBFD operation some UEs may be configured to transmit in the UL-SB while other UEs are configured to receive in the DL-SB and this causes Inter-UE Cross link interference due to UL signal leakage to neighbouring RBs.

Observation #5: In Rel-19 SBFD operation PDSCH frequency domain should be signalled to DL usable PRBs in typical scenario.

Observation #6: In Rel-19 SBFD operation it is UEs responsibility to exclude CSI-RS frequency resources outside DL usable PRBs.

Observation #7: In Rel-19 SBFD operation if UE is configured with two CSI-ReportConfig, one will likely observe Inter-UE CLI, whereas the other does not.

Observation #8: RAN4 already includes tests to verify interference suppression of UE with MMSE-IRC receiver.

Observation #9: Testing cross-link interference suppression with baseband demodulation requires first defining a common signal leakage model, which currently does not exist.

Observation #10: The demodulation performance under Cross link interference is likely dominated by the RF impacts rather than potential baseband interference suppression.

Proposal #1: We propose not to introduce demodulation test for interference suppression of UE with MMSE-IRC receiver.

Observation #11: Testing UE ability to exclude CSI-RS frequency resources outside DL usable PRBs is functional test.

Proposal #2: We propose not to introduce functional CSI reporting test to verify UE can exclude CSI-RS frequency resources outside DL usable PRBs.

# Reference

- RP-250328, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Charter Communications, Qualcomm, Fraunhofer HHI, Fraunhofer IIS

- 3GPP TR 38.858 V18.2.0 (2024-12), Study on Evolution of NR Duplex Operation (Release 18)