3GPP TSG-RAN WG4 Meeting #115	R4-2505603

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.21.4

Source:	CAICT

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

1	Introduction

In this contribution, we provide our views on left FFS in following issues,

- Issue 2-1: Metrics/KPIs for beam ID prediction

- Issue 2-3: Relative RSRP accuracy

2	Discussion

2.1 Metrics/KPIs for beam ID prediction

In RAN4 #114bis meeting, following agreements had been achieved.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

The existing definition guarantees that the actual best beam among the top-K predicted beams has an RSRP no less than a certain threshold. However, the actual best beam from the top-K predicted beams may not be the top-1 predicted beam. In the reporting of beam prediction results, ranking of beams also provides significant information to NW side. If the NW directly selects the top-1 predicted beam based on UE reporting, there may be a situation where the test is passed with the predicted top-K beams meet the threshold, but the NW finally selects a beam with an RSRP below the threshold.

Based on above discussion, we think it is necessary to consider top-N predicted beams in the success rate evaluation.

Proposal 1: suggest to consider top-N (N</=K) predicted beams in the success rate evaluation.

2.2 Relative RSRP accuracy

Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Regarding the FFS highlighted in yellow, the discussion on reporting details is within RAN1 scope. RAN1 had following agreements on this issue.

Agreement (RAN 1 #120) For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement (RAN 1 #117) Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

For BM-case 2, and beams that are not configured with measurement of BM-case 1, it is straightforward that reported value can only be predicted L1-RSRP. The unclear part exists for the beams that are configured for measurement of BM-case 1. Based on our understanding of RAN1 agreements, the reported value could be either measured or predicted RSRP, which is up to UE-vendor implementation.

Observation 1: For beams that are configured for measurement under BM-case 1, the reported value could be either measured or predicted RSRP, up to vendor implementation.

Conclusion

In this contribution, RRM core requirements for beam management are discussed with following proposal and observation.

Proposal 1: suggest to consider top-N (N</=K) predicted beams in the success rate evaluation.

Observation 1: For beams that are configured for measurement under BM-case 1, the reported value could be either measured or predicted RSRP, up to vendor implementation.