# 3GPP TSG-RAN WG4 Meeting #115                                 R4-2505691

# Malta, MT, 19th – 23th May, 2025

Title: 	Discussion on performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Source: 	OPPO

Agenda item:	7.18.5

Document for:	Approval

Introduction

According to the existing RAN4 discussion, the core requirements for FR2-1 L3 measurement delay was almost completed. In last meeting, the test case list for FR2-1 L3 measurement delay was discussed and agreed. In this contribution, we will provide our views on the test case list and configurations for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor and CSSF.

Discussion

Optimizing Rx beam sweeping factor

In last meeting, the following test case list was agreed for BSF optimization:

Issue 2-1-1: TC list for Rx BSF optimization Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Samsung. 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown); OPPO. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown); Intel.

For the test case list agreed as above, to reduce the number of test cases, we slightly prefer to select a subset of the cases, e.g., choose scenario 1 and 4 to test the intra-frequency and inter-frequency measurement requirements. But if companies have strong view to guarantee the coverage of test cases, we are also fine to define test cases for all the scenarios 1/2/3/4/5/7, and for scenario 5, both intra-frequency and inter-frequency handover can be defined. For each of the test case, single set of test configurations can be defined.

Regarding the test configuration, for scenario 5/7, since faster beam sweeping always applies during handover and RRC re-establishment, the existing configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

For scenario 1/2/3/4, since the trigger condition of FBS based measurement is defined, besides taking the existing test configurations as baseline, the transmission power of serving cell from the start of T2 shall be set to meet the trigger condition of FBS based measurement. Regarding the quit condition, we don’t think it needs to be tested since the requirements after quitting are same as the legacy. So the test consists of two successive time periods, with time duration of T1 and T2 respectively, and there are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2). During time duration T1, the UE shall not have any timing information of Cell 2. At the beginning of T2, Cell 2 starts transmission and the transmission power of Cell 1 is set to meet the trigger condition of FBS based measurement. UE perform FBS based measurement during T2.

Proposal 1: For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

Proposal 2: For BSF optimization, for scenario 1/2/3/4, no need to test quit condition.

Proposal 3: For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. And the transmission power of serving cell from the start of T2 is set to meet the trigger condition of FBS based measurement.

Optimizing CSSF

For CSSF enhancement, the following test case list was agreed for solution 1 and the test case list for solution 3 is still FFS:

Issue 2-2-1: TC list for CSSF optimization - TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE.

Comparing to legacy measurements, solution 1 is to report only one carrier when multiple carriers are configured in one band which is based on UE capability. We understand the main difference is the number of carriers configured in the test and the other configurations can be reused to test new UE capability.

Also, the legacy test cases include different configurations e.g., with and without DRX, with and without SSB index reporting, etc. To reduce the number of test cases, we don’t think all the legacy configurations need to be tested for solution 1 and the test for without DRX case without SSB index reporting is enough.

Proposal 4: For solution 1 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be used as baseline with the following updates:

- The test cases are defined for without DRX and without SSB index reporting only.

- Configure [3] neighbor cells with different carriers in one band.

- It is up to NW/TE whether to indicate specific carrier to report in the test.

For solution 3, it is similar as solution 1 that only UE implementation/capabilities are updated and the legacy test configurations can be reused.

Proposal 5: Define the test cases for solution 3 of CSSF enhancement following the table below:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |

Proposal 6: For solution 3 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be reused and only the test requirements need to be updated based on new searcher assumption.

And based on the previous agreement, the CSSF enhancement are applied for the following MG related features:

- R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP

- R17 NCSG measurement with ‘nogap-noncsg’

- R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

We understand these features will not impact the implementation of solution 1. And since they have been already tested in previous release, we think we do not need to test such features with solution 1.

Proposal 7: Not to define additional test cases for MG related features with solution 1 or solution 3 of CSSF optimization.

Conclusions

In this paper, we provided our views on the performance requirements for FR2-1 SSB based L3 measurement delay reduction for optimizing BSF and CSSF. And the following proposals are provided.

Proposal 1: For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

Proposal 2: For BSF optimization, for scenario 1/2/3/4, no need to test quit condition.

Proposal 3: For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. And the transmission power of serving cell from the start of T2 is set to meet the trigger condition of FBS based measurement.

Proposal 4: For solution 1 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be used as baseline with the following updates:

- The test cases are defined for without DRX and without SSB index reporting only.

- Configure [3] neighbor cells with different carriers in one band.

- It is up to NW/TE whether to indicate specific carrier to report in the test.

Proposal 5: Define the test cases for solution 3 of CSSF enhancement following the table below:

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | Without DRX and without SSB index reporting |

Proposal 6: For solution 3 of CSSF optimization, the legacy test configurations in A.7.6.1 and A.7.6.2 can be reused and only the test requirements need to be updated based on new searcher assumption.

Proposal 7: Not to define additional test cases for MG related features with solution 1 or solution 3 of CSSF optimization.

Reference

- R4-2504899 WF on RRM requirements for NR_RRM_Ph5_Part1