# 3GPP TSG-RAN WG4 Meeting #115	R4-2505702

# Malta, MT, 19th – 23rd May, 2025

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	7.27.3.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved for OD-SSB based requirements [1].

In this contribution, we will provide discussion on RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. In this paper, we mainly focus on the discussion on OD-SSB based deactivated SCell L3 measurement (Case 1). Most proposals about the fast mode windows below can also apply for case 2.

Open issues for Case 1

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value Sub-issue 2: CSSF Agreement - When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer - Extend the FMW window time by one OD-SSB periodicity - Longer measurement delay of other frequency layers with AO-SSBs are expected Sub-issue 3: OD-SSB in FMW collided with MG Wayforward - FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion. Sub-issue 5: Searcher assumption Agreement - RAN4 shall not change searcher assumption in this work item.

As for deactivated SCell measurement, OD-SSB based measurement within FMW is preformed without gap, but whether it can be performed within or outside gap needs to be discussed. If following the principle for legacy SSB based measurement, gap-less measurements should skip the occasions of gap by extending the measurement period with the sharing factor. However, since we already agreed in last meeting that when NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer, OD-SSB carrier will be prioritized among all legacy SSB carriers. RAN4 needs to clarify whether this prioritization is applicable for SSB based measurement without gap, within gap, or both. We prefer to consider both gap-less measurement and gap-assisted measurement assuming OD-SSB carrier/layer is prioritized. In this case, the OD-SSB can be utilized efficiently without loss due to collision or skipping.

On the other hand, for the impact to gap-assisted measurement, when counting the number of MOs within the gap, two options can be provided. Option 1 is one more MO needs to be counted into the total number of MOs when calculating the CSSF within gap, and longer measurement period is needed with updated CSSF within gap. Option 2 is the measurement period for measurement within gaps is kept unchanged assuming UE can perform parallel measurements on both OD-SSB based gap-less measurement and other gap-assisted measurement. In option 2, UE needs a separate or additional measurement engine on top of current searcher assumption. As in last meeting we agreed RAN4 shall not change searcher assumption in this work item, option 2 is precluded in this release. RAN4 needs to further discuss whether option 1 can be agreed.

Proposal 1: Prefer to prioritize the OD-SSB carrier/layer within FWM regardless of measurement outside gap or within gap.

Proposal 2: RAN4 to discuss the impact on the delay requirements of measurement within gap due to collision with OD-SSB in FWM.

Proposal 3: Based on Proposal 2, one more MO needs to be counted into the total number of MOs when calculating the CSSF within gap, and longer measurement period is needed with updated CSSF within gap.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure - Background - The deactivated SCell measurement requirement in FR1 is as follow. DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - The OD-SSB based deactivated SCell measurement procedure is shown as follow. Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2. Issue 1-1-3: OD-SSB based deactivated SCell measurement – FMW start point Close the issue. Check the CR directly

In last meeting, RAN4 agreed on the introduction of fast time window and expected minimum interval (T2) between multiple FMWs. The network and UE behaviours need to further discuss.

To ensure that UE can have enough number of OD-SSB samples to achieve the measurement requirement during one shot window, RAN4 defined the fast time window length. But also UE is not required to perform “fast measurement” more frequently than the minimum interval T2.

For the case NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. However, the intention of reactivation in such a short interval could be that network expect UE can quickly complete fast measurement and report again assuming the SCell is still a detected cell to UE. Therefore, UE can perform “fast measurement” without cell search immediately after T2, assuming skipping the PSS/SSS detection and/or SSB index acquisition.

This could be a well-balanced requirement to consider both network’s indication and UE power consumption. Otherwise, the network indication of OD-SSB will be totally ignored by UE within T2, and UE could miss one change to fulfil the network’s expectation.

Proposal 4: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2, and is required to perform “fast measurement” without cell search immediately assuming the SCell is still a detected cell after T2.

For the start point and duration of T2, the start point can be the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

Proposal 5: Prefer the start point of T2 is defined as the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

For the duration of T2, it should be based on network configuration of both measCycleSCell and DRX cycles, e.g., 5*max (measCycleSCell, DRX cycles).

Proposal 6: The minimum interval(T2) between multiple FMWs is based on network configuration of both measCycleSCell and DRX cycles, e.g., 5*max (measCycleSCell, DRX cycles)

Open issues for Case 2

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreement - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue. Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures. Issue 1-2-3: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement. Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies) Agreement RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015. - Option 1: RAN4 to define requirement for different frequencies - Option 2: RAN4 not to define requirements for different frequencies

RAN4 agreed to specify scenario 1 and 4. For other scenarios with different periodicities or different frequencies, more complicated UE implementation and requirements need to be investigated and discussed. We suggest RAN4 to down-prioritize the other scenarios including scenario 2 and 3 here.

For scenario 2 with non-periodic SSB (i.e., RAN1 Alt Time-C2), since UE will follow one DL timing for measuring SSBs on the same frequency, non-periodic SSBs will bring extra efforts for UE to detect and measure. From UE side, we prefer not to define any enhanced requirements for this scenario under NES operation.

Proposal 7: For scenario 2 with non-periodic SSB (i.e., RAN1 Alt Time-C2), RAN4 not to define any new requirements.

For scenario 3 (OD-SSB and always-on SSB are within different frequencies), since RAN2 agreed on a new MO for OD-SSB frequency layer which is different from AO-SSB frequency layer, network should enable one MO with reporting from OD-SSB MO and AO-SSB MO for the SCC, but not in parallel. Because usually one servingcellMO is configured or selected for one carrier. Even for Redcap UE, NCD-SSB and CD-SSB in servingcellMOs are not measured simultaneously. Otherwise, it is up to UE implementation for this scenario.

Proposal 8: For scenario 3 (OD-SSB and always-on SSB are within different frequencies), RAN4 not to define requirements for different frequencies.

Other open issues

Issue 2-1-1: OD-SSB based multiple SCells activation Wayforward - Option 1: - RAN4 to define OD-SSB based multiple SCells activation delay requirements. - The limitation is needed - It doesn’t impact the WI’s completion. - Option 2: - Not define multiple OD-SSB carriers requirement in this release.

RAN4 has agreed to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. We think if network wants to enable OD-SSB on different Carriers, TDM manner can be used. In R19, due to limited TUs and unexpected big changes on UE architectures including both RF and baseband, it is more realistic that RAN4 only defines single OD-SSB carrier under CA operation.

Proposal 9: In R19, it is more realistic that RAN4 only defines requirements for single OD-SSB carrier under CA operation, but won’t define multiple OD-SSB carriers requirement in this release.

Conclusions

In this paper, we discuss the open issues for OD-SSB based deactivated SCell L3 measurement in Case 1 and Case 2.

Proposal 1: Prefer to prioritize the OD-SSB carrier/layer within FWM regardless of measurement outside gap or within gap.

Proposal 2: RAN4 to discuss the impact on the delay requirements of measurement within gap due to collision with OD-SSB in FWM.

Proposal 3: Based on Proposal 2, one more MO needs to be counted into the total number of MOs when calculating the CSSF within gap, and longer measurement period is needed with updated CSSF within gap.

Proposal 4: When NW re-activates the OD-SSB within the T2, UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2, and is required to perform “fast measurement” without cell search immediately assuming the SCell is still a detected cell after T2.

Proposal 5: Prefer the start point of T2 is defined as the time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

Proposal 6: The minimum interval(T2) between multiple FMWs is based on network configuration of both measCycleSCell and DRX cycles, e.g., 5*max (measCycleSCell, DRX cycles)

Proposal 7: For scenario 2 with non-periodic SSB (i.e., RAN1 Alt Time-C2), RAN4 not to define any new requirements.

Proposal 8: For scenario 3 (OD-SSB and always-on SSB are within different frequencies), RAN4 not to define requirements for different frequencies.

Proposal 9: In R19, it is more realistic that RAN4 only defines requirements for single OD-SSB carrier under CA operation, but won’t define multiple OD-SSB carriers requirement in this release.

Reference

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1