# 3GPP TSG-RAN WG4 Meeting #115	R4-2505703

# Malta, MT, 19th – 23rd May, 2025

Title: 	On RRM requirements of SSB adaptation

Source: 	OPPO

Agenda item:	7.27.3.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved for SSB adaptation [1]. In this contribution, we will provide discussion on RRM requirements for SSB adaptation.

Discussion

### Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. ### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs. ### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations. ### Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

RAN4 agreed to define requirements based legacy L3 measurement framework for SSB adaptation for both deactivated Scell and activated Scell. For neighbour cell L3 measurement, legacy requirements apply. RAN4 needs to further discuss whether to introduce requirements related to additional SMTC.

  - According to RAN2 agreements as below, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

  - After SSB adaptation, UE follows the SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

  - For deactivated Scell, the requirements of measurement period are still based on meascycleScell, and the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation.

  - For activated Scell, UE just follows the SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

- RAN2 #129 agreement - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. - RAN2 #129b agreement - Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. - For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. - Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

  - Proposal 1: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation.

  - Proposal 2: For activated Scell, UE just follows the SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

  - For L3 requirement at transition, we think the principle could be different as that for L1 measurement, because multiple L1 RS resources will be configured for RLM or BFD, but the dedicated L3 RS resources is used for L3 RRM measurements. Anyway, L1 measurement results or report, for serving cell or beam quality, are allowed to combine from the measurements based on different RS resources. But for L3 measurements for mobility, the measurement report could be not reliable if UE cannot support mixing the measurement results before and after the transition for L3 measurement. Thus, we prefer to define the mimimum requirement for L3, and UE needs to restart the L3 measurement based on the SMTC of adapted SSB periodicity after SSB adaption command.

  - Proposal 3: At transition, UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation.

About the applicable SCell activation requirements, RAN4 agreed to define requirements for SSB adaption, including normal SCell activation, direct SCell activation and PUCCH SCell activation.

For FFS part, we prefer not to include Rel-18 SCell activation with L3 reporting because the scenario in R18 is for unknown deactivated SCell, and the SCell activation requirements are already supported based on the SSB periodicity, while DCI triggered SSB adaptation could not bring too much gain on top of that. And for multiple SCells activation, we are open to discuss.

Proposal 4: RAN4 not to include the case of R18 Scell activation with L3 reporting in this WI.

Conclusions

In this paper we provided our views on the SMTC impact on RRM requirements due to SSB adaptation and/or OD-SSB.

  - Proposal 1: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation.

  - Proposal 2: For activated Scell, UE just follows the SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

  - Proposal 3: At transition, UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation.

Proposal 4: RAN4 not to include the case of R18 Scell activation with L3 reporting in this WI.

Reference

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2