3GPP TSG-RAN WG4 Meeting #115				                                R4-2505775

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.10.4.5

Title: 	Discussion on the performance part requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In the previous RAN4 meeting, the RRM core requirements [1] related to the less than 5MHz NTN were almost completed [2]. Accordingly, RAN4 is required to define the corresponding test cases to verify the RRM requirements in TS38.133 [3].

In the last RAN4 meeting, the agreements on the common testing configuration and part of test cases needed were agreed. In this paper we provide further consideration on the open issues below.

- Test cases for cell Re-selection for NR UE for Satellite Access

- The detailed test configuration

- Discussion

- Issue 1: Methodology to introduce the test cases for NTN less than 5MHz

According to RAN4’s discussion on the core part, for NTN less than 5MHz, only part of new requirements were introduced in Rel19. For examples, for the requirements in IDLE/inactive mode, the core requirements for NTN in Rel18 can be reused.

Table 1. RRM impacts summary due to spectrum less than 5MHz

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Need the new requirements in Rel19 |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | None |
| RRC connection mobility control | 6.2C RRC Connection Mobility Control for Satellite Access | None |
| Handover | 6.1C Handover for SAN | Yes |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | None |
| RLM | 8.1C Radio Link Monitoring for Satellite Access 8.5C Link Recovery Procedures for Satellite Access | Yes |
| Measurement | 9.2C NR intra-frequency measurements for SAN 9.3C NR inter-frequency measurements for SAN | Yes |

However, for the test cases themselves, in our views in Rel18 test case for NTN [3, TS38.133 v18.6.0] the test configuration for 10MHz bandwidth was introduced only.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v18.7.0] the test configuration for less than 5MHz was not included.

Therefore, in order to verify the performance of NTN less than 5MHz in which the legacy requirements were leveraged (e.g. the requirements for IDLE/Inactive mode, timing) the additional test configuration for NTN less than 5MHz needed be introduced. For UE support NTN less than 5MHz only, this test configuration shall be applicable. Otherwise, the test configuration with 10MHz BW needed to be pass.

Proposal 1: RAN4 can define the additional testing configuration with less than 5MHz for NTN RRC IDLE and Inactive mode test cases. And UE supporting ONLY NTN less than 5MHz shall pass such test.

- Issue 2: Detailed test case list and configuration for NTN less than 5MHz

In the last meeting, the following test cases for NTN less than 5MHz core requirement were agreed.

Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

However, for test case “MR-2”, the core requirements with SSB index reading could be impacted by less than 5MHz. That is the test case for inter-frequency without SSB index reading is completely redundant. Thus, we suggest that:

Proposal 2: The test for NTN less than 5MHz inter-frequency measurement with SSB index reading is needed.

Hereby, we would provide further consideration on the detailed configuration for each test cases in the table below.

| Test case | Requirements | Configurations |
| --- | --- | --- |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | Intra-frequency , unknown target cell, 15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB |
| CHO-1 | 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | Intra-frequency , unknown target cell,15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB, without L3 measurements |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Intra-frequency , unknown target cell ,15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| RLM-2 |  | RLM IS, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | intra-frequency, non-DRX, no-gap, 12PRB SSBs, 3MHz BW, with SSB index detection |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | inter-frequency, non-DRX, no-gap, 12PRB SSBs, 3MHz BW, with SSB index detection |

Proposal 3: The new test cases configuration for NTN less than 5MHz can be summarized as Table 1 above.

- Summary

This paper proposes the main configurations parameters and test cases list to verify the RRM requirements related to NTN less than 5MHz.

Observation 1: in Rel18 test case for NTN [3, TS38.133 v18.7.0] the test configuration for less than 5MHz was not included.

Proposal 1: RAN4 can define the additional testing configuration with less than 5MHz for NTN RRC IDLE and Inactive mode test cases. And UE supporting ONLY NTN less than 5MHz shall pass such test.

Proposal 2: The test for NTN less than 5MHz inter-frequency measurement with SSB index reading is needed.

Proposal 3: The new test cases configuration for NTN less than 5MHz can be summarized as below.

| Test case | Requirements | Configurations |
| --- | --- | --- |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | Intra-frequency , unknown target cell, 15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB |
| CHO-1 | 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | Intra-frequency , unknown target cell,15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB, without L3 measurements |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Intra-frequency , unknown target cell ,15KHz, NGSO, RACH-based handover, 3MHz BW, 12 PRB SSB |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| RLM-2 |  | RLM IS, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD, FR1 PCell only, non-DRX, 3MHz BW, 15RB PDCCH and 12 PRB SSB |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | intra-frequency, non-DRX, no-gap, 12PRB SSBs, 3MHz BW, with SSB index detection |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | inter-frequency, non-DRX, no-gap, 12PRB SSBs, 3MHz BW, with SSB index detection |

- References

- RP-240857

- R4-2502589

- 3GPP TS 38.133 v18.7.0