3GPP TSG-RAN WG4 Meeting #115				                                            R4-2505777

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.27.3.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and the objectives for OD-SSB are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [2]. Therefore, in this discussion, we provided more analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - General On-demand SSB(OD-SSB) requirements

In the previous RAN4 meetings, whether the OD-SSB based cell identification before deactivated SCell measurement can be skipped were discussed but without any agreements.

R4#114bis discussion Moderator suggests the group to check the following options(except the Case 1 Option 2). - The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios: - Case 1: No always-on SSB on the cell - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band - Option 2-1: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter. - Other detail condition is FFS. (Note: this option has already covered in issue 1-2-3) - Option 2-2: UE has sent a valid measurement report based on OD-SSB for the SCell being activated [5]s before, and then the OD-SSB is transmitted again with the same spatial reception parameter same transmission power, and on the same frequency carrier. - Other detail condition is FFS. - Option 3: Extend referenceCell to OD-SSB SCell - Other option is not precluded - Case 2: Always-on SSB on the cell - Option 1: Cell identification can be always skipped - Option 1a: Cell identification can be always skipped in scenario 2 - Option 2: follow SCell activation known condition - Other option is not precluded

Firstly, for case1, given RAN1 agreements below, we can conclude that：

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

R1#118 discussion Agreement: For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

On the other hand, OD-SSB based measurements for the deactivated SCell can be feasible with the valid timing and cell information. That is the pre-required information can be obtained from the serving cells associated with the SCells with OD-SSB if they can meet the specific conditions (e.g. adjacent together in both frequency domain and spatial domain).

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Also the specific known condition for the cell identification can be feasible. But if such known condition can’t be satisfied, how UE can obtain the necessary timing information shall be considered as well. In our view, in case of unknown condition, UE can also rely on the timing of associated serving cell.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Therefore, in order to simplify our specification, we prefer to define such condition for case 1 by Option 1 directly.

Secondly, for case2, since there accompanied always-on SSB is available, it is quite simple for UE implementation to detect/identify SCell to be measured with the always-on SSB indeed.

Thus, we can propose that:

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

- Case 2: Always-on SSB on the cell

  - Case 1- OD-SSB based deactivated SCell L3 measurement

When the OD-SSB was triggered for SCell activation, the periodicity of OD-SSB is expected to be short as possible to enable UE can complete the measurement quickly which can save the network power. That is the exact meaning of “fast L3 measurement” in RAN4 agreements [4]. However, for the deactivated SCell, the periodicity of OD-SSB can be much sparser than these for the SCell activated or to be activated. As a result, UE need not to measure the OD-SSB so frequently to minimize UE’s power consumption. This is what “slow measurement” mean in comparison with the former one.

Thus, in the previous RAN4 meeting, the harmonized solution to balance the power consumption and the reliable measurement results via OD-SSB occasions is to restrict the periodicity of measurement based OD-SSB in the fast mode window (“Tfast”) and after that (e.g.“Tslow”) UE can relax the measurement with “measCycleSCell” or stop the measurement as agreements below.

Figure 1. OD-SSB measurement stages [4]

And in the last RAN4 meeting, the agreements on how to define the time window (Tfast) and corresponding UE measurement behavior was achieved in [4].

- Sub-issue 2: OD-SSB based deactivated SCell measurement – Procedure

Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW. What’s the UE’s behaviour when OD-SSB further activation/reconfiguration after T2? - WF/Agreements RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement.

In our view, the minimum T2(also denoted as Tsleep below) is up to the different purpose or UE behavior after Tfast and Tslow. For an example, as discussed in the last RAN4 meeting, there are several ways to define UE behaviors.

- When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2.

UE with Option 1’s behavior is the typical usage after the fast measurement window. In other words, UE behavior within T2 is same as that within Tslow. It seems unnecessary to define additional behavior for UE within T2.

For Option2’s behavior, it is similar as known/unknown conditions for intra/inter-frequency measurements in RAN4. That is if UE performing the deactivated SCell measurement within a specific duration, the synchronization timing can be assumed valid. UE needs only performing RRM measurements but not the cell detection.

Observation 4: How to define UE’s behavior within “T2” is highly dependent with the other sequential stages before it.

Moreover, as discussed in the last meeting, there are too diverse understandings on the definition of T2. For an example, if the start point of T2 can’t be aligned, the exact requirement on T2 and corresponding UE behavior is unclear.

Figure 2.

Observation 5: There are too diverse understandings on the definition of T2 in RAN4 which results in the ambiguous UE behavior during T2 and the stages after T2.

Therefore, according to the previous RAN4 discussion and agreements above, we can clarify the UE behavior during the different stages under OD-SSB measurement:

- Within Tfast, UE needs perform the SCell measurement with OD-SSB per the OD-SSB periodicity

- Within Tslow, UE can perform the SCell measurement with OD-SSB per the legacy deactivated SCell measurement requirements (e.g. based on “measSCellCycle”)

- Within T2(Tsleep) from the deactivation OD-SSB(t3) to the next OD-SSB activation(t4), UE can stop any measurements

- Within Tre-act, the UE behavior can be upon the duration of Tsleep, which can be FFS.

Proposal 2: The overall procedure for deactivated SCell with OD-SSB shall be clarified especially after the initial OD-SSBs were deactivated.

Since after OD-SSB deactivation, there is no any OD-SSB available during T2, UE can stop its measurement on SCell in case 1 from that point. In our views, the it is better to define the start point of T2 window as the time point of OD-SSB deactivation (Option 4). It is more clear to distinguish the different stage of UE behaviors.

Proposal 3: The start point of T2 can be the time point of OD-SSB deactivation.

Proposal 4: UE can stop its measurement during T2. And from RAN4 perspective, there is no any requirements for UE within T2.

But after T2, when the OD-SSB was re-triggered, in order to further reduce UE power consumption, UE can follow the sparser measurements in comparison with the measurement in Tfast if the previous measurement result is still valid. This is similar with the known condition for the SCell activation in TS38.133 [3]. Otherwise, UE can follow the same UE behavior as that within Tfast.

Proposal 5: When the OD-SSB was re-activated, within Tre-activate (after T2) UE can follow the sparer measurements instead of OD-SSB cycle under the specific conditions.

- FFS on the conditions by which UE can relax its measurement with OD-SSB after T2

- Sub-issue 4: OD-SSB in FMW collided with MG

In the last RAN4 meeting the issue for OD-SSB colliding with MG was discussed.

Figure 3.

Since the handling on the MG colliding with other signal is extremely complicated issue before, we also prefer to reuse the legacy rules. For an instance, in the legacy measurement requirements, we supposed that the intra-frequency measurement will be dropped. Thus, if we following the same principle the measurements on the deactivated SCell based on OD-SSB shall be discarded.

Proposal 6: If OD-SSB within the FMW overlapped with the measurement gap occasion in time domain, UE can deprioritize OD-SSB measurement.

But on the other hand, in the previous RAN4 discussion on the length of FMW, such impacts due to the collision with MG was missed.

Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

Observation 6: There is impacts on the effective measurement window length of FMW because of the collision with the MG.

Therefore, we suggest that:

Proposal 7: RAN4 can clarify the potential impacts on the length of FMW agreed in RR4#114 due to OD-SSB collision with MG.

- Sub-issue 4: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

Wayforward - Option 1: - RAN4 to define OD-SSB based multiple SCells activation delay requirements. - The limitation is needed - It doesn’t impact the WI’s completion. - Option 2: - Not define multiple OD-SSB carriers requirement in this release.

In the previous meeting, whether the requirements for the multiple active OD-SSB carriers scenarios shall be defined was discussed. In our views, there are lots of open issues for this scenario. For an example, like the discussion of fast measurement window for the signal OD-SSB carrier, RAN4 needs plenty of time to reach the technical consensus. Regarding to limitation of TUs in this Rel19 WI, we can only consider single  OD-SSB carriers in Rel19 WI.

Proposal 8: Prefer to deprioritize the cases with multiple active OD-SSB carriers

  - Interruption

In the last meeting the following CR split works related to interruption requirements in TS38.133 was agreed.

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |

However, the SCell addition/release will be rely on the legacy RRCConfiguration. Thus, for the interruption at SCell addition/release UE behaviour will be little impacted because of OD-SSB itself.

Observation 7: the interruption at SCell addition/release with OD-SSB can be same as the legacy requirements in TS38.133 for SCell with always-on SSB.

On the other hand, for the interruption during measurement on deactivated SCC, when the deactivated SCC measurement based on OD-SSB, the potential impacts on current requirement in TS38.133 shall be investigated as mentioned in [5][6]. For an example, the “measCycleSCell” parameter used to define the legacy deactivated SCell measurement [3, TS38.133] could not available/used when SCell is activated after the fast mode window.

However, according TS38.331, measCycleSCell can be used for the SCell in the deactivated state only.

Therefore, we suggested that:

Proposal 9: The change on the clause 8.2.2.2.3 of TS 38.133 is needed. The CRs works for interruption requirements in TS38.133 can be updated as:

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |
| 8.2.2.2.3 | Interruptions during measurement on deactivated SCC |

Accordingly, we can also directly discuss the draftCR provide in [7].

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

General On-demand SSB(OD-SSB) requirements

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

- Case 2: Always-on SSB on the cell

Case 1- OD-SSB based deactivated SCell L3 measurement

Observation 4: How to define UE’s behavior within “T2” is highly dependent with the other sequential stages before it.

Observation 5: There are too diverse understandings on the definition of T2 in RAN4 which results in the ambiguous UE behavior during T2 and the stages after T2.

Proposal 2: The overall procedure for deactivated SCell with OD-SSB shall be clarified especially after the initial OD-SSBs were deactivated.

Proposal 3: The start point of T2 can be the time point of OD-SSB deactivation.

Proposal 4: UE can stop its measurement during T2. And from RAN4 perspective, there is no any requirements for UE within T2.

Proposal 5: When the OD-SSB was re-activated, within Tre-activate (after T2) UE can follow the sparer measurements instead of OD-SSB cycle under the specific conditions.

- FFS on the conditions by which UE can relax its measurement with OD-SSB after T2

Proposal 6: If OD-SSB within the FMW overlapped with the measurement gap occasion in time domain, UE can deprioritize OD-SSB measurement

Observation 6: There is impacts on the effective measurement window length of FMW because of the collision with the MG.

Proposal 7: RAN4 can clarify the potential impacts on the length of FMW agreed in RR4#114 due to OD-SSB collision with MG.

Proposal 8: Prefer to deprioritize the cases with multiple active OD-SSB carriers

Interruption

Observation 7: the interruption at SCell addition/release with OD-SSB can be same as the legacy requirements in TS38.133 for SCell with always-on SSB.

Proposal 9: The change on the clause 8.2.2.2.3 of TS 38.133 is needed. The CRs works for interruption requirements in TS38.133 can be updated as:

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |
| 8.2.2.2.3 | Interruptions during measurement on deactivated SCC |

- References

- RP-240830

- R4-2420098 WF on RRM requirements for Netw_Energy_NR_enh_Part1

- TS38.133 v18.6.0

- R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1

- R4-2418282	Topic summary for [113][224] Netw_Energy_NR_enh_Part1

- R4-2503124

- R4-2505780