3GPP TSG-RAN WG4 Meeting #115           	R4-2505971

Malta, MT, 19th-23th, May, 2025

Agenda item:	7.12.7

Source:	Samsung

Title:	View on BS demodulation requirement for ATG

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the New WI was endorsed to enhance the ATG network for NR with the following objectives in WID [1]

- Objective of Core part - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4] - FR1 intra-band contiguous CA - Example band combination: n79 - FR1+FR1 inter-band CA - Example band combination: n3+n39 - Specify the RF requirements for support of UL MIMO with 2Tx for single cc for UE. [RAN4] - Specify signaling and UE capabilities if necessary [RAN2] - Objective of Performance part - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter band co-located DL CA - Specify corresponding demodulation requirements for support of UL MIMO with 2Tx for single CC for UE with dual-polarization antenna

In the last meeting, the test scope of performance part of ATG for Rel-19 was under discussion. The related agreement was captured into WF in [2] as following

- UL MIMO - Whether to reuse existing requirements for ATG UL MIMO - The legacy PUSCH test cases in Clause 8.2.1with 2Tx 2 layer will be applied for ATG - FFS - Whether other legacy test cases are needed. - Whether to introduce new test cases for ATG UL MIMO - Option 1: incremental PUSCH 2layer test cases need to be introduced - Option 2: Do not introduce new 2Tx PUSCH requirements for ATG scenario - Antenna configuration for incremental test - Option 1 - 2T2R, 2T4R and 2T8R with X-pol low correlation - Option 2 - 2T2R, 2T4R and 2T8R - Option 3 - 2T2R - Propagation condition for incremental test - Option 1: For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference - Bandwidth & SCS for incremental test - Option 1: 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS - Rank for incremental test - Option 1: Rank 2 - MCS for incremental test - Option 1: 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation - Option 2: consider low modulation order for high rank transmission - Option 3: MCS16 - DMRS port for incremental test - DMRS port {0,1} - TPMI for incremental test - TPMI index 0 - Others - Option 1 - Type A - HARQ retransmission - 70% of maximum throughput - Option 2 - Others same as existing Rel-15 BS PUSCH demodulation requirement with 2TX

In this contribution, the view on remaining issue of test scope of BS demodulation requirement for ATG is provided.

# 2	Test Scope

Whether to reuse existing requirements for ATG UL MIMO

In Rel-18, RAN4 has finalized the air to ground (ATG) network WI for FR1 single carrier scenario, where only 1Tx for UE was considered. For Rel-19, to further enhance the throughout for both DL and UL, CA and UL MIMO feature are considered. For UL MIMO, in Rel-15 NR BS demodulation, RAN4 has also defined demodulation requirements with 2Tx for PUSCH. In the last meeting, RAN4 agreed that legacy PUSCH test cases in Clause 8.2.1with 2Tx 2 layer will be applied for ATG, and whether other legacy cases are needed will be further studied. In current legacy cases in 8.2, excepting for 8.2.1, other cases mainly target for 1 layer transmission. Therefore, other legacy cases in Clause 8.2 are not applied for Rel-19 ATG scenario.

Proposal 1: Only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG.

Whether to introduce new test cases for ATG UL MIMO

For ATG scenario, the CPE will be deployed at the airplane, where large frequency and timing shift will be experienced at baseband. In Rel-18 discussion, the baseline assumption is that UE can do pre-compensation on UL frequency shift and UL timing shift for ATG demodulation.  Therefore, based on UE pre-compensation assumption, the residual Doppler shift will be small with 0.1ppm, up to 500Hz residual carrier frequency offset will be experienced at the gNB.

For Rel-18 ATG WI, compare to existing TN BS, additional cases were introduced, where residual frequency offset was considered with single path AWGN channel. With the same UE pre-compensation assumption, the baseband processing for ATG scenario should be same as TN BS. Therefore, form baseband processing to support 2Tx perspective, existing requirement can fulfill the test purpose of UL MIMO with 2-layer transmission. From baseband processing to correct frequency offset perspective, it has been verified in Rel-18. We think existing requirements can fulfill the test purpose of frequency offset tracking.

Observation 1: From baseband processing to correct frequency offset perspective, it has been verified in Rel-18 ATG WI

Observation 2: From baseband processing to support UL MIMO, existing requirements can fulfill the test purpose of UL MIMO with 2-layer transmission

Therefore, we think there is no need to introduce new test cases for ATG UL MIMO. While we are open to further discuss.  If necessary, to consider the test coverage of ATG deployment, only limited cases, one case with UL MIMO for each SCS can be introduced. And If BS declared to support ATG scenario, it can pass the existing TN BS requirement and the dedicated requirement, it can apply for ATG scenario

Proposed 1: There is no need to introduce new test cases for ATG UL MIMO. If necessary, considering  the test coverage of ATG deployment, only limited cases, one case with UL MIMO transmission for each SCS can be introduced.

Since the main target for UL MIMO is to improve the UL throughput, PUCCH and PRACH is not related with 2 Tx. Therefore, there is no PUCCH and PRACH requirement for UL MIMO.

Proposal 2:  No PUCCH and PRACH requirement for UL MIMO with 2Tx.

UL MIMO with 2Tx

For ATG UE UL-MIMO, based on RAN4 RF core requirement as following

Agreement in RAN4#110bis - Clarification of UL-MIMO - Agreement - Introduce UL MIMO requirement with 2Tx - Max layer number of UL-MIMO - Agreement - 2 layers Agreement in RAN4#114 - FFS to support ULFPx (UL full power transmission) mode for UL MIMO - FFS to preclude the single antenna port related requirement

For other models, including ULFPx mode and single antenna port transmission TxD mode, whether they can be supported for ATG UE with UL MIMO are still not decided yet based on RF core requirement discussion. Considering ULFPx and single antenna port transmission TxD (Transmit diversity) are UE optional feature with capability signaling, meanwhile, for existing TN BS, there is no related demodulation requirement for 2Tx with UE supporting ULFPx mode and single antenna port transmission TxD mode. Therefore, our preference is not to define demodulation requirement for UL MIMO with 2Tx for UE supporting ULFPx mode and single antenna port transmission TxD mode.

Proposal 3:  Not consider the demodulation requirement for UL MIMO with 2Tx for UE supporting ULFPx mode and single antenna port transmission TxD mode. If UL MIMO with 2Tx requirement introduced, only consider demodulation requirement for UL MIMO with 2Tx and 2 layers with multiplexing.

# 3	Test Setup

In this section, the initial test setup for BS demodulation for ATG is discussed, if the demodulation requirement with UL MIMO with 2Tx will be introduced.

Channel Model

Regarding the channel model, the following was agreed in Rel-18 ATG, considering AWGN channel as following

Agreement in RAN4#107bis - Channel Model - At least single path AWGN channel with Doppler

For UL MIMO with 2Tx, we think the same assumption can be applied. Since the AWGN channel is almost as LOS channel condition. In order to support UL MIMO with 2 layers transmission, we think the channel condition to support Rank 2 should be guaranteed.

For legacy requirement, different antenna configurations with 2Rx/4Rx/8Rx were considered. For the dedicated requirement with UL MIMO for ATG scenario, to reduce test effort, our preference is to only consider 2Rx for requirement if UL MIMO requirement will be introduced.

The following channel matrix can be considered for 2Tx with 2Rx as starting point.

For 2Rx

.

Proposal 4:  If the demodulation requirement with UL MIMO with 2Tx will be introduced. Considering the following channel matrix to specify the PUSCH requirement with 2 Tx with 2Rx as starting point

For 2Rx

.

Doppler Shift assumption

Based on Rel-18 ATG RF discussion, the following frequency error requirements with below values are considered

- For FDD channel model, set Doppler as [220Hz]

- For TDD channel model, set Doppler as [400/500Hz]

And 200Hz and 500Hz are considered for BS demodulation requirement definition for FDD and TDD, separately.  Same assumption can be considered for evaluating the PUSCH with 2Tx.

Proposal 5:  Apply the same Doppler to  specify the PUSCH with UL MIMO with 2Tx

- For FDD, set Doppler as 200Hz for UL

- For TDD, set Doppler as 500Hz for UL

BW

Regarding the BW, as our preference, limited cases can be considered. Similar as Rel-18, the minimum CBW for each SCS can be considered for evaluation.

Mapping Type

Regarding the mapping type, mapping type A can be considered

DM-RS configuration

For DM-RS, we think Rel-15 DM-RS type should be considered

MCS

For MCS, in Rel-18 ATG, we prefer to reuse the MCS in existing TN BS demodulation requirement as starting point, where MCS 16 was used.

For other test parameters, the existing Rel-15 BS PUSCH demodulation requirement with 2Tx can be reused

Proposal 6:  If the demodulation requirement with UL MIMO with 2Tx will be introduced, consider the following assumption for evaluation the PUSCH performance with UL MIMO with 2Tx

- BW

  - 15KHz SCS, 5MHz

  - 30KHz SCS, 10MHz

- Mapping Type

  - Mapping type A

- DM-RS configuration

  - Rel-15 DMRS type

- MCS

  - MCS 16

- Others same as existing Rel-15 BS PUSCH demodulation requirement with 2Tx

# 4	Conclusion

In this contribution, the view on the BS demodulation requirement with UL MIMO with 2Tx for ATG was provided

Proposal 1: Only the applicable PUSCH test cases in Clause 8.2.1 with 2Tx 2 layer will be applied for ATG.

Observation 1: From baseband processing to correct frequency offset perspective, it has been verified in Rel-18 ATG WI

Observation 2: From baseband processing to support UL MIMO, existing requirements can fulfill the test purpose of UL MIMO with 2-layer transmission

Proposed 1: There is no need to introduce new test cases for ATG UL MIMO. If necessary, considering  the test coverage of ATG deployment, only limited cases, one case with UL MIMO transmission for each SCS can be introduced.

Proposal 2:  No PUCCH and PRACH requirement for UL MIMO with 2Tx.

Proposal 3:  Not consider the demodulation requirement for UL MIMO with 2Tx for UE supporting ULFPx mode and single antenna port transmission TxD mode. If UL MIMO with 2Tx requirement introduced, only consider demodulation requirement for UL MIMO with 2Tx and 2 layers with multiplexing.

Proposal 4:  If the demodulation requirement with UL MIMO with 2Tx will be introduced. Considering the following channel matrix to specify the PUSCH requirement with 2 Tx with 2Rx as starting point

Proposal 5:  Apply the same Doppler to  specify the PUSCH with UL MIMO with 2Tx

- For FDD, set Doppler as 200Hz for UL

- For TDD, set Doppler as 500Hz for UL

Proposal 6:  If the demodulation requirement with UL MIMO with 2Tx will be introduced, consider the following assumption for evaluation the PUSCH performance with UL MIMO with 2Tx

- BW

  - 15KHz SCS, 5MHz

  - 30KHz SCS, 10MHz

- Mapping Type

  - Mapping type A

- DM-RS configuration

  - Rel-15 DMRS type

- MCS

  - MCS 16

- Others same as existing Rel-15 BS PUSCH demodulation requirement with 2Tx

# References

RP-240839, New WID on Enhancements for Air-to-ground network for NR in Rel-19, CMCC

R4-2504760, Way Forward for [114bis][332] NR_ATG_enh_demod, CMCC