3GPP TSG-RAN WG4 Meeting # 115                                                              R4-2506013

Malta, MT, 19th – 23rd May, 2025

Title: 	                 Discussion on RF requirements for CW for D1T1

Source: 	CATT

Agenda item:	7.26.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1].

- RF Requirement for CW for D1T1

2.1 General

In WID [2], RF requirements for Type 1-C Ambient-IoT BS need to be specified. Scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. For scenario D1T1-B, the CW is transmitted from outside the topology.

2.2 CW requirements

2.3.1 ON/OFF power

In the D1T1-B scenario, CW is transmitted from outside the topology, CW nodes only transmit signals and don’t receive signals, and the CW/D2R spectrum and R2D spectrum are different, so the ON/OFF power requirement is not necessary. If define CW off power, the value can be -65dBm/15kHz.

Proposal 1:  The CW off power can be -65dBm/15kHz.

2.3.2 Phase noise

- Issue 3-4: Phase noise

- Agreement:

  - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI.

  - Encourage companies provide phase noise profile

The phase noise of the CW signal is related to the phase noise of the reference clock signal. The reference clock can be generated by the CW node itself or input from A-IoT BS. A-IoT BS and CW nodes are usually implemented by the same vendor, so phase noise requirements can be left to implementation in Rel-19 WI.

Proposal 2: The phase noise of CW signal can be left to implementation in Rel-19 WI.

2.3.3 Unwanted emission

- Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

Channel bandwidth for CW signal needs to be defined, and both the definition of the requirement and the CW interference cancellation capability requirement are related to it. The channel bandwidth for CW can be defined as 15kHz.

Proposal 3: Channel bandwidth for CW signal needs to be defined.

Proposal 4: The channel bandwidth for CW signal can be defined as 15kHz.

Table 1: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

The spectrum emission mask for CW node can be defined as table 1. The Δf is defined from the center of the CW transmission. The frequency range of Δf from  0 to 90kHz can be covered by phase noise and can be left to implementation.

Proposal 5: The frequency range of Δf from 0 to 90kHz can be covered by phase noise and left to implementation.

Proposal 6: The spectrum emission mask for CW node can be defined as table 1.

2.3.4 Spurious emissions

The maximum offset of the spectrum emission mask from the operating band edge is Δf. The spectrum emission mask define all unwanted emissions in each supported downlink operating band plus the frequency ranges Δf above and Δf below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement. CW node can reuse NR requirement, and the Δf is 10MHz.

Proposal 7: The boundary between spectrum emission mask domain and spurious emission domain for CW node can be 10 MHz.

2.3.5 Intermodulation

If A-IoT BS and CW node are co-located, and the CW node works with other devices and other A-IoT BS, if the A-IoT BS receiver and the co-located CW node operate simultaneously, assume the coupling loss between them is 30dB, then the CW interference signal level at the A-IoT BS input will be large and it will have a great impact on the Rx blocking requirement of A-IoT BS, even will block the A-IoT BS receiver, so this scenario is not recommended.

Proposal 8: Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Since continuous waves with multiple tones are allowed in RAN1, reverse IM products from the same location/nearby continuous wave transmitter should be suppressed. However, in WID [2], it is specified that carrier wave is only waveform 1 and without hopping, so it is a single tone, and the intermodulation requirement is not needed.

Proposal 9: The intermodulation requirement is not needed.

- Summary

This contribution provides our analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1], and our proposals can be summarized in the following:

Proposal 1:  The CW off power can be -65dBm/15kHz.

Proposal 2: The phase noise of CW signal can be left to implementation in Rel-19 WI.

Proposal 3: Channel bandwidth for CW signal needs to be defined.

Proposal 4: The channel bandwidth for CW signal can be defined as 15kHz.

Proposal 5: The frequency range of Δf from 0 to 90kHz can be covered by phase noise and left to implementation.

Proposal 6: The spectrum emission mask for CW node can be defined as table 1.

Proposal 7: The boundary between spectrum emission mask domain and spurious emission domain for CW node can be 10 MHz.

Proposal 8: Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Proposal 9: The intermodulation requirement is not needed.

Reference

[1] R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#114bis