3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506040

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on system parameter for NTN support spectrum less than 5 MHz

Agenda item:	7.10.4.1

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, a WF [1] on NR NTN support spectrum less than 5 MHz system parameter was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on system parameter for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on system parameter for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

Issue 1-3: The clarification on the design of SS/PBCH, RACH preamble formats and CORESET#0 to support 3MHz Channel bandwidth for NTN operation - Option 1: (Ericsson) - 3MHz CBW for NTN shall follow the same design as for TN for a general case of 3 MHz CBW: - 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH. - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. - 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0. - Option 2: (CATT) - For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported. - Recommended WF - TBD

Since 3 MHz channel bandwidth will be introduced, we believe that the behaviour of UE to support 3 MHz channel bandwidth can be introduced accordingly. Considering that a normal SSB of 5 MHz contains 20 RBs and its occupied bandwidth is 3.6 MHz (15 KHz SCS), it means that its SSB bandwidth will be larger than the 3 MHz channel bandwidth, and thus a UE operating on 3 MHz channel bandwidth will not be able to decode the SSB correctly.

For TN networks supporting 3 MHz channel bandwidth, RAN1 has designed an RB-level puncturing method that reduce the number of RBs in the PBCH from 20 RBs to 12 RBs, thereby reducing the bandwidth occupied by the SSB to 2.16 MHz. This approach allows the SSBs to be correctly placed in 3 MHz channel bandwidths, and therefore we believe this approach could be reused for NTN networks supporting 3 MHz channel bandwidth.

Proposal 1: For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported.

- Conclusion

This contribution provides our initial views on system parameters for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported.

- References

- R4-2504713, WF for [114-bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #114-bis