3GPP TSG RAN WG4 Meeting #115		R4-2506059

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, discussion on power domain enhancements for single carrier continued, and a WF was approved to capture the agreements and identified open issues [1]. Open issues discussed in this contribution are listed in the following table:

#### Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting Issue 1-1-4: Guard band for extended CBW - WF - FFS in next meeting Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Case 1: Case 3: - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW Issue 1-2-4: Relaxation of the extended IBE - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation

We present our further views on these open issues.

# 2 Discussion

Derivation of new inner RB region in extended CBW

- Regions based on original UE CBW

- Regions based on extended UE CBW

- Regions from shifted (a) and (b)

Fig. 1, Regions of extended UE CBW Vs Shifted regions of original UE CBW

First of all, the introduction of the concept “Extended UE CBW” is to define inner/outer/edge regions following legacy way but based on the extended channel bandwidth. Extension is done by keeping the same channel centre of original UE CBW and the extended CBW, hence in order to obtain the new inner region where enhanced MPR requirements are applied, we need to shift the starting RB in the original UE CBW with an offset such that the channel centre of the original UE CBW and extended CBW are the same, hence the new inner region should be intersect of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW.

Fig. 1 illustrates how to obtain the final region which enhanced MPR requirements are applied.

Proposal 1: RAN4 to define new inner region in extended UE CBW where enhanced MPR requirements are applied as the intersect of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW.

Guard band for extended CBW

As stated above, the main reason to introduce a concept of extended CBW is to have the same inner/outer/edge partitioning in an aligned way as before, thus it does not represent or require a new “channel filter”. With this motivation, there is no need to introduce guard band for extended CBW.

Proposal 2: RAN4 not to introduce guard band for extended CBW since extended CBW is a virtual concept to facilitate depicting of the new inner region where enhanced MPR requirements are applied.

Whether the extended IBE is applied in the GB of original UE CBW

The similar reasoning can be applied, i.e., there is not a real channel filter corresponding to extended CBW, therefore, only IBE within the original UE CBW is guaranteed, and RAN4 can specify IBE for the whole extended RBs as the IBE value calculated based on the outmost of allocated RBs in the original UE CBW, not a step-wise way, i.e., Case 1.

Proposal 3: RAN4 to specify the extended IBE value calculated IBE for the whole extended RBs as the IBE value calculated based on the outmost of allocated RBs in the original UE CBW, i.e., Case 1.

Relaxation of the extended IBE

As discussed in last meeting, if Case 1 is agreed, then use BPSK EVM in the IBE formula for all extended RBs.

Proposal 4: RAN4 to use BPSK EVM in the IBE formula for all extended RBs.

# 3 Conclusion

In this contribution, we have the following proposals for power domain enhancements for single carrier:

Proposal 1: RAN4 to define new inner region in extended UE CBW where enhanced MPR requirements are applied as the intersect of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW.

Proposal 2: RAN4 not to introduce guard band for extended CBW since extended CBW is a virtual concept to facilitate depicting of the new inner region where enhanced MPR requirements are applied.

Proposal 3: RAN4 to specify the extended IBE value calculated IBE for the whole extended RBs as the IBE value calculated based on the outmost of allocated RBs in the original UE CBW, i.e., Case 1.

Proposal 4: RAN4 to use BPSK EVM in the IBE formula for all extended RBs.

# Reference

[1] R4-2505099, “WF on power domain enhancements”, Huawei, HiSilicon