3GPP TSG-RAN WG4 Meeting #114bis	R4-2503160

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on NTN NGSO Performance Requirements

Agenda item:	7.10.4.2

Document for:	Discussion

Introduction

This document present Nokia’s views on IoT NTN SAN demodulation for discussion in RAN4#115.

Discussion

Channels to be assessed

Within Rel-19 , a WI has been agreed to specify a TE-emulated channel model for NTN aspects, within this performance requirements are expected to be derived with varying doppler and deplay shifts for NR-NTN and IoT-NTN

Nokia believes that as IoT-NTN is the most likely to be commercially deployed this work should aim to define the performance of NPUSCH within IoT NTN Deployments.

RAN4 shall define requirements for IOT NTN NPUSCH with HARQ disabled.

Specifically Nokia also argues that the existing parameters for IOT NTN could be used in this case, which are as follows:

NPUSCH with HARQ disabled parameters

Nokia feels that a sensible starting point would be to utilise the same parameters as have recently been used for defining PUSCH with HARQ enabled, which are as follows, taken from TS 36.181[1].

There are existing PUSCH requirements defined in TS 36.181 that can be utilised for alignment for requirements derivation for PUSCH with HARQ disabled.

Table 8.2.1.5-1 Minimum requirements for PUSCH, 1.4 MHz channel bandwidth for Mode A, 1Tx Number of TX antennas; Number of RX antennas; CE Mode; Propagation conditions and correlation matrix (annex F); FRC (annex A); Fraction of maximum throughput; SNR (dB). 1; 1; Mode A; NTN-TDLA100-5; A5-2; 70%; [-2.5]. 1; 1; Mode A; NTN-TDLC5-5; A5-2; 70%; [-2.8]. 1; 2; Mode A; NTN-TDLA100-5; A5-2; 70%; [-6.7]. 1; 2; Mode A; NTN-TDLC5-5; A5-2; 70%; [-6.7].

More specifically this becomes the following observations:

RAN4 currently uses Rank 1 for PUSCH performance requirements for IOT-NTN.

RAN4 currently uses 1Tx for PUSCH performance requirements for IOT-NTN.

RAN4 currently uses both 1 and 2Rx for PUSCH performance requirements for IOT-NTN.

RAN4 currently uses QPSK Modulation for PUSCH performance requirements for IOT-NTN.

RAN4 currently uses 1/3 code rate for PUSCH performance requirements for IOT-NTN.

RAN 4 shall utilise the same parameters as those already existing for PUSCH for IOT-NTN.

Revisiting 0.1 ppm

In the existing work of 3GPP in IOT NTN and NR NTN has made assumptions that the residual doppler is 0.1 ppm and the new channel is designed specifically to model variable doppler and delay, Nokia believes that this assumption of 0.1 ppm should be revisited.

RAN4 currently assumes that residual doppler at the UE is 0.1 ppm, however the new channel model will have variable doppler.

RAN4 to revisit the assumption of residual doppler of 0.1 ppm

Use of new channel model

As the Rel-19 Work Item focus is on the new channel model Nokia believes that all new requirements should be defined with said channel model in it’s current form.

RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state.

Conclusion

The paper discusses the topic of NTN NGSO Channel model for discussion at RAN4#114-bis

The following Observations and Proposals were made:

Proposal 1: RAN4 shall define requirements for IOT NTN NPUSCH with HARQ disabled.

Observation 1: There are existing PUSCH requirements defined in TS 36.181 that can be utilised for alignment for requirements derivation for PUSCH with HARQ disabled.

Observation 2: RAN4 currently uses Rank 1 for PUSCH performance requirements for IOT-NTN.

Observation 3: RAN4 currently uses 1Tx for PUSCH performance requirements for IOT-NTN.

Observation 4: RAN4 currently uses both 1 and 2Rx for PUSCH performance requirements for IOT-NTN.

Observation 5: RAN4 currently uses QPSK Modulation for PUSCH performance requirements for IOT-NTN.

Observation 6: RAN4 currently uses 1/3 code rate for PUSCH performance requirements for IOT-NTN.

Proposal 2: RAN 4 shall utilise the same parameters as those already existing for PUSCH for IOT-NTN.

Observation 7: RAN4 currently assumes that residual doppler at the UE is 0.1 ppm, however the new channel model will have variable doppler.

Proposal 3: RAN4 to revisit the assumption of residual doppler of 0.1 ppm

Proposal 4: RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state.

- References

- TS 36.181 Technical Specification Group Radio Access Network, Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node (SAN) conformance testing (Release 18) v18.1.0, September 2023

- TS 36.102 Technical Specification Group Radio Access Network, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access (Release 18) v18.3.0, September 2023