3GPP TSG-RAN4 #115	R4-2506281

St. Julian’s, Malta, 19th May – 23rd May 2025

Title:	FR2 test setup for AI/ML beam management evaluation

Source:			Anritsu Corporation

Agenda Item:			7.21.4

Document for:			Approval

# 1.	Introduction

At the RAN4 #114-bis meeting in Wuhan, we continued a discussion on the AI/ML topics. But due to the time constraint we could not have an official discussion on the beam management test setup. A part of the way forward [1] is extracted for reference.

Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

In this contribution, we share our views on test setup for AI/ML beam management evaluation and its associated channel model.

# 2.	Discussion

# 2.1. General view on AI/ML beam management

RAN4 continued a discussion on AI/ML beam management for several meetings. However, when we consider test cases for the beam management evaluation, we need to note that TR 38.843 [2] is assuming two kinds of sub-use cases, i.e. BM-Case 1 and BM-Case 2. And it appears that RAN4 was discussing the associated test setup mainly based on BM-Case 1 as follows.

BM-Case 1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams

BM-Case 2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams

To fulfill the test coverage of beam management, the associated test setup needs to consider both BM-Case 1 and Case 2 along with their channel models to be applied. Thus, we need to clarify all the necessary conditions for both BM-Case 1 and Case 2. Also in these days, some companies are mentioning that tests need to ensure a prediction performance of an AI model in the field. From that perspective, considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test. Companies are encouraged to bring views on the necessity of such mixed conditional tests.

Observation 1: RAN4 needs to clarify all the necessary conditions of associated test setup for both BM-Case 1 and Case 2 along with their channel models to be applied.

Observation 2: Considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test.

Proposal 1: Companies are encouraged to bring views on the necessity of such mixed conditional tests. (i.e. BM-Case 1 and/or BM-Case 2)

Also, from the perspective of ensuring the field performance by the AI model, we’d like to make the following two comments. The first point is that we have not concluded regarding channel models to apply to the BM tests in RAN4. Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice. The second point is that we think we need to conduct tests with multiple channel models. In a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc., so testing under a single condition may not be sufficient. Therefore, it can be said that a test result does not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model, and we propose defining multiple channel models like the existing CDL models.

Observation 3: Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice.

Observation 4: Because in a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. Therefore a test result does not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model.

Proposal 2: Define multiple channel models like the existing CDL models (e.g. CDL-1/2/3) to ensure the field performance of the AI model for beam management. FFS for the necessary number of channel models.

# 2.2 Views on Beam management case 1

BM-Case 1 (Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams)

We would first like to discuss presumable AI model patterns for BM-Case 1 based on the previous discussions in RAN1 and RAN4. Based on the previous discussions in the group and the report in TR 38.843 [2], when we conduct the evaluation of BM-Case 1, we think there are currently three possible AI model input assumptions as follows.

Observation 5: There are currently three kinds of presumable assumptions from the AI model input PoV.

AI-model-input assumption 1: No need to input Rx Beam information in the AI model.

In this case, it is expected that the UE may use an Rx beam for its RSRP measurement with a single direction.

AI-model-input assumption 2: One Rx Beam information is input to the AI model.

In this case, the direction of the UE Rx beam is the same as AI-model-input assumption 1 and that Rx Beam information is input to the AI model.

AI-model-input assumption 3: Multiple Rx Beam information is input to the AI model.

In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID.

Next, we’d like to consider the feasibility of BM-Case 1 tests for each AI-model-input assumption above.

Feasibility of BM-Case 1 tests with AI-model-input assumption 1 and 2

As already explained in the previous contribution [3], BM-Case 1 tests for AI-model-input assumption 1 and assumption 2 can be conducted by 1AoA test setup with the following conditions. (Note that we previously described those assumptions as UE pattern 1, 2 and 3.)

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the test.

Though there are some conditions to be decided, we think these conditions are not critical for the tests since it is just a matter of choice.

Details of the actual test procedure are described in Appendix A at the end of this contribution for reference.

Observation 6: BM-Case 1 tests with AI-model-input assumption 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) UE can be conducted by 1AoA setup with the following choices.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the evaluation.

As for the feasibility of the BM-Case 1 tests by multi-AoA test setup, as already mentioned above at observation 3 and 4, it depends on the channel model to be applied to conduct the tests. If multiple channel models are necessary to evaluate by multi-AoA test setup, or in a case the group identifies the performance issue with the simplified CDL model, there will be a testability issue since the measurement probes in the multi-AoA test setup are fixed in an OTA chamber, and there is a limit on a number of measurement probes to reduce the system complexity.

Observation 7: The feasibility of BM-Case 1 tests with AI-model-input assumption 1 and 2 UE by multi-AoA test setup depends on the channel models to be applied.

Feasibility of BM-Case 1 tests with AI-model-input assumption 3

As for the AI-model-input assumption 3, we would like to show our expectation of the behavior of the UE Rx beam during the test. Figure 2.2-1 depicts an example of the Rx beam behavior during the test of BM-Case 1 in MPAC. During the test in the MPAC, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to the UE implementation. In this situation the UE can obtain multiple measurement results of L1-RSRP even for one Tx Beam ID setting. Note the Rx beam direction is not always matching the direction of each measurement probe.

Observation 8: During the test with AI-model-input assumption 3 in the multi-AoA setup, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to UE implementation.

Figure 2.2-1: Example of the Rx beam behavior of the UE supporting AI-model-input assumption 3 during the test of BM-Case 1 in MPAC

Next, we would like to consider a corresponding test procedure with the 1AoA test setup. Since there is only one measurement probe in the chamber, measurement procedures are slightly different from the case of MPAC, and there are some conditions which need to be agreed and are expected to be difficult to converge.

Observation 9: To enable the test of UE with the AI-model-input assumption 3 by 1AoA test setup, there is a need to agree on the use of test modes.

Presumable conditions are described in Appendix B at the end of this contribution.

As for the feasibility of the tests by the multi-AoA setup, due to the same reason as observation 7, we think it also depends on the channel models to be applied.

Observation 10: The feasibility of BM-Case 1 tests with AI-model-input assumption 3 UE by multi-AoA test setup also depends on the channel models to be applied.

# 2.3 Views on Beam management case 2

BM-Case 2 (Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams)

As mentioned in sub-clause 2.1, there are two sub-use cases for beam management. And we think the group also needs to discuss associated channel models to be used for the evaluation of this BM-Case 2. According to the report attached in TR 38.843 [2], the link level simulation in RAN1 appears to apply an assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

Observation 11: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

UE moves from one point to another during the BM-Case 2 test.

Figure 2.3-1: Example of channel model in TR 38.901 clause 7.6.3 – Spatial consistency

Observation 12: When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE is moving, though there is a limitation that it cannot change the Rx Beam ID in a short time.

Observation 13: There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE moves, though there is a limitation that the system cannot change the Rx Beam ID in a short period without test mode.

When it comes to the multi-AoA setup, we anticipate that it also has some restrictions while testing the BM-Case 2. For example, it would be possible to temporally change transmitted powers from measurement probes. However, the following limitations are expected.

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

Observation 14: For BM-Case 2, it is expected that there are some testability issues with the multi-AoA setup.

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

# 2.4 Testability analysis of 1AoA and multi-AoA systems

Finally, testability analyses of each test system to conduct the tests of BM-Case 1 and/or Case 2 are summarised in Table 2.4-1 and Table 2.4-2.

Observation 15: Testability analysis of 1AoA and multi-AoA test setups are summarized as follows.

Table 2.4-1: Testability analysis of 1AoA test setup for BM-Case 1 and/or Case 2

| Test of BM-Cases | AI-model-input assumption |  |
| --- | --- | --- |
|  | Assumption 1 and 2 (Less than or equal to one Rx beam information is input) | Assumption 3 (Multiple Rx beam information is input) |
| BM-Case 1 | Yes Possible with any channel model conditions by simulating all channels within a fading simulator. | It depends. Testability issue remains in a case test mode is not applicable since the system cannot change the UE Rx beam ID in a short period. |
| BM-Case 2 |  |  |
| Both BM-Case 1 and 2 |  |  |
| Note | - Rx beam profile(s) needs to be standardized or obtained by measurement to include it in the fading simulator. - Feasibility issue of Rx beam sweeping with a UE supporting AI-model-input assumption 3 remains in a case the test mode is not applicable. - The required functions of the test mode could be either a function to pause L1-RSRP measurement or a function to control the Rx beam directions (ID). |  |

Table 2.4-2: Testability analysis of multi-AoA test setup for BM-Case 1 and/or Case 2

| Test of BM-Cases | AI-model-input assumption |  |
| --- | --- | --- |
|  | Assumption 1 and 2 (Less than or equal to one Rx beam information is input) | Assumption 3 (Multiple Rx beam information is input) |
| BM-Case 1 | It depends. Could be possible by applying a simplified CDL. FFS on the acceptable levels of the simplification. Testability issue remains in a case multiple CDL models need to be used or if the simplified CDL model cannot be used to ensure the prediction performance of the AI model in a field. |  |
| BM-Case 2 | It depends. There are some restrictions on the achievable channel environment - Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce. - Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber. |  |
| Both BM-Case 1 and 2 |  |  |
| Note | - A study has not been completed on the applicable channel models to ensure the prediction performance of the AI model in the field. So, it is not clear yet if the simplified CDL channel model can suffice. |  |

As a summary, considering the observations in this paper, we assume that there is no comprehensive test solution to evaluate BM-Case 1 and BM-Case 2 at this moment. Therefore, as long as we need to conduct the tests for both BM-Case 1 and BM-Case 2, we believe that we still need to keep both 1AoA and multi-AoA setups as test system candidates and use the applicable system with case-by-case manner for the evaluation of these beam management AI performances.

Observation 16: There is no comprehensive test solution to evaluate both BM-Case 1 and BM-Case 2 at this moment.

Proposal 3: Use different types of test setups (i.e. 1AoA or multi-AoA setup) depending on the AI-model-input assumptions and BM-Case 1 and/or 2 with case-by-case manner.

- Conclusion

In this contribution, we shared our views on test setup for AI/ML beam management evaluation and its associated channel model.

Observation 1: RAN4 needs to clarify all the necessary conditions of associated test setup for both BM-Case 1 and Case 2 along with their channel models to be applied.

Observation 2: Considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test.

Proposal 1: Companies are encouraged to bring views on the necessity of such mixed conditional tests. (i.e. BM-Case 1 and/or BM-Case 2)

Observation 3: Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice.

Observation 4: Because in a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. Therefore a test result does not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model.

Proposal 2: Define multiple channel models like the existing CDL models (e.g. CDL-1/2/3) to ensure the field performance of the AI model for beam management. FFS for the necessary number of channel models.

Observation 5: There are currently three kinds of presumable assumptions from the AI model input PoV.

AI-model-input assumption 1: No need to input Rx Beam information in the AI model.

In this case, it is expected that the UE may use an Rx beam for its RSRP measurement with a single direction.

AI-model-input assumption 2: One Rx Beam information is input to the AI model.

In this case, the direction of the UE Rx beam is the same as AI-model-input assumption 1 and that Rx Beam information is input to the AI model.

AI-model-input assumption 3: Multiple Rx Beam information is input to the AI model.

In this case, for each Tx Beam ID in set B, UE will change its Rx beam direction to multiple directions and multiple L1-RSRP measurements are made per Tx Beam ID. Multiple Rx Beam information is input to AI model per Tx Beam ID.

Observation 6: BM-Case 1 tests with AI-model-input assumption 1 and 2 (i.e. less than or equal to one Rx Beam information is input to the AI model) UE can be conducted by 1AoA setup with the following choices.

- Rx beam pattern is standardized or,

- Rx beam pattern is measured during the evaluation.

Observation 7: The feasibility of BM-Case 1 tests with AI-model-input assumption 1 and 2 UE by multi-AoA test setup depends on the channel models to be applied.

Observation 8: During the test with AI-model-input assumption 3 in the multi-AoA setup, UE Rx beam would be swept to multiple directions even for the measurement period of one Tx Beam ID, where the detailed number of Rx Beam ID to be used for the measurement would be up to UE implementation.

Observation 9: To enable the test of UE with the AI-model-input assumption 3 by 1AoA test setup, there is a need to agree on the use of test modes.

Observation 10: The feasibility of BM-Case 1 tests with AI-model-input assumption 3 UE by multi-AoA test setup also depends on the channel models to be applied.

Observation 11: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

Observation 12: When considering the test method of BM-Case 2 in RAN4, we need to study how we can create the channel environment by the candidate test setups (1AoA or multiple AoA) in a case that the UE moves from one point to another.

Observation 13: There is a benefit that 1AoA test system with the fading simulator can simulate the channel model even with the case that UE moves, though there is a limitation that the system cannot change the Rx Beam ID in a short period without test mode.

Observation 14: For BM-Case 2, it is expected that there are some testability issues with the multi-AoA setup.

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

Observation 15: Testability analysis of 1AoA and multi-AoA test setups are summarized as follows.

Table 2.4-1: Testability analysis of 1AoA test setup for BM-Case 1 and/or Case 2

| Test of BM-Cases | AI-model-input assumption |  |
| --- | --- | --- |
|  | Assumption 1 and 2 (Less than or equal to one Rx beam information is input) | Assumption 3 (Multiple Rx beam information is input) |
| BM-Case 1 | Yes Possible with any channel model conditions by simulating all channels within a fading simulator. | It depends. Testability issue remains in a case test mode is not applicable since the system cannot change the UE Rx beam ID in a short period. |
| BM-Case 2 |  |  |
| Both BM-Case 1 and 2 |  |  |
| Note | - Rx beam profile(s) needs to be standardized or obtained by measurement to include it in the fading simulator. - Feasibility issue of Rx beam sweeping with a UE supporting AI-model-input assumption 3 remains in a case the test mode is not applicable. - The required functions of the test mode could be either a function to pause L1-RSRP measurement or a function to control the Rx beam directions (ID). |  |

Table 2.4-2: Testability analysis of multi-AoA test setup for BM-Case 1 and/or Case 2

| Test of BM-Cases | AI-model-input assumption |  |
| --- | --- | --- |
|  | Assumption 1 and 2 (Less than or equal to one Rx beam information is input) | Assumption 3 (Multiple Rx beam information is input) |
| BM-Case 1 | It depends. Could be possible by applying a simplified CDL. FFS on the acceptable levels of the simplification. Testability issue remains in a case multiple CDL models need to be used or if the simplified CDL model cannot be used to ensure the prediction performance of the AI model in a field. |  |
| BM-Case 2 | It depends. There are some restrictions on the achievable channel environment - Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce. - Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber. |  |
| Both BM-Case 1 and 2 |  |  |
| Note | - A study has not been completed on the applicable channel models to ensure the prediction performance of the AI model in the field. So, it is not clear yet if the simplified CDL channel model can suffice. |  |

Observation 16: There is no comprehensive test solution to evaluate both BM-Case 1 and BM-Case 2 at this moment.

Proposal 3: Use different types of test setups (i.e. 1AoA or multi-AoA setup) depending on the AI-model-input assumptions and BM-Case 1 and/or 2 with case-by-case manner.

# 4.	References

[1] R4-2505105, “WF on requirements for AI/ML air interface normative work”, Moderator (Qualcomm), RAN4 #114-bis, Wuhan, China

[2] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V18.0.0

[3] R4-2503711, “FR2 test setup for AI/ML beam management evaluation”, Anritsu, RAN4 #114-bis, Wuhan, China

[4] TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz", v18.0.0, 2024-03

# Appendix A: Measurement procedures of 1AoA + Fading simulator with wireless (virtual) cable connection

In this appendix, we provide supplemental explanations on the measurement procedures of 1AoA + Fading simulator with wireless cable connection in sub-clause 2.2. Measurement procedures may vary depending on an assumption whether the UE Rx beam pattern is the AI model input or not, and whether the actual UE Rx beam pattern is necessary or not.

AI-model-input assumption 1: UE Rx beam information is not AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator, though it is not mandatory.

AI-model-input assumption 2: UE Rx beam information is AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator.

We will explain the associated measurement procedure for each case as follows.

AI-model-input assumption 1 a): UE Rx beam information is not AI model input and use a standardized UE Rx beam pattern in the fading simulator

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at the position where the UE Rx beam directs towards the bore sight direction and set the UE in the Rx-beam-lock mode. The Rx-beam-lock mode needs to be kept in step 5 and step 6.

- Establish a wireless (virtual) cable connection.

- Set a standardized Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management.  (If it is required for the AI model to correctly output inferring results with any Rx-antenna steering patterns, it would be necessary to perform testing multiple times with various combinations of a UE orientation and a standardized Rx-antenna pattern being loaded into the fading simulator.)

AI-model-input assumption 1 b): UE Rx beam information is not AI model input and use an actually measured UE Rx beam pattern in the fading simulator

The measurement procedure for “AI model input assumption 1 b)” would be similar to the one in “AI model input assumption 2”.  The difference would be as the following.

- The same measurement procedure for “AI model input assumption 2” can be used for “AI model input assumption 1 b)”, but it is not required that the Rx-beam recognition in the UE and the Rx-beam pattern used in the fading simulator are aligned.  So, in the case of “AI model input assumption 1 b)”, it should be allowed to change the Rx-beam direction between the time when measuring the Tx/Rx-beam pattern and the time when establishing the wireless connection.  If necessary, the wireless connection can be established with UE Rx-beam directed towards the bore sight direction.

Comparing cases between the AI model input assumption 1 a) and 1 b), AI model input assumption 1 b) has a benefit that the fading simulator can use the actual UE Rx beam pattern in its channel simulation with a cost of an additional measurement time.

AI model input assumption 2 a): UE Rx beam information is AI model input and use a standardized UE Rx beam pattern in the fading simulator

Refer to the previous AI-model-input assumption 1 a) since the procedures are same.

AI model input assumption 2 b): UE Rx beam information is AI model input and use an actually measured UE Rx beam pattern in the fading simulator

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at a certain position and set the UE in the Rx-beam-lock mode by UBF test command. The Rx-beam-lock mode needs to be kept in step 5, 6 and 7.

- Rotate the UE orientation by the positioner to measure a 3-dimensional Tx-antenna pattern of the UE, while assuming reciprocity between Tx and Rx antenna pattern. (Without ATF: Antenna Test Function being equipped in the UE, it wouldn’t be possible to directly measure Rx-antenna pattern.)

- Set the UE orientation to the same position as in step 4 and establish a wireless (virtual) cable connection. By this connection, DL signals based on the simulated channel model will be distributed virtually directly to an antenna connector of the UE.

- Set the measured Tx/Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management, where the Rx-beam ID in UE recognition should be aligned with the Rx-antenna pattern in the fading simulator.  (If necessary, we can perform testing several times with various UE orientations in the simulation while fixing the Rx-beam ID.)

Cross-polarized antenna

UE

Positioner

Step 4. : Stop changing UE orientation and set Rx-beam-lock mode

Cross-polarized antenna

UE

Positioner

Step 5. : Rotate UE while measuring Tx-antenna pattern

Cross-polarized antenna

UE

Positioner

Step 6. : Set UE orientation to the one in step4 to establish wireless-cable connection

Wireless cable connection

Rotate

If it is required to perform testing with multiple Tx/Rx-antenna patterns or Rx-beam IDs, repeat from step 3 to step 7.

Appendix B: Conditions to test UEs with AI-model-input assumption 3 by 1AoA test setup

Here are the conditions to enable testing UEs with AI-model-input assumption 3 by 1AoA test setup. Generally speaking, a test mode is necessary to conduct the associated tests.

Condition 1: Need to allow some time to move UE positions by a positioner.

For example, introduce a test mode to set a gap to run measurements of L1-RSRP, or signaling is introduced to pause and resume L1-RSRP measurements while the positioner is rotating. Or a remote command to control Rx beam directions (ID) could also work.

Condition 2: Rx beam directions for the measurement are declared by UE vendors.

There is a need to align the Rx beam directions between the actual UE beam and the internally simulated Rx beam patterns during the test.

Condition 3: Like the AI-model-input assumption 1 and 2, Rx beam profile is standardized.