# 3GPP TSG-RAN WG4 Meeting # 115		                              R4-2506323

# Malta, MT, 19th – 23rd May, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on BS RF requirements for supporting 7 MHz channel bandwidth

# Agenda Item:	7.8.4

# Document for:	Approval

- Introduction

- In the previous meetings, RAN4 have made lots of progress in 7 MHz BS RF requirements[1], however there are still some remaining issues left for further discussions. In this contribution, we provide some further opinions on the ICS requirement.

- Discussion

In last meeting, RAN4 have achieved a consensus that the legacy FRCs are reused for 7 MHz requirements, so for ICS, G-FR1-A1-7 can be used as the reference measurement channel. Moreover, type of interfering signal for 7 MHz should also be considered. In our view, taking 15 PRBs DFT-s-OFDM NR signal as the interfering signal is preferred, as the wanted signal and interfering signal can be placed adjacently around center frequency Fc. The parameters for ICS requirement are listed in Table 1 below:

Table 1 Parameters for 7 MHz in-channel selectivity for conducted characteristics

| Base station classes | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| WA | 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| MR |  |  |  | -95.6 | -74.6 |  |
| LA |  |  |  | -92.6 | -71.6 |  |

For NB-IoT operation in NR in-band, legacy FRCs are reused for 7 MHz, and the requirements of interfering signal are aligned with that of wide band operation. The parameters for NB-IoT operation in NR in-band are listed in Table 2 below:

Table 2 Parameters for 7 MHz in-channel selectivity for NB-IoT operation

in NR in-band for conducted characteristics

| Base station classes | BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| WA | 7 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -124.3 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15RBs |
| MR |  |  | -119.3 | -74.6 |  |
| LA |  |  | -116.3 | -71.6 |  |
| WA | 7 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -130.2 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| MR |  |  | -125.2 | -74.6 |  |
| LA |  |  | -122.2 | -71.6 |  |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |  |

For radiated characteristics, only BS type 1-O need to be considered, the offset -ΔminSENS is considered based on the conducted characteristics to determine the ICS requirement parameters, as listed in Table 3:

Table 3 Parameters for 7 MHz in-channel selectivity for radiated characteristics

| Base station classes | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| WA | 7 | 15 | G-FR1-A1-7 | -100.6-ΔminSENS | -79.6-ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| MR |  |  |  | -95.6-ΔminSENS | -74.6-ΔminSENS |  |
| LA |  |  |  | -92.6-ΔminSENS | -71.6-ΔminSENS |  |

Proposal 1: To consider the parameters in Table 1, Table 2 and Table 3 for BS in-channel selectivity RF requirement for 7 MHz channel bandwidths.

- Conclusion

In this contribution, some views on BS RF requirements are provided and the proposals ares made as following:

Proposal 1: To consider the parameters in Table 1, Table 2 and Table 3 for BS in-channel selectivity RF requirement for 7 MHz channel bandwidths.

- Reference

R4-2505098, WF on requirements for 7MHz channel bandwidth, Ericsson.

RP-243323, 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA.

RP-250373, NR FR1 7 MHz Channel Bandwidth , China Telecom, T-Mobile USA.

3GPP TS38.104 NR Base Station (BS) radio transmission and reception.

3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio                           transmission and reception"