3GPP TSG-RAN WG4 Meeting #115	R4-2506361

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for FR2-1 L3 measurement enhancement

Agenda Item:	7.18.5

Document for:	Discussion

# Introduction

This contribution outlines our point of view on performance requirements for FR2-1 L3 measurement enhancement based on the relevant core requirements which have been agreed in previous meetings.

# Discussion

## Topic #1: Performance requirements for FBS L3 measurement

The below test cases have been agreed and outlined in the WF in the last meeting.

| Index | Scenario | Need or not | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 3 | Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 2 | Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra | 1 or 2 TCs | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 4 | Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter |  | TBA(e.g., DRX or non-DRX, testing quitting FBS or not) |
| 5 | Scenario 5: Handover | 1 or 2 TCs | TBA(e.g., Intra-frequency or inter-frequency, unknown) |
| 6 | Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection | 1 or 2 TCs | TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown) |

The leftover is DRX configurations and testing quitting FBS or not.

Proposal 1: RAN4 to consider at least the below configurations for the test cases with respect to the agreed scope in the last meeting:

- Inter-frequency handover from FR2 to FR2; unknown target cell for UE configured with [FBS signaling]

- Inter-frequency RRC Re-establishment in FR2, with unknown cells.

- Intra/Inter-frequency Measurements without DRX.

Proposal 2: Quitting FBS only needs to be verified in few test cases, e.g., Scenario 1: SSB based Intra-frequency measurement without MG and Scenario 3: SSB based Inter-frequency measurement without MG.

## Topic #2: Performance requirements for CSSF enhancement

The below agreements on test cases for solution 1 were reached.

| TC index | Scenario | Need or not | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them | TBA |

Proposal 3: For CSSF solution 1, FR2 CA (PCC and SCC both are FR2, in a same band) and EN-DC (one FR2 PSC and one FR2 SCC, in a same band) shall be considered.

Proposal 4: For CSSF solution 1, the below configurations are considered:

- Without SSB time index detection

- When DRX is not used

The below agreements on test cases for solution 3 were reached.

| TC index | Aspects | CA/DC mode | Detailed configurations and scopes |
| --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them | TBA |

For solution 3, the test cases shall capture the shorted measurement delay by applying the 3rd searcher. The 3rd searcher can behave the effect when there are at the least 2 CCs, which is for the 2nd searcher today, are configured.

Proposal 5: For CSSF solution 3, 2 either CA or DC configuration shall configure 2 CCs.

Proposal 6: For CSSF solution 3, the below configurations are considered:

- Without SSB time index detection

- When DRX is not used

# Conclusion

Proposal 1: RAN4 to consider at least the below configurations for the test cases with respect to the agreed scope in the last meeting:

- Inter-frequency handover from FR2 to FR2; unknown target cell for UE configured with [FBS signaling]

- Inter-frequency RRC Re-establishment in FR2, with unknown cells.

- Intra/Inter-frequency Measurements without DRX.

Proposal 2: Quitting FBS only needs to be verified in few test cases, e.g., Scenario 1: SSB based Intra-frequency measurement without MG and Scenario 3: SSB based Inter-frequency measurement without MG.

Proposal 3: For CSSF solution 1, FR2 CA (PCC and SCC both are FR2, in a same band) and EN-DC (one FR2 PSC and one FR2 SCC, in a same band) shall be considered.

Proposal 4: For CSSF solution 1, the below configurations are considered:

- Without SSB time index detection

- When DRX is not used

Proposal 5: For CSSF solution 3, 2 either CA or DC configuration shall configure 2 CCs.

Proposal 6: For CSSF solution 3, the below configurations are considered:

- Without SSB time index detection

- When DRX is not used

# References