3GPP TSG-RAN WG4 Meeting #115                                   R4-2506529

# Malta, MT, 19th – 23rd May, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on NTN Ku band VSAT RF requirements

Agenda Item:	7.11.5

Document for:	Approval

- Introduction

In RAN4#114bis, the WF for NTN Ku band UE RF requirements [1] was agreed upon. However, there are still some unresolved issues that need further discussion. In this paper, we will provide our views on the remaining issues.

- Issue 1-1-4: General RF requirements impacts on numerologies dependency - The following aspects are considered when defining the general RF requirements for the Ku bands. - No update needed on 9.3.1 Minimum output power, 9.3.2 transmit off power, 9.3.4 Power control, and 9.4 Transmitter signal quality for supporting Ku bands. - For 9.3.3 Transmit ON/OFF time mask, - The time masks and transient period times will be re-used for FR2-NTN Ku bands - FFS on FR1-NTN Ku bands - Issue 1-1-5: Handling of FCC, ISED, CEPT regulation related requirement - New NSs will be defined for the Ku band concerning different regulations - FFS on whether different NS numbers will be used for FR1-NTN Ku bands and FR2-NTN Ku bands - Different NS numbers will be used to handle FCC and CEPT regulation at least - FFS on how to handle FCC and ISED requirements in TS 38.101-5 - Option 1: capture the most stringent requirement with same NSs - Option 2: separate NSs - Issue 1-2-1: Minimum EIRP requirements - Consider the following as the starting point Operating band; UE Type; Min peak EIRP (dBm). [When communicating with LEO]; [When communicating with GSO]. n248, n247, n509, n508; 4; [64]; [70]. 5; [64]; [70]. - FFS on whether further clarification is needed

- Issue 1-3-1: EIS requirement Agreement: - FFS on the following two options - Option 1: - Defining EISREFSENS_10MHz for FR1-NTN Ku bands, and define EISREFSENS_50MHz for FR2-NTN Ku bands - Option 2: - Consider define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands - FSS on the value of the EISREFSENS_10MHz and EISREFSENS_50MHz - The requirement for other channel bandwidth will be scaled based on PRB based

- Discussion

  - NS numbers used for FR1-numerology Ku bands and FR2-numerology Ku bands

Whether different NS numbers will be used for FR1-numerology Ku bands and FR2-numerology Ku bands when considering the same regulation. In our view, to mitigate the burden on the network, the same NS number should be used for FR1-numerology Ku bands and FR2-numerology Ku bands as the starting point when considering the same regulation. If there is an issue with using the same NS, different NS numbers could be considered.

Proposal 1 : RAN4 to consider using the same NS number for FR1-numerology Ku bands and FR2-numerology Ku bands as the starting point when considering the same regulation. If there is an issue with using the same NS, different NS numbers could be considered.

  - Minimum peak EIRP

In the last RAN4 meeting, the minimum peak EIRP for UE type 4/5 was discussed and specified as two sets of requirements for communicating with LEO and GSO, as shown in Table 1. Further clarification is needed for the two sets of minimum peak EIRP for communication with LEO and GSO. From the type description for NTN VSAT type 4 and 5, which involve mobile VSAT communicating with GSO and LEO using mechanical steering antennas and electric steering antennas respectively, as shown in Table 2, it seems that supporting both GSO and LEO in one VSAT is not mandatory according to TS 38.101-5. Currently, some commercial VSATs only support LEO, while others support both LEO and GSO. Therefore, RAN4 needs to clarify whether the mobile NTN VSAT for type 4/5 could only support LEO or support both LEO and GSO in one VSAT.

Observation 1: From the type description for NTN VSAT type 4 and 5, which involve mobile VSAT communicating with GSO and LEO using mechanical steering antennas and electric steering antennas respectively, as shown in Table 2, it seems that supporting both GSO and LEO in one VSAT is not mandatory according to TS 38.101-5.

Proposal 2 : RAN4 needs to clarify whether the mobile NTN VSAT for type 4/5 could only support LEO or support both LEO and GSO in one VSAT.

If the mobile NTN VSAT for type 4/5 supporting both GSO and LEO in one VSAT is not mandatory, it means that it could support only LEO or support both LEO and GSO. Therefore, the minimum peak EIRP for UE type 4/5 could be specified as two sets of requirements. According to the WF [2] for NTN Ku-band coexistence simulation, some parameters are updated, and these parameters are used to specify VSAT RF requirements. The uplink link budget with 100MHz CBW for GSO and LEO is shown in Table 3. The NTN VSAT has a minimum peak EIRP of 61 dBm for communicating with LEO and a minimum peak EIRP of 82 dBm for communicating with GSO with 100MHz CBW. It should be noted that the minimum peak EIRP is up to 82 dBm when communicating with GSO with 100MHz CBW, which leads to a large antenna size. In our view, the CBW can be further discussed for the minimum peak EIRP when communicating with GSO to make the antenna size more feasible for mobile VSAT.

Table 1.

| Operating band | UE Type | Min peak EIRP (dBm) |  |
| --- | --- | --- | --- |
|  |  | [When communicating with LEO] | [When communicating with GSO] |
| n248, n247, n509, n508 | 4 | [64] | [70] |
|  | 5 | [64] | [70] |

Table 2

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247 |  |  |

Table 3: NTN Ku-band UL link budget analysis

|  | Ku band |  |  |
| --- | --- | --- | --- |
| Satellite type | GSO | LEO 1200 | LEO 600 |
| Elevation angle (VSAT) | 35 deg | 35 deg | 35 deg |
| Distance (VSAT-SAN) | 38178.5 km | 1830.6 km | 967.3 km |
| UL frequency | 14500MHz | 14500MHz | 14500MHz |
| Free space path loss (dB) | 207.3 | 180.9 | 175.4 |
| Atmosphere loss (dB) | 2 | 2 | 2 |
| BW (MHz) | 100 | 100 | 100 |
| target SNR | -1 | -1 | -1 |
| SAN noise figure (dB) | 3 | 3 | 3 |
| SAN Rx gain (dBi) | 44 | 38.5 | 38.5 |
| Implementation margin | 2 | 2 | 2 |
| minimum EIRP | 81.4 | 60.5 | 54.9 |

Proposal 3: The CBW can be further discussed for the minimum peak EIRP when communicating with GSO.

Proposal 4: RAN4 to consider a VSAT with a minimum peak EIRP of 61 dBm when communicating with LEO.

  - EIS

For the EIS , there are some unsolved issues are list as below:

- FFS on the following two options

  - Option 1:

    - Defining EISREFSENS_10MHz for FR1-NTN Ku bands, and define EISREFSENS_50MHz for FR2-NTN Ku bands

  - Option 2:

    - Consider define the common EIS requirement for both FR1-NTN Ku bands and FR2-NTN Ku bands

- FSS on the value of the EISREFSENS_10MHz and EISREFSENS_50MHz

In our view, we prefer that the EIS requirement is defined based on the smallest supported channel bandwidth declared by the vendor and use this to derive EIS for larger supported channel bandwidths. Therefore, we support option 1, which defines EISREFSENS_10MHz for FR1-numerology Ku bands and EISREFSENS_50MHz for FR2-numerology Ku bands. Based on parameter assumptions from the WF [2] for NTN Ku-band coexistence simulation, the DL link budget calculation is shown in Table 4. From Table 4, we can see that the EIS for LEO is the most stringent one. Therefore, EIS for 50MHz CBW should be less than -124dBm, and EIS for 10MHz CBW could be -124 + 10*log(10/50) = -131dBm.

Proposal 5: RAN4 to consider the EISREFSENS_50MHz should be less than -124 dBm and the EISREFSENS_10MHz should be less than -131 dBm.

Table 4: NTN Ku-band DL link budget analysis

|  | Ku band |  |
| --- | --- | --- |
| Satellite type | GSO | LEO 600 |
| Elevation angle (VSAT) | 35 deg | 35 deg |
| Distance (VSAT-SAN) | 38178.5 km | 967.3 km |
| DL frequency | 11000MHz | 11000MHz |
| density(dBW/MHz) | 40 | 4 |
| BW (MHz) | 50 | 50 |
| Satellite EIRP (dBm) | 87 | 51 |
| Free space path loss (dB) | 204.9 | 173 |
| Atmosphere loss (dB) | 2 | 2 |
| VSAT DL Received signal level at OTA (dBm) | -119.9 | -124 |

- 3	Conclusion

In this contribution, we provide our views on the remaining issues in the last RAN4 meeting. The proposals are summarized as follows:

Proposal 1 : RAN4 to consider using the same NS number for FR1-numerology Ku bands and FR2-numerology Ku bands as the starting point when considering the same regulation. If there is an issue with using the same NS, different NS numbers could be considered.

Observation 1: From the type description for NTN VSAT type 4 and 5, which involve mobile VSAT communicating with GSO and LEO using mechanical steering antennas and electric steering antennas respectively, as shown in Table 2, it seems that supporting both GSO and LEO in one VSAT is not mandatory according to TS 38.101-5.

Proposal 2 : RAN4 needs to clarify whether the mobile NTN VSAT for type 4/5 could only support LEO or support both LEO and GSO in one VSAT.

Proposal 3: The CBW can be further discussed for the minimum peak EIRP when communicating with GSO.

Proposal 4: RAN4 to consider a VSAT with a minimum peak EIRP of 61 dBm when communicating with LEO.

Proposal 5: RAN4 to consider the EISREFSENS_50MHz should be less than -124 dBm and the EISREFSENS_10MHz should be less than -131 dBm.

- 4	References

- R4-2504720, “Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF”, CHTTL, RAN4#114bis.

- R4-2504723, “Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence”, Intelsat, RAN4#114bis.