3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506567

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.29.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s beneficial to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

2.1 General

In R18 mobility, RAN4 has agreed that the cell switch delay requirement is applicable for the case without L1 measurement and report under certain conditions for FR1. Meanwhile, for FR2, the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. Using the same processing method, we agreed that CLTM can be triggered through L3 measurement in FR1 in previous meeting, while FR2 does not support L3 measurement triggering. In this case, we need to discuss whether to support deactivated SCell on a case by case basis.

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

In the last meeting, we discussed the applicable scenarios for conditional LTM and there us an issue need further discusse that whether to support deactivated SCell and the cells with same frequency as deacticated Scell for CLTM triggered by L1 measurement. As far as we know, this is related to the mobility requirements of R18. In R18, there is no requirement for L1-RSRP measurement on deactivated SCell. Therefore, if we want to support deactivated SCell for L1 triggered CLTM, L1-RSRP measurement on deactivated SCell should be condidered at first.

RAN4#111 Issue 2-1-1: whether to consider L1-RSRP measurement on deactivated SCell Online agreement < Agreement>: - In Rel-18, RAN4 will not define the requirement for L1-RSRP measurement on deactivated SCell. - It is up to UE on how to perform the measurement if the L1-RSRP measurement on deactivated SCell is supported based on RAN1/2 design.

Deactivated SCell is already supported in R18, but we did not define the requirements during the maintenance phase due to potential impact on other work groups and time constraints. In our understanding, if time permits, we tend to consider the case. Meanwhile, we can compromise to not define requirement.

Proposal 1：For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included if time permits.

Regarding the issue of UE based TA measurement, RAN4 has also discussed the same issue in R18 mobility, but due to time constraints, we did not define dedicated requirements. In the discussion of R19 conditional LTM, RAN2 agreed to consider two TA acquisition methods at the RAN2#127bis meeting, including UE based TA measurement and PDCCH ordered early TA acquisition. At the same time, signaling details were discussed in RAN2#128 meeting. So, from the perspective of RAN4, we should define the relevant requirements. However, based on the current RAN2 signaling design, even without defining corresponding requirements for UE based TA measurement, the network can still send TA values in a separate MAC CE. When both the network indication and the estimated TA value of the UE are present, the UE should follow the values captured based on the network ordered TA. UE based TA estimation is only applicable when the network does not indicate a TA value.

Observation 1: Even without defining corresponding requirements for UE based TA measurement, the network can still send TA values in a separate MAC CE.

2.2 Conditional LTM delay

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

In last meeting, RAN4 has reached agreement that CHO requirements framework can be used as baseline for Conditional LTM. As to the definiation of components, we have discussed one by one and there were still some FFS need to be considered in this meeting.

Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

For the first FFS, Tmeasure for CSI-RS based L1-RSRP measurement should be discussed based on the agreement for other part, therefore, we should study the CSI-RS based L1-RSRP measurement period first and finish the definiation of Tmeasure as soon as progress is made.

For the second FFS,

As to interruption, there are two options to choose according to the summary of the previous meeting.

  - TCLTM_interrupt = TLTM-processing + TLTM-IU ms

  - TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Furthermore, we need to discuss whether T/F fine tracking can be performed before Tprocessing. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. If T/F fine tracking is needed, we doubt whether it can be executed before target cell parameters apply and L1/L2 change. Of course, the value of Tfirst-RS and TRS-proc can be zero under some condition, such as early sync is supported and maintain.

In our understanding, it is beneficial that T/F fine tracking if needed can be executed at first and then TLTM-processing during conditional LTM, however, it’s diffcult to define the value that the uncertainty in acquiring the first SSB for T/F tracking on the new cell. Maybe it can be discussed in further release.

Proposal 2: For R19 CLTM, TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms.

2.3 Subsequent Conditional LTM delay

As we mentioned before, the difference between conditional LTM delay and subsequent conditional LTM delay is whether to consider RRC procedure. Therefore, Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Meanwhile, the definition of TEvent_DU need to be updated, which is similar as subsequent CPAC in R18.

RAN4#106bis Issue 1-1: starting point of subsequent CPC in RRM requirements Starting point of subsequent CPC in RRM requirements is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. Issue 1-2: ending point of subsequent CPC in RRM requirements Ending point of subsequent CPC in RRM requirements is the transmission of PRACH preamble towards the target PSCell. Issue 1-3: subsequent CPC delay requirements For subsequent CPC delay requirements: if starting point is the time when UE completes the previous CPC/CPA, Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms and the definition of each component is the same as 8.11B.2 in TS38.133 except that TEvent_DU needs to be updated. Issue 1-4: TEvent_DU in subsequent CPC delay requirements TEvent_DU is the delay uncertainty which is the time from [when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change] until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

Proposal 3: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1：For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included if time permits.

Observation 1: even without defining corresponding requirements for UE based TA measurement, the network can still send TA values in a separate MAC CE.

Proposal 2: For R19 CLTM, TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms.

Proposal 3: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

References

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom