3GPP TSG-RAN WG4 Meeting #115	R4-2506627

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.17.4

Document for:	Information

# Introduction

In RAN4#114bis meeting, companies had agreement on simulation assumptions for Rel-19 NR MMSE-IRC BS demodulation requirement [1]. In this contribution, simulation results are delivered for collection summary.

# Simulations

In this simulation, the DIP values in LTE MMSE-IRC BS interference profile are converted to INR values.

2.1	Simulation assumptions

Table 2.1-1 Mapping from LTE DIP values to INR values

| Scenario | Interferer number | (DIP1, DIP2) [dB] | (INR1, INR2) [dB] |
| --- | --- | --- | --- |
| HomNet | 1 interferer | (-1.11, NA) | (5.36, NA) |
|  | 2 interferers | (-1.11, -10.91) | (7.29, -2.5) |
| HetNet | 1 interferer | (-0.43, NA) | (9.83, NA) |
|  | 2 interferers | (-0.43, -13.78) | (12.38, -0.97) |

Table 2.1-2: Test parameters

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A, B |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |  |  |

2.2	Simulation results

Table 2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.52 | 15.26 | 3.74 |
|  |  | TDLA30-10 |  |  | B |  | 11.63 | 15.33 | 3.70 |
|  |  | TDLC300-100 |  | 5.36 | A | 16 | 11.75 | 13.35 | 1.60 |
|  |  | TDLC300-100 |  |  | B |  | 11.81 | 13.42 | 1.61 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 7.55 | 13.65 | 6.1 |
|  |  | TDLA30-10 |  |  | B |  | 7.77 | 13.66 | 5.89 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 8.18 | 11.31 | 3.13 |
|  |  | TDLC300-100 |  |  | B |  | 8.24 | 11.37 | 3.13 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 3.55 | 10.13 | 6.58 |
|  |  | TDLA30-10 |  |  | B |  | 3.68 | 10.22 | 6.54 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 4.14 | 7.51 | 3.37 |
|  |  | TDLC300-100 |  |  | B |  | 4.22 | 7.69 | 3.47 |

Table 2.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 20 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.86 | 15.60 | 3.74 |
|  |  | TDLA30-10 |  |  | B |  | 12.02 | 15.60 | 3.58 |
|  |  | TDLC300-100 |  | 5.36 | A | 16 | 11.96 | 13.45 | 1.49 |
|  |  | TDLC300-100 |  |  | B |  | 11.99 | 13.49 | 1.50 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 8.19 | 14.11 | 5.92 |
|  |  | TDLA30-10 |  |  | B |  | 8.31 | 14.13 | 5.82 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 8.30 | 11.42 | 3.12 |
|  |  | TDLC300-100 |  |  | B |  | 8.33 | 11.43 | 3.10 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 3.79 | 10.31 | 6.52 |
|  |  | TDLA30-10 |  |  | B |  | 3.92 | 10.33 | 6.41 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 4.21 | 7.56 | 3.35 |
|  |  | TDLC300-100 |  |  | B |  | 4.32 | 7.63 | 3.31 |

Table 2.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.65 | 15.29 | 3.64 |
|  |  | TDLA30-10 |  |  | B |  | 11.66 | 15.29 | 3.63 |
|  |  | TDLC300-100 |  | 5.36 | A | 16 | 12.09 | 13.61 | 1.52 |
|  |  | TDLC300-100 |  |  | B |  | 12.10 | 13.61 | 1.51 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 8.13 | 13.94 | 5.81 |
|  |  | TDLA30-10 |  |  | B |  | 8.18 | 13.95 | 5.77 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 8.29 | 11.47 | 3.18 |
|  |  | TDLC300-100 |  |  | B |  | 8.35 | 11.46 | 3.11 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 4.09 | 10.29 | 6.20 |
|  |  | TDLA30-10 |  |  | B |  | 4.13 | 10.25 | 6.12 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 4.34 | 7.70 | 3.36 |
|  |  | TDLC300-100 |  |  | B |  | 4.36 | 7.70 | 3.34 |

Table 2.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 100 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | PUSCH Mapping type | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  | IRC | MRC | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | A | 16 | 11.61 | 15.44 | 3.83 |
|  |  | TDLA30-10 |  |  | B |  | 11.64 | 15.43 | 3.79 |
|  |  | TDLC300-100 |  | 5.36 | A | 16 | 12.23 | 13.70 | 1.47 |
|  |  | TDLC300-100 |  |  | B |  | 12.26 | 13.70 | 1.44 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 8.35 | 14.00 | 5.65 |
|  |  | TDLA30-10 |  |  | B |  | 8.41 | 14.03 | 5.62 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 8.06 | 11.24 | 3.18 |
|  |  | TDLC300-100 |  |  | B |  | 8.13 | 11.24 | 3.11 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | A | 16 | 4.24 | 10.24 | 6.00 |
|  |  | TDLA30-10 |  |  | B |  | 4.31 | 10.25 | 5.94 |
|  |  | TDLC300-100 |  | (7.29, -2.5) | A | 16 | 4.24 | 7.54 | 3.30 |
|  |  | TDLC300-100 |  |  | B |  | 4.28 | 7.55 | 3.27 |

# Conclusions

The simulation results for NR MMSE-IRC PUSCH are delivered.

# References

[1] R4-2504768, “Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#114bis, Wuhan, China.