3GPP TSG-RAN4 Meeting #115	R4-2506836

Malta, MT, 19th – 23th May, 2025

Agenda item:			7.5.3

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN4#114bis meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

# Ad hoc meeting agreements #### Issue 2-3: Triggering condition to enable ”One Rx RF chain” RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode: - Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed - Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified - Note: detailed conditions to enable/disable can be further discussed and decided separately # Agreements during/after online discussion #### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers #### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations - Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed #### Issue 2-4: Discussion on applicability for PCell and SCell A more detailed proposal based on the tentative agreement in online discussion: - To address the potential impacts on the DL performance for PCell, RAN4 agrees to further discuss the following aspects: - To limit the use of this feature to CA configurations where the impact is limited - In the CA configurations where there is limited impact, how to further mitigate the impact by means including restricting the PCC and SCC frequency location, or restricting UE locations (e.g., the feature is not enabled when the UE is located at cell edge where wanted signal level is close to receiver sensitivity) # Open issues UE capability indication for supporting ”one Rx RF chain” for DL fragmented carriers For information: Companies are encouraged to provide pros and cons analysis that can further consider aspects below: - For futureproofness, how to clearly indicate applicable intra-band CCs for the band where there are more than two non-contiguous CCs - RAN4 spec impact and work load

- Discussion

  - UE capability indication for supporting ”one Rx RF chain” for DL fragmented carriers

Regarding the UE capability indication for supporting ”one Rx RF chain”, in total there are two mainstream options:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication.

Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling.

For Alt1, it should introduce more indication details about which CCs can perform RX chain sharing. For example, in a combination of CA_nX(3A)-nY (2A), if only two CCs in the nX band can share the receive chain together due to the total bandwidth of 100M, then it would be very confusing for NW which two CCs can share the receive link if the ability to support RX chain sharing is reported by ‘per BC per band’. Considering the granularity of the capability, it seems that when the capability is specified as ‘per CC per band per BC’, it would be clearer.

Observation 1: ‘Legacy band combination reporting scheme + Rx sharing capability indication’ should consider the granularity of the capability because which CCs could be shared needs to be clarified.

Proposal 1: For ‘Legacy band combination reporting scheme + Rx sharing capability indication’, the granularity of the capability should be set as ‘per CC per band per BC’.

For Alt3, we propose to use notation FnA to represent there are n NC CCs sharing one RX RF chain, for example: when there are 2 NC CCs of band nX in the separate state, the CA combination type is CA(2A), when switching to shared state, i.e., one RX RF chain mode, the CA combination type should change to CA(F2A), F represents the two NC CCs are in the “FC” mode. Similarly, CA_nX(F2A-A) means there are 3 NC CCs and two of them are in the sharing chain (one RX RF chain), CA_nX(F2A-F2A) means there are 4 NC CCs and two of them are in one sharing chain, the other two are in another sharing chain.

Proposal 2: Recommend to use notation “FnA” to represent that there are n NC CCs sharing one RX RF chain, e.g., CA(F2A) represents there are two NC CCs and in the “FC” mode.

### 2.2. Discussion on semi-statically switch hardware resources procedure and fall back considerations

When either the in-gap interference or the self-interference is too high, the receiver is hard to maintain the shared receiving state, i.e., one RX RF chain, and the “fallback” behaviour needs to be studied. In order to make the application of this function more future-oriented, some higher CA combinations are also considered.

In total, two aspects should be involved when discussing fallback behaviour: “CA fallback” and “architecture switching”.

“CA fallback” means releasing at least one SCell in one band of the band combination, while “architecture switching” means UE architecture is changed from “FC” architecture (i.e., One Rx RF Chain) to legacy CA architecture (i.e., fully separated Rx Chain or partially shared Rx Chain) at least for one band of a band combination.[2]

If we go further and categorize “CA fallback”, the following scenarios are involved:

- Legacy CA combination falls back to lower order CA

- Legacy CA combination belongs to inter-band CA and intra-band CA with more than 2CCs, etc.

- Legacy CA combination falls back to “non-CA” mode

- Legacy CA combination belongs to intra-band CA combination with 2CCs.

If we go further and categorize “architecture switching”, the following scenarios are involved:

- Architecture switches from “FC” mode to fully-separate mode

- Before sharing, the architecture should be fully-separate mode

- Architecture switches from “FC” mode to partially-shared mode

- Before sharing, the architecture should be partially-shared mode

Based on the analysis above, the fallback behaviour should be totally divided into the following three scenarios:

- Scenario 1: CA fallback without architecture switching

- Scenario 2: CA fallback with architecture switching

- Scenario 3: architecture switching without CA fallback

For Scenario 1:

When the UE receives two CCs in “FC” mode but experiences significant performance degradation on one CC due to excessive interference, the NW could just release the degraded CC. The local oscillator frequency can either remain unchanged or revert to the center frequency of the remaining CC. If ensuring the PCC's reception performance is mandatory, the SCC can be directly released, with the LO shifted to the PCC's center frequency and the filter bandwidth narrowed—effectively reverting to a single-carrier transmission architecture.

This fallback mechanism is similar to legacy CA combination fallback procedure, so the UE should inherently support this capability by default without requiring additional feature.

Proposal 3: For CA fallback without architecture switching, the behaviour should be default supported by the UE without additional feature.

For Scenario 2:

Since this scenario involves a switch in the receiver architecture, additional chain support is required to transition CC1+CC2 from a shared to an independent reception state. This capability is not inherently supported by default and depends on UE implementation. A new UE capability indication would be needed. [3]

For Scenario 3:

Similar to Scenario 2, maintaining the same CA combination while switching CC1+CC2 from a shared back to an independent reception state requires either: Reducing the MIMO layers for part of the CCs, or using additional idle standby links to facilitate the transition. Thus, this fallback still depends on UE implementation and requires additional capability support.

Proposal 4: For CA fallback with architecture switching / architecture switching without CA fallback, the behaviour is UE implementation dependent and requires a new UE capability indication to support the corresponding fallback mechanism.

Regarding the decision-making authority for switching-to-sharing and fall-back behaviours, whether there is any change to the CA combination type, or MIMO layer should be judged firstly, if so then it could only be configured by the NW. If not then it could also be triggered by the UE itself.

Proposal 5: If the CA combination type, or MIMO layers need to be changed, the switch-to-sharing and fall-back behaviour should be configured by the NW. Otherwise, the UE could autonomously determine the switch and fall-back procedure by itself.

When the blocker’s power suddenly increases to a rather high level or the target CC’s power drops to a rather low level for some uncertain reason, the NW may lose connection with the UE and the UE also cannot report the poor measurement results to the NW immediately. In this case, the UE has to decide to fall back on its own. Hence, the UE decision-making mechanism should be considered as a supplement. In the extreme cases mentioned above, when the UE has to decide to fall back on its own, and there should be a default process for the UE to follow, e.g., For single band operation, there must be either one of the two-fragmented carriers to be a PCELL, and when the UE needs to fall back then it would switch to the non-CA mode. It could also be extended to one band with more than 2CCs. However, for the higher band combination with more than one band, generally not all the bands would receive in one RX RF mode, so fallback may not be a big issue.

Proposal 6: For a mechanism in which the UE determines the switching of reception modes, a default fall-back process is necessary, e.g., one of the two fragmented carriers should be configured as PCELL for the single band operation.

The following analysis about the specific categories of switch-to-sharing and fall-back behavior is also captured in the proposed TP.[4]

#### 2.2.1. Case1: Partially shared switch to fully shared & fallback behaviour

According to session 5.2, the partially shared architectures contain both type2: Internal Rx path split, and type3: External Rx path split. The two partially shared architectures are expressed in the same form as below, i.e., the antenna is shared between CC1/2/3, and the other devices before down-conversion such as LNA, BPF are omitted here.

When partially shared architecture switches to fully shared architecture, for band nX CC1 and CC2 would be received by one RX RF chain together instead of each CC taking up one chain. Therefore, there would be one chain being released after each antenna.

According to the utilization of the released chain, there are two sub-scenarios: the released chain is not used (Fig 8.2.1-1), or the released chain is used to aggregate more CCs in the same band (Fig 8.2.1-3). As the new aggregated CCs belong to the same band and there is no new demand in the UE's hardware capability, it could be predicted that there is no new capability needed except for supporting the “FC” architecture.

- After sharing, the released chain is not used.

- Partially separate→ “FC”

Fig 8.2.1-1. Partially shared switch to “FC” mode

As can be seen in Fig 8.2.1-1, before sharing the partially shared architecture composes of one set of RX chain (one main Rx + one diversity Rx). After sharing half of the chain is released and unused which may have the benefit of power saving.

- Fallback behaviour:

Fig 8.2.1-2. Fallback to lower order CA combination (scenario1), or CA combination unchanged (scenario3)

In Fig 8.2.1-2, when fall back behaviour is triggered, there are two options:

Falling back to scenario1, which means only the combination type becomes lower without changing architecture.

Falling back to scenario3, which means CA combination and layer unchanged, while it needs architecture switching, i.e., CC1 and CC2 should be split into 2 chains, and each has one LO supported.

- After sharing, the released chain is used to aggregate more CCs in the same band.

- Partially separate→ “FC” & Fallback behaviour

The following analysis takes the 4RX-chain architecture as an example, and the analysis of the 2RX-chain architecture is similar.

There are three different fallback choices when the “FC” mode has 4RX chains, and it also depends on the original architecture before sharing: One possibility is 4RX UE with one main receiver branch and three diversity receiver branches, which could be seen in Fig 8.2.1-3. Another possibility is the original architecture has 2 sets of RX chains with each set consisting of one receiver branch and one diversity receiver branch, which could be seen in Fig 8.2.2-5.

Fig 8.2.1-3. 4RX UE, switch to “FC” mode

Fig 8.2.1-4. 4RX UE, fall-back to a lower order CA combination

In Fig 8.2.1-4, for 4Rx UE the fallback behaviour would lead to a lower order CA combination with architecture changed (scenario1) or unchanged (scenario2).

Fig 8.2.1-5. two sets of RX chains, switch to “FC” mode

Fig 8.2.1-6. two sets of RX chains, fallback with CA combination unchanged

Comparing to Fig 8.2.1-4, there is another fall-back option as scenario3 in Fig 8.2.1-6: switching the architecture without CA fallback, after fallback the layer of CC1/2 would be reduced.

#### 2.2.2. Case2: fully separate switch to fully shared & fallback behaviour

According to the discussion before, there is another popular implementation architecture before sharing: fully-separate mode, which means each CC has exclusive access to one single RX RF chain from antenna to ADC.[4]

The following analysis primarily focuses on the utilization of idle links after switching to “FC” mode and its differences from the partially shared architecture. The fallback behaviour is similar and therefore omitted here.

Fig 8.2.2-1. After switching to “FC” mode, more CC from another band could be aggregated (case A), or MIMO layer could be higher

When fully-separate architecture changes to fully-shared architecture, some RX chains would also be released and the band combination or MIMO layer could be increased. Since the switching scheme of case A may involve changes in frequency and/or bandwidth of some devices as the part of the architecture would switch to receiving the CC3 from another band, it should be supported by additional capabilities.

- Conclusion

Observation 1: ‘Legacy band combination reporting scheme + Rx sharing capability indication’ should consider the granularity of the capability because which CCs could be shared needs to be clarified.

Proposal 1: For ‘Legacy band combination reporting scheme + Rx sharing capability indication’, the granularity of the capability should be set as ‘per CC per band per BC’.

Proposal 2: Recommend to use notation “FnA” to represent that there are n NC CCs sharing one RX RF chain, e.g., CA(F2A) represents there are two NC CCs and in the “FC” mode.

Proposal 3: For CA fallback without architecture switching, the behaviour should be default supported by the UE without additional feature.

Proposal 4: For CA fallback with architecture switching / architecture switching without CA fallback, the behaviour is UE implementation dependent and requires a new UE capability indication to support the corresponding fallback mechanism.

Proposal 5: If the CA combination type, or MIMO layers need to be changed, the switch-to-sharing and fall-back behaviour should be configured by the NW. Otherwise, the UE could autonomously determine the switch and fall-back procedure by itself.

Proposal 6: For a mechanism in which the UE determines the switching of reception modes, a default fall-back process is necessary, e.g., one of the two fragmented carriers should be configured as PCELL for the single band operation.

- References

- R4- 2505109, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#114bis.

- R4-2503366, On fallback behavior and signalling aspect for Fragmented carriers, Samsung, #114bis.

- R4-2504283, On signalling aspect of fragmented carriers, HW, #114bis.

- R4-2506835, TP for TR38.755 on fall-back considerations, vivo, #115.

- R4-2411310, Views on UE RF architecture and NW deployment assumption for fragmented carriers, Samsung, TELUS, Bell mobility, #112.