3GPP TSG-RAN WG4 Meeting #115	R4-2506845

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | <CR#> | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.870 on test method of AIoT device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R19, the OTA test method for AIoT device is studied and this CR create new sections to capture related conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the device type, operationg band and related test parameters, test setup, test procedure, and MU assessment for ambient IoT device |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test details for ambient IoT device is absent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.3.2, 4.3.3, 6.1, new 7.2.2, new 7.4.7, new 7.4.8, new B.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.191 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[5]	3GPP TS 38.521-1: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”.

[6]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[7]	3GPP TS 38.508-1: “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment “.

[8]	3GPP TR 25.914: “Measurements of radio performances for UMTS terminals in speech mode”.

[9]	IEEE Std 149: “IEEE Standard Test Procedures for Antennas”, IEEE.

[10]	JCGM 100:2008: “Evaluation of measurement data — Guide to the expression of uncertainty in measurement”.

[11]	ETSI TR 102 273-1-1: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 1: Introduction”.

[12]	ETSI TR 100 028-2: “ElectroMagnetic Compatibility and Radio Spectrum Matters (ERM); Uncertainties in the measurement of mobile radio equipment characteristics; Part 2”.

[13]	ETSI TR 102 273-1-2: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes”.

[14]	CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.71 Device Setup and Positioning Guidelines”, latest active version available at: https://ctiacertification.org/test-plans/

[15]	Foegelle, M.D., “The Surface Standard Deviation Method for TRP Measurement Uncertainty”, 25th Proceedings of the Antenna Measurement Techniques Association (AMTA 2003), A03-027

[16]	3GPP TR 37.902: “Measurements of User Equipment (UE) radio performances for LTE/UMTS terminals; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test “.

[17]		3GPP TS 37.544: “Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing “.

[18]	3GPP TR 37.941: “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”

[19]	3GPP TR 38.810: “NR; Study on test methods”

[20]	3GPP TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”

[21]	“Reverberation Chamber Metrology for Wireless Internet of Things Devices”, Anouk Hubrechsen, Kate A. Remley and Sara Catteau, IEEE Microwave Magazine, February 2022, pp.75-85

[22]	“Proximity and antenna orientation effects for large-form-factor devices in a reverberation chamber” Willem T. C. Burger, Kate A. Remley, Christopher L. Holloway, John M. Ladbury, 2013 IEEE International Symposium on Electromagnetic Compatibility, pp.671-676

[23]	“A Significance Test for Reverberation-Chamber Measurement Uncertainty in Total Radiated Power of Wireless Devices”, Kate A. Remley, Chih-Ming Jack Wang, Dylan F. Williams, Johannes J. aan den Toorn and Christopher L. Holloway, IEEE TRANSACTIONS ON ELECTROMAGNETIC COMPATIBILITY, VOL. 58, NO. 1, FEBRUARY 2016, pp.207-219

[24]	3GPP TR 38.834: “Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology (Release 17)”

[25]	CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.72: Near-Field Phantoms”, latest active version available at: https://ctiacertification.org/test-plans/

[26]	CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.70: Measurement Uncertainty”, latest active version available at: https://ctiacertification.org/test-plans/

[27]	3GPP R4-2412049, Final Analysis of 3GPP Rel-18 TRP TRS AC lab alignment and RC harmonization measurement results, vivo

[28]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[29]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[30]	3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access".

[31]	3GPP TS 38.521-5: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[32]	3GPP TS 36.521-4: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing".

[33]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[34]	3GPP TS 38.191: "Ambient IoT device radio transmission and reception".

## <<Unchanged part is omitted >>

4.1	Device types

The following device types are within the scope of TRP TRS test:

-	Smartphone

-	Considering UEs with antenna configurations of 1Tx, 2Tx, 2 Rx and 4 Rx

-	Wearable (e)Redcap UE

-	XR devices, including glasses and head mounted display

-	Only test XR devices with 5G link to gNB in networks, not those with side link or wired connections

-	1Tx XR devices is prioritized

-	NTN devices, including both NR-NTN and IoT-NTN,

-	handheld UE type is first priority

-	Tablet

-	Laptop embedded equipment (LEE)

-	Laptop mounted equipment (LME)

-	Ambient IoT device

## <<Unchanged part is omitted >>

4.3.2	Operating bands

Operating bands for NR FR1 are defined in Table 5.2-1 and Carrier Aggregation (CA) are defined in Clause 5.5A in TS 38.101-1 [3]. The operating bands for EN-DC are defined in Clause 5.5B in TS 38.101-3 [4].

The FR1-NTN bands are defined in Table 5.2.2-1 in TS 38.101-5 [29]. The IoT-NTN bands are defined in Table 5.2-1 in TS 36.102 [30].

For Ambient IoT bands are defined in Table [5.2-1] in TS 38.191 [34]

4.3.3	Test parameters for each band

## <<Unchanged part is omitted >>

Table 4.3.3-10: TRS measurement parameters for NB-IoT NTN UE

| NR Band | CBW (MHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | MUL/ MDL | UL Configuration Ntones | DL Configuration Subcarriers |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 0.2 | QPSK | BPSK | Low | 261845 | 1980.1 | 229077 | 2170.1 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 262143 | 2009.9 | 229375 | 2199.9 | 0 |  |  |
| 255 | 0.2 | QPSK | BPSK | Low | 261505 | 1626.6 | 228737 | 1525.1 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 261843 | 1660.4 | 229075 | 1558.9 | 0 |  |  |
| 254 | 0.2 | QPSK | BPSK | Low | 261340 | 1610.1 | 228572 | 2483.6 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 261503 | 1626.4 | 228735 | 2499.9 | 0 |  |  |

Table 4.3.3-10: Tx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (kHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK/BPSK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Table 4.3.3-11: Rx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (MHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [0.16] | [0.2] | 15 | OOK | OOK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

## <<Unchanged part is omitted >>

6.1	Free space

For Free space configuration, the centre of the reference coordinate system shall be aligned with the geometric centre of the DUT in order to minimize the offset between antenna arrays integrated at any position of the UE and the centre of the quiet zone.

Table 6.1-1: UE positioning for Free space

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Free space DUT | α = 0º; β = 0º; γ = 0º |  |

For Ambient IoT device, if the device has a rectangular shape, the DUT orientation in Table 6.1-1 is applied and front and back side of device is based on device declaration. Otherwise, the device positioning is based on device declaration.

## <<Unchanged part is omitted >>

7.2	Test setup

7.2.1 General

For FR1 TRP TRS testing, both Single-antenna and multiple-antennas anechoic chambers can be applied. In Figure 7.2.1-1, an example TRP TRS test system with combined axes system is presented.

Figure 7.2.1-1: Example of a FR1 TRP TRS OTA test system with combined axis

In Figure 7.2.1-2, an example TRP TRS test system with distributed axes system is presented.

Figure 7.2.1-2: Example of a FR1 TRP TRS OTA test system with distributed axis

7.2.2 Test setup for ambient IoT device

For ambient IoT device testing, both Single-antenna and multiple-antennas anechoic chambers can be applied. In Figure 7.2.2-1, an example test system with combined axes system is presented.

Figure 7.2.2-1: Example of an ambient IoT device OTA test system with combined axis

In Figure 7.2.2-2, an example test system with distributed axes system is presented.

Figure 7.2.1-2: Example of an ambient IoT device OTA test system with distributed axis

The test antenna for CW is at fixed position and D2R power at test antenna should be at least [15] dB higher than CW leakage.

## <<Unchanged part is omitted >>

7.4.7	Radiated transmitter test procedure for Ambient IoT device

7.4.7.1	Device configuration

TBD

7.4.7.1	Carrier wave source configuration

TBD

7.4.7.3	Test procedure

TBD

7.4.8	Radiated receiver test procedure for Ambient IoT device

7.4.8.1	Device configuration

TBD

7.4.8.1	Carrier wave source configuration

TBD

7.4.8.3	Test procedure

TBD

## <<Unchanged part is omitted >>

B.6	MU Assessment for TRP of ambient IoT device

B.6.1	MU Assessment for TRP in Anechoic Chamber

The uncertainty contributions related to TRP of ambient IoT device are listed in Table B.6.1-1.

Table B.6.1-1 Uncertainty contributions in TRP measurement for anechoic chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | B.2.1 |
| 2 | Insertion loss of transmitter chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | CW for backscatter measurement or R2D for sensitivity measurement: uncertainty of the absolute output level | FFS |
| 5 | Sensitivity measurement: output level step resolution | FFS |
| 6 | Measurement distance | B.2.7 |
| 7 | Quality of quiet zone | B.2.8. |
| 8 | DUT sensitivity drift | FFS |
| 9 | Coarse sampling grid | B.2.12 |
| 10 | Random uncertainty | B.2.13 |
| 11 | Frequency Response | B.2.14 |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | B.2.15 |
| 13 | Mismatch of transmitter chain | B.2.1 |
| 14 | Insertion loss of transmitter chain | B.2.2 |
| 15 | Mismatch in the connection of calibration antenna | B.2.1 |
| 16 | Influence of the calibration antenna feed cable | B.2.3 |
| 17 | Influence of the measurement antenna cable | B.2.3 |
| 18 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | B.2.16 |
| 19 | Measurement distance | B.2.7 |
| 20 | Quality of quiet zone | B.2.8 |
| Systematic Errors |  |  |
| 21 | Systematic Error related to TRS grids | B.2.12 |

## <<End of Change >>